3GPP TSG-RAN2 Meeting #127	R2-2407397

Maastricht, The Netherlands, 19 - 23 August 2024	Revision of R2-2404997

Agenda Item:	8.4.3 RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Source: 	Ericsson

Title:  	LP-WUS and RRM measurements

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS and RRM measurements are discussed further.

For the WID objectives and RAN1, RAN2 and RAN4 agreements the reader is referred to the Annex.

# Discussion

## MR serving cell relaxation without LP-WUS

Serving cell measurement relaxation is neither supported for normal broadband UE nor for RedCap UE in NR. Serving cell measurement relaxation is only supported for NB-IoT (limited mobility) and MTC in LTE for UEs that support WUS, i.e. the serving cell relaxation is coupled with WUS support:

In LTE only NB-IoT/MTC UEs supporting WUS are allowed to relax serving cell measurements provided that the neighbour cell measurements are relaxed and the S criteria is met with 2 dB margin.

For a normal broadband UE that do not support LP-WUS there is no benefit in serving cell relaxation, because the UE anyways has to monitor paging using MR every DRX cycle. The same applies to a UE supporting LP-WUS, when that UE is outside LP-WUS coverage:

There is no benefit of serving cell measurement relaxation when the UE does not support WUR, because the MR has to monitor Paging every DRX cycle.

The mobility and paging performance may be impacted when the UE does not have an LR to perform the serving cell measurements when the MR is relaxed. Serving cell relaxation for a UE not supporting LP-WUS is outside the scope of the LP-WUS work item in our view. Furthermore serving cell relaxation may impact other features and should not be discussed in this session:

Serving cell measurement relaxation when a UE does not support LP-WUS is out of scope of the LP-WUS work item.

RAN4 did not identify use cases for MR relaxation/offloading without LR ON, i.e. RAN2 should not discuss use cases that are out of scope in RAN4:

RAN2 does not discuss use cases that are out of scope in RAN4, i.e. RAN2 only discusses MR serving cell relaxation when LR is ON.

## MR offloading and relaxation

RAN2 status

RAN2 has discussed MR serving cell relaxation, without WUR support, under the Idle/Inactive agenda item. Furthermore RAN2 discussed MR serving cell relaxation and fully offloading under the RRM agenda item separately. RAN2 agreed that the LR entry condition for paging monitoring makes use of MR serving cell measurements (FFS if LR measurements are needed). The MR exit condition only uses LR serving cell measurements. When the LR entry condition is fulfilled the UE may monitor Paging using LP-WUS, and when the UE uses the LR to monitor paging then the UE may stop monitoring legacy PO using MR.

RAN4 status

RAN4 identified the following use cases for MR offloading and relaxation when the LR is ON:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Case #1 (i.e. MR offloading) is agreed, while case #2, #3 and #4 (i.e. MR relaxation) are FFS.

The following observations can be made:

The term “fully” offloading is somewhat misleading, because only serving cell measurements can be offloaded. Furthermore this offloading is conditional and temporary. The MR cannot be offloaded when the entry condition to use the LR is not fulfilled. The MR has to start serving cell measurements when the LR exit condition is met.

Both MR serving cell offloading and MR serving/neighbour cell relaxation are conditional and temporary, i.e. only for some time the MR is not required to perform serving/neighbour cell measurements. Due to mobility, serving cell degradation or for higher priority frequencies the UE may be required to perform neighbour cell measurements. MR serving cell offloading implies the presence of an LR, while MR neighbour cell relaxation does not require an LR.

In our view it is only relevant to talk about MR serving cell offloading/relaxation in the context of WUR. Because the LR is not able to perform neighbour cell measurements, i.e. the MR neighbour cell measurements do not change when the UE supports WUR:

MR serving cell offloading/relaxation can be discussed when the UE supports WUR, because the LR can also perform serving cell measurements. The MR neighbour cell measurement requirements do not change when the UE supports WUR because the LR cannot perform neighbour cell measurements.

Furthermore the difference between fully offloading and relaxation of the MR serving cell measurements is not relevant from RAN2 perspective because the RAN2 agreed entry/exit condition for paging monitoring using LP-WUS monitoring can be re-used for LR serving cell measurements using PSS/SSS and LP-SS:

- When the LR entry condition is fulfilled and the LR performs serving cell measurements, then the MR is not required to measure serving cell.

- When the LR exit condition is met, then the MR is required to measure serving cell.

There is no/little benefit to have separate entry conditions for LR serving cell measurements and paging monitoring. Because if only the entry condition for paging is fulfilled, then the MR still needs to perform serving cell measurements every DRX cycle. If only the entry condition for serving cell measurements is fulfilled, then the MR still needs to perform paging monitoring every DRX cycle.

Furthermore the entry/exit condition for LR paging monitoring is based on LP-SS or SSB measurements, i.e. it is academic to define different threshold values for paging and serving cell measurements:

When the LR entry condition is fulfilled and the LR performs serving cell measurements then the MR may stop serving cell measurements. FFS whether there are cases when the entry condition is fulfilled and the MR and LR are required to perform serving cell measurements simultaneously.

Concerning the FFS: RAN2 and RAN4 have discussed cases where the UE may be required to measure serving cell using both MR and LR, e.g.:

- LP-SS measurements as part of the LR entry condition.

- MR measurements to retune the LR timing/frequency e.g. when the LR quality is bad [3].

When the LR exit condition is fulfilled the MR is required to perform serving cell measurements:

When the LR exit condition is fulfilled the LR shall stop serving cell measurements and the MR shall start serving cell measurements.

RAN4 has discussed use cases where both MR and LR perform serving cell measurements [4]:

We agree that such use cases exist, but the motivation to support such use cases should be discussed carefully, see the two examples above.

We do not think that MR serving cell measurements can compensate for or improve the LR measurement accuracy for example. And either the MR serving cell measurements are performed very infrequent to be effective, or they are performed too frequent and destroy the power saving gains of WUR.

RAN4 has been discussing how to capture the LP-SS measurement accuracy requirements, e.g. as dedicated accuracy requirements, potentially including test cases, or as a cell reselection margin (see legacy 3dB margin below) to the core requirements:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that:

when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

In legacy the UE is required to be able to determine that an intra-frequency neighbour cell is 3 dB better within Tevaluate,NR_Intra. For LP-WUS a larger margin could be defined. A larger cell reselection margin implies that cell reselection is delayed:

RAN2 to discuss the impact of a larger cell reselection margin when LP-SS measurements are used.

## MR neighbour cell measurement relaxation

RAN2 agreed to use the Rel-16 relaxation criteria as a baseline:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

Neighbour cell measurement relaxation (i.e. intra-/inter-/iRAT-frequency measurement relaxation) is supported in NR for normal broadband UEs and (e)RedCap UEs. In both cases the basic principle is the following:

- A mobility and/or good coverage criterion can be configured in SIB2.

- When mobility has not been detected for a period of TSearch and/or when the UE is in good coverage:

  - UE may perform relaxed RRM measurements for intra-/inter-/iRAT-frequencies as specified in 38.133.

  - The relaxation level depends on whether both mobility and good coverage criterion need to be fulfilled simultaneously or not.

- When highPriorityMeasRelax is not configured then the UE is required to perform higher priority frequency measurements every 60 seconds as in legacy. Otherwise the higher priority frequency measurements can be relaxed when no mobility has been detected and the UE is in good coverage.

Higher priority frequency measurements ensure that the UE is always on the highest priority frequency, even when the UE is above the inter-frequency measurement threshold on the current frequency. These higher priority frequency measurements are performed in the background, i.e. the UE is only required to measure an inter-/iRAT frequency every 60 seconds.

The neighbour cell measurement relaxation depends on the relaxedMeasurement configuration in SIB2 and whether the relaxation criteria are met. In best the case, i.e. when the UE is stationary and/or in good coverage, the UE is not required to perform intra-/inter-/iRAT frequency measurements for 1 hour. In such case the UE can also relax higher priority frequency measurements up to 1 hour, provided that highPriorityMeasRelax is set.

Using the legacy measurements thresholds (SIntraSearch, SNonIntraSearch) and Rel-16 threshold (Not-at-cell-edge) different areas can be identified with different measurement requirements, that also depend on the UE mobility (lowMobility):

The LP-WUS coverage is independent from the neighbour cell measurements areas above, i.e. LP-WUS can cover the complete cell, or only a small portion in the cell centre.

However RAN4 seems to assume a coupling between RRM relaxation and LP-WUS coverage with the use cases RAN4 identified:

When the UE supporting LP-WUS is outside LP-WUS coverage or LP-WUS is not configured then the UE uses MR serving cell measurements with the Rel-16 criteria, similar as UEs that do not support LP-WUS:

When WUR is not configured or the LR entry condition is not fulfilled or the UE does not support WUR then the MR serving cell measurements are used to determine when the Rel-16 criteria for neighbour cell relaxation are fulfilled.

RAN2 only needs to discuss how neighbour cell relaxation is defined when the entry condition is fulfilled and the LR is performing serving cell measurements.

The OFDM-based serving cell measurements using PSS/SSS are quite similar in latency and performance compared to the MR serving cell measurements, if not identical. In this case the LR can continue to do serving cell measurements using the Rel-16 criteria:

For OFDM-based WUR when the LR performs serving cell measurements the LR determines based on the Rel-16 neighbour cell relaxation criteria, when the MR needs to be activated to perform neighbour cell measurements.

FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

The trigger for MR neighbour cell measurements is not part of the LR exit condition, i.e. the LR is allowed to continue serving cell measurements when the MR is performing neighbour cell measurements.

Below a picture of the LR entry/exit condition and triggers to activate the MR for paging/neighbour cell measurements:

There is perhaps limited power saving gain when the MR only needs to perform neighbour cell each DRX cycle, i.e. LR can continue to perform serving cell measurements. The serving cell measurements come for free when the MR performs intra-frequency neighbour cell measurements.

Neighbour cell relaxation beyond Rel-16

Furthermore when the Rel-16 measurement requirements are further relaxed, this also means that the relaxation criteria will become stricter, i.e. the measurement requirements cannot be relaxed without a change in the relaxation criteria. The area and scenarios where the UE is allowed to even further relax will become smaller and fewer, i.e. the impact is significantly reduced. Furthermore a correlation can be expected between LR exit (LR link is bad and can no longer perform serving cell measurements) and neighbour cell relaxation is no longer fulfilled, i.e. there is only a certain/small area where further relaxation can be applied. The power saving gain for a feature for which the power consumption is already low is in most case insignificant.

Neighbour cell relaxation beyond the Rel-16 criteria is not discussed further unless it can be proven that it can provide significant power saving gains.

## LP-SS measurement accuracy and latency

In legacy the UE has to take the average of 2 RSRP/RSRQ measurement samples made during consecutive DRX cycles:

Thus the DRX cycle determines the latency in which the UE detects when the serving cell exceeds or drops below a threshold.

In case of OOK-based WUR and LP-SS the measurement latency is independent of the DRX cycle. The assumption is that the UE is required measure 5 samples of LP-SS measurements that are spaced 320 ms apart:

This means that if the DRX cycle is 320 ms, then the LR measurements are 2x slower compared to the MR measurements.

The MR and OFDM-based WUR measurement delay equals the DRX cycle, while the OOK-based WUR measurement delay is 1.28 seconds, i.e. in dependent of the DRX cycle.

We are not sure if there is a problem with the LP-SS measurement latency that needs to be solved, i.e. in many cases a DRX cycles of 1.28 seconds is configured, in which case the MR and LR measurement latency are the same.

But in the transition from serving cell measurement using MR to serving cell measurements using LR there is potentially an additional latency when the LR measurements have to started from scratch when the MR measurements are stopped. For example detecting that the serving drops below a threshold may be delayed with another 1.28 seconds during this transition. This problem is even bigger when the LR has to start detecting whether the UE is in not low mobility state. Similar as during cell re-selection, the UE is assumed to reset the LR RSRP reference value to the LR measured RSRP and start to monitor when the RSRP has dropped with a certain delta:

RAN2 to discuss whether there is additional latency when serving cell measurements on MR are stopped and serving cell measurements on LR are started (and vice versa).

Another is that the LR serving cell measurements to detect the need to start MR neighbour cell measurements have to take into account that the MR may have to wake-up from ultra-deep sleep.

## LR neighbour cell measurements

RAN2 assumes that the LR does not perform neighbour cell measurements:

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

For OFDM-based WUR using PSS/SSS it may be easier to perform neighbour cell measurements compared to OOK-based WUR. But in any case, the MR will perform cell reselection.

There is a benefit when the LR perform neighbour cell measurements, in case neighbour cell measurements are performed for a long time, but cell reselection is not triggered. This may happen when the measurement threshold is high. The LR would need to perform cell ranking, and trigger the LR to perform cell reselection:

RAN2 to discuss the potential benefit of OFDM-based WUR to perform neighbour cell measurements and cell ranking.

## Measurement quantity in multibeam operation

In case of multi-beam operation the measurement quantity for LP-SS serving cell measurements needs to be discussed further, e.g. whether the UE uses the highest beam measurement quantity or the average over a number of beams.

In legacy, i.e. when the MR performs RSRP/RSRQ measurements, then in case of cell selection the measurement quantity is up to UE implementation. But in case of cell reselection the UE has to follow the configuration provided in nrofSS-BlocksToAverage and absThreshSS-BlocksConsolidation (see 38.304):

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

The LR is not performing cell reselection, but we think that RAN2 should discuss the measurement quantity for RR serving cell measurements:

RAN2 to discuss whether the measurement quantity in multi-beam operation for LR serving cell measurements is the same as for MR measurements during cell (re)selection.

## LR serving cell measurement relaxation

The LP-SS monitoring occasions are decoupled from the DRX, i.e. paging monitoring occasions. It is assumed that the UE has to take 5 LP-SS measurements samples, while for SSB only 2 measurement samples are required. Furthermore these SSB occasions are close to the DRX cycle when the UE has to monitor paging:

The OFDM-based LR has to wake-up less frequently compared to OOK-based LR. The OOK-based UE is required to measure LP-SS every measurement occasion (320 ms), while OFDM based UE has to measure one SSB occasion when it wakes-up to monitor paging.

The time to switch LR on/off (TLR, ramp-up) is FFS and the transition energy is TLR, ramp-up *(PON-POFF)/2. Whether the LR goes to sleep in between LP-SS MOs and POs can and should be left to UE implementation:

It is left to UE implementation whether the LR enters sleep state between measurement occasions for serving cell measurements and Paging Occasions (POs).

When the LR uses PSS/SSS then the UE measures the serving cell using the legacy requirements, similar as the MR, i.e. typically every DRX cycle. The measurement performance is expected to be similar when using MR.

When the LR is performing serving cell measurements the power consumption is very low, and OOK-based WUR UE has to measure LP-SS every 320 ms to meet the accuracy/latency requirement:

Relaxation of LR serving cell measurement requirements is not discussed further.

## UE mobility and speed

For the simulations RAN4 assumed w.r.t. UE speed:

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UE speed |  | FR1: 30 km/h |  |

The WID does not explicitly mention mobility requirements. But it is assumed that WUR is not limited to limited mobility scenarios like NB-IoT, but the WUR is supported for a normal broadband UE.

The LR entry/exit condition is independent of the UE speed.

The UE can be moving with a very high speed, e.g. in case of High Speed Train (HST). But it may be challenging to support WUR with HST, i.e. RAN4 should confirm the feasibility. One possibility is to follow the approach when both RRM relaxation and HST is configured and low mobility criterion is fulfilled it is up to UE implementation to use the WUR measurement requirements or HST measurement requirements:

When low mobility criterion is fulfilled it is up to UE implementation to use the WUR or HST measurement requirements.

# Summary

RAN2 is kindly asked to discuss WUR and RRM measurements:

Observation 1	In LTE only NB-IoT/MTC UEs supporting WUS are allowed to relax serving cell measurements provided that the neighbour cell measurements are relaxed and the S criteria is met with 2 dB margin.

Observation 2	There is no benefit of serving cell measurement relaxation when the UE does not support WUR, because the MR has to monitor Paging every DRX cycle.

Observation 3	The term “fully” offloading is somewhat misleading, because only serving cell measurements can be offloaded. Furthermore this offloading is conditional and temporary. The MR cannot be offloaded when the entry condition to use the LR is not fulfilled. The MR has to start serving cell measurements when the LR exit condition is met.

Observation 4	Both MR serving cell offloading and MR serving/neighbour cell relaxation are conditional and temporary, i.e. only for some time the MR is not required to perform serving/neighbour cell measurements. Due to mobility, serving cell degradation or for higher priority frequencies the UE may be required to perform neighbour cell measurements. MR serving cell offloading implies the presence of an LR, while MR neighbour cell relaxation does not require an LR.

Observation 5	The MR and OFDM-based WUR measurement delay equals the DRX cycle, while the OOK-based WUR measurement delay is 1.28 seconds, i.e. in dependent of the DRX cycle.

Proposal 1	Serving cell measurement relaxation when a UE does not support LP-WUS is out of scope of the LP-WUS work item.

Proposal 2	RAN2 does not discuss use cases that are out of scope in RAN4, i.e. RAN2 only discusses MR serving cell relaxation when LR is ON.

Proposal 3	MR serving cell offloading/relaxation can be discussed when the UE supports WUR, because the LR can also perform serving cell measurements. The MR neighbour cell measurement requirements do not change when the UE supports WUR because the LR cannot perform neighbour cell measurements.

Proposal 4	When the LR entry condition is fulfilled and the LR performs serving cell measurements then the MR may stop serving cell measurements. FFS whether there are cases when the entry condition is fulfilled and the MR and LR are required to perform serving cell measurements simultaneously.

Proposal 5	When the LR exit condition is fulfilled the LR shall stop serving cell measurements and the MR shall start serving cell measurements.

Proposal 6	RAN2 to discuss the impact of a larger cell reselection margin when LP-SS measurements are used.

Proposal 7	When WUR is not configured or the LR entry condition is not fulfilled or the UE does not support WUR then the MR serving cell measurements are used to determine when the Rel-16 criteria for neighbour cell relaxation are fulfilled.

Proposal 8	For OFDM-based WUR when the LR performs serving cell measurements the LR determines based on the Rel-16 neighbour cell relaxation criteria, when the MR needs to be activated to perform neighbour cell measurements.

Proposal 9	FFS for OOK-based WUR whether separate Rel-16 neighbour cell relaxation criteria (values) need to be configured.

Proposal 10	The trigger for MR neighbour cell measurements is not part of the LR exit condition, i.e. the LR is allowed to continue serving cell measurements when the MR is performing neighbour cell measurements.

Proposal 11	Neighbour cell relaxation beyond the Rel-16 criteria is not discussed further unless it can be proven that it can provide significant power saving gains.

Proposal 12	RAN2 to discuss whether there is additional latency when serving cell measurements on MR are stopped and serving cell measurements on LR are started (and vice versa).

Proposal 13	RAN2 to discuss the potential benefit of OFDM-based WUR to perform neighbour cell measurements and cell ranking.

Proposal 14	RAN2 to discuss whether the measurement quantity in multi-beam operation for LR serving cell measurements is the same as for MR measurements during cell (re)selection.

Proposal 15	It is left to UE implementation whether the LR enters sleep state between measurement occasions for serving cell measurements and Paging Occasions (POs).

Proposal 16	Relaxation of LR serving cell measurement requirements is not discussed further.

Proposal 17	The LR entry/exit condition is independent of the UE speed.

Proposal 18	When low mobility criterion is fulfilled it is up to UE implementation to use the WUR or HST measurement requirements.

# References

- R2-2404996, WUR in Idle and Inactive, Ericsson DISC, RAN2#126

- RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson, WID, RAN#103

- R4-24xxxxx, Discussion on LP-WUS RRM requirement, Ericsson, DISC, RAN4#112

- R4-2409687, Discussion on core parts on LPWUS, ZTE, DISC RAN4#111

# Annex: WID and agreements

## WID

- For IDLE/INACTIVE modes

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

## RAN1 agreements (selection)

Agreement

For LP-SS design from RAN1 perspective, consider at least the following as the design target:

- For RRM measurement performed by LP-WUR based on LP-SS, UE can satisfy measurement accuracy based on X LP-SS samples within a period which is comparable to Y=the length of I-DRX cycle that is larger or equal to 1.28s.

  - FFS: X

  - Note: Y is chosen for evaluating LP-SS design.

  - Network overhead and network power consumption are to be considered

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

## RAN2 agreements

- ?? On RRM relaxation of UE MR for serving cell measurements: RAN2 assumes RRM measurement relaxation on serving cell via MR if the configured criteria is fulfilled is possible, final decision is up to RAN4.

- ?? FFS in this case whether LR is also used for RRM measurement on serving cell.

- ?? Detailed methods of measurement relaxation and offloading are up to RAN4.

Serving cell measurements related

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Neighboring cell measurements related

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

LP-WUS/Paging monitoring related

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

## RAN4 agreements (selection)

Agreement:

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

  - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Other related requirements are FFS

Agreement: Agreement on the target SNR for RAN4 simulation:

- The outcome of RAN1’s study in Rel-19 WI on SNR/SINR target is used as the starting point for RAN4 LP-WUR requirement study.

- Aspects not considered in RAN1 (if any) can be further discussed in RAN4.

Agreement:

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states:

- RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples.

- RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement.

For information only:

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states:

Options for further discussion (the options are only for information purpose)

- Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case.

- Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part.

- Other options are not precluded.

Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |

RAN4 to further discuss case #2 to #4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

Agreement:

- As starting point for RAN4 simulation purpose, uses 320 ms as the LP-SS periodicity in IDLE/Inactive mode, and other values based on RAN1/2 discussion are not precluded.

  - Not consider the MR DRX configuration in the simulation.

Agreement:

First start the simulation for FR1, and FFS for FR2 pending on the RF progress.

Recommendations:

Companies are encouraged to provide simulation assumptions based on framework in the appendix.

Agreement: Simulation assumptions (for information purpose only)

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| System bandwidth | 20/100 MHz; |
| DRX | No |
| Measurement period (in number of measurement samples) | OOK based: [5, other number of samples may also be studied upon a need] OFDM based: [5] |
| LP-SS/SSB burst periodicity | [320 ms] OFDM based: [320 ms] |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UE speed |  | FR1: 30 km/h |  |