3GPP TSG-RAN2 Meeting #127	R2-2407398

Maastricht, The Netherlands, 19 - 23 August 2024	Revision of R2-2404998

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson

Title:  	LP-WUS in Connected

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS in connected mode is discussed further.

For the WID objectives and RAN1/RAN2 agreements the reader is referred to the Annex.

# Discussion

## Observations

The study report [1] shows that the main power saving gains with LP-WUS are obtained in Idle and Inactive mode. In connected mode most of the power is consumed by transmitting (and receiving) data, i.e. LP-WUS power saving gains are limited.

In connected mode the MR cannot enter ultra-deep sleep because this requires at least 10-20 seconds sleep time to be beneficial, i.e. the transition energy in and out of ultra-deep sleep is high. The connection is typically released before that time. But the MR can enter deep sleep because the deep sleep transition time is relatively short (20 ms) and transition energy more limited (450) compared to ultra-deep sleep.

According to the WID, in connected mode the MR is used for RRM/RLM/BFD/CSI measurements [2], i.e. the LR does not perform serving cell measurements using LP-SS nor PSS/SSS:

Compared to Idle and Inactive mode the expected power saving gains of LP-WUS in Connected mode are (very) limited because ultra-deep sleep cannot be used and most of the power consumption is determined by data transmission and reception.

Only for special uses cases, when there is only a small amount of data to send and receive, real power saving gains can be expected in connected mode. Therefore complex solutions for LP-WUS in connected mode with large specification impact and impact on different working groups are not justified:

The connected mode solution is kept simple with limited specification impact and limited impact on the different working groups.

## Options 1-1, 1-2 and 1-3

RAN2 agreed to discuss the impact of options1-1, 1-2 and 1-3 identified by RAN1:

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

The most recent RAN1 description is as follows:

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring

Interpretation of the RAN1 options:

- The Active Time for option 1-3, refers to the Active Time (AT) except when the drx-OnDurationTimer is running. Thus the inside AT does not include when drx-OnDurationTimer is running.

- The outside AT for option 1-2 may include when the drx-OnDurationTimer is running in particular for option 1-2-2.

- With option 1-2-1 the PDCCH monitoring triggered by LP-WUS is coupled to the legacy DRX parameters. With option 1-2-2 the PDCCH monitoring triggered by LP-WUS is decoupled from the legacy DRX parameters. Thus PDCCH monitoring triggered by LP-WUS coupled to a new DRX parameter is part of option 1-2-2.

Observations of the RAN1 options:

The option that LP-WUS can be used without C-DRX configuration has been eliminated. So whether the PDCCH monitoring triggered by LP-WUS is coupled with legacy DRX parameters or not, C-DRX is always configured.

An example of option 1-2-2 is when LP-WUS is used during the pdcch-SkippingDuration, because then the LP-WUS triggered PDCCH monitoring is not coupled to the legacy C-DRX, i.e. it is hidden in L1. But these solutions should not be discussed in RAN2, i.e. they are not visible in MAC. These PDCCH monitoring requirements are and should be captured in RAN1 specification 38.213. If RAN1 wants to discuss PDCCH monitoring requirements that are not coupled with legacy DRX parameters like PDCCH skipping or SSSG switching, and this does not impact MAC, then they can of course also do that. FYI RAN1 agreed:

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

- Rel-16 DCP

- Rel-17 PDCCH skipping

- Rel-17 SSSG switching

- Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN2 should discuss PDCCH monitoring that is triggered by legacy or new DRX parameters. These PDCCH monitoring requirements are captured in RAN2 specification 38.321.

In the remainder of this contribution we will discuss the RAN1 options with the following understanding:

RAN2 should discuss LP-WUS triggered PDCCH monitoring based on legacy or new DRX parameters, which are captured in RAN2 specification. RAN2 should not discuss LP-WUS triggered PDCCH monitoring on a very short time scale of 1 or more mseconds like PDCCH skipping which are captured in RAN1 specification.

NOTE: RAN2 should try to understand the RAN1 options correctly and discuss the RAN2 impact. But the RAN1 solutions have changed in the past and might also change in the future.

Before the options w.r.t. LP-WUS triggered PDCCH monitoring are discussed further. First more about measurements and CSI reporting in connected mode: The MR performs RRM/RLM/BFD/CSI measurements, which means that the MR has to wake-up to do:

- RRM neighbour cell measurements (intra- and inter-frequency neighbours)

- RRM serving cell measurements (e.g. L1-RSRP)

- RLM measurements (for RLF detection)

- BFD measurements (for beam management)

- CSI reporting (for link adaptation)

- SRS transmissions (for antenna switching)

When the UE is in low mobility and/or good coverage, i.e. when the relaxation criterion in section 5.7.13 in 38.331 is fulfilled, then the UE may relax RLM/BFD measurements as specified in 38.133. The (e)RedCap UE reports via UAI signalling when the RRM relaxation criterion is fulfilled (see section 5.7.4.4 in 38.331). This reporting enables the gNB to reconfigure the RRM measurements in the (e)RedCap UE for improved power savings:

The existing RLM/BFM relaxation for normal and (e)RedCap UE can be configured when LP-WUS is configured.

Both RAN1 and RAN2 have been focussed on the PDCCH monitoring aspects with LP-WUS. But in the end an integrated solution has to be selected that includes both PDCCH monitoring and RLM/BFD measurements and CSI reporting:

When discussing the LP-WUS triggered PDCCH monitoring, the RLM/BFD measurements and CSI reporting requirements are discussed as well.

Option 1-1

For option 1-1 RAN2 agreed:

- For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

- For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

In our view option 1-1 is the baseline solution for connected mode. Option 1-1 resembles DCP, but with WUR the LR is used to monitor LP-WUS instead of the MR that monitors PDCCH format 2_6. Option 1-1 can provide power saving gains when the UE has little data to send in connected. Furthermore the solutions adopted for DCP can be re-used for LP-WUS (e.g. CSI reporting, LP-WUS monitoring, RAN1/RAN2 specification impact), which will limit the effort to develop the feature, and provide more time for LP-WUS in Idle/Inactive which is prioritized.

Option 1-1 is the baseline solution for LP-WUS in connected. Extensions beyond option 1-1 can be discussed further but have to be well motivated, i.e. LP-WUS in Idle and Connected mode is prioritized and should consume the majority of the time units.

Option 1-2

Option 1-2-1 does not improve the UE power consumption, but it reduces the DL data delay for the first packets that arrive in the traffic burst after an idle period. Connected mode latency is not part of the work item objectives, but an argument can be made that LP-WUS outside Active Time has to potential to improve the downlink latency with a low increase of power consumption using LP-WUS monitoring.

However option 1-1 with a very short DRX cycle can provide the same latency improvement as option 1-2-1 that uses a similar short LP-WUS periodicity. But when option 1-1 is configured with a very short DRX cycle, then the UE has to perform more frequent RRM/RLM/BFD measurements and more frequent CSI/L1-RSRP reporting.

During Active Time when the drx-InactivityTimer is running and data is being send and received, then the UE has to perform frequent RRM/RLM/BFD measurements based on the configured SSB or CSI-RS periodicity (e.g. every 40 ms). During Active Time when the drx-OnDurationTimer is running, this activity is reduced to:

- Single BFD measurement unless beam failure is detected

- Single RLM measurement unless T310 is started

The measurement activity duringthe  drx-OnDurationTimer is thus low compared to during the drx-InactivityTimer. But with a short DRX cycle the UE has to wake-up more frequently which consumes power, and the UE may have to transmit periodic CSI/ L1-RSRP reporting which consumes power:

RAN2 to discuss how RLM/BFD measurements and periodic CSI/L1-RSRP reporting can be relaxed with short DRX cycles with minimum specification and RAN4 impact.

During Rel-17 RLM/BFD measurement relaxation has been discussed in RAN4, but it was not agreed. However the RLM/BFD measurement requirements for e.g. 40 and 160 ms DRX are already defined in 38.133. Perhaps one option is to specify in RAN2 specification that with a 40 ms C-DRX the UE may use the RLM/BFD requirements for n x C-DRX cycle, i.e. the UE applies a relaxation factor n. The reader is reminded that 38.133 does also not explicitly distinguish between short and long DRX, i.e. it just mentions the DRX cycle. Furthermore RAN2 recently clarified in 38.300 w.r.t. RLM/BFD relaxation with short and long DRX configured:

When the short DRX cycle is smaller than or equal to 80 ms and the long DRX cycle is larger than 80 ms then the UE is allowed to perform RLM and/or BFD relaxation while drx-ShortCycleTimer is running and the relaxed measurement criteria are met. In this case the UE does not report change of its relaxation state for RLM and/or BFD relaxation due to start or stop of drx-ShortCycleTimer, if configured.

With option 1-2-1 additional LP-WUS occasions outside Active Time can be configured with a certain periodicity using the LP-WUS configured in front of the drx-OnDurationTimer (option 1-1) as a reference/anchor. Thus option 1-2-1 can be considered and extension of option 1-1 providing additional LP-WUS occasions outside Active Time. When an additional LP-WUS is triggered, either the drx-InactivityTimer or a new timer is started and the UE enters Active Time. Thus when there is no traffic the UE is not required to perform frequent RLM/BFD measurements:

RAN2 to discuss the impact to configure additional periodic LP-WUS occasions outside Active Time relative to the LP-WUS occasion in front of the OnDuration of option 1-1. When an additional LP-WUS occasion outside Active Time is triggered the UE starts either the drx-InactivityTimer or a new timer and enters Active Time.

When and additional or option 1-1 LP-WUS is triggered and the UE starts PDCCH monitoring, then the gNB intends to schedule the UE i.e. the UE should not go to sleep. Furthermore the drx-OnDurationTimer is typically a short time, i.e. shorter than the drx-InactivityTimer, and there is little power savings to use LP-WUS during drx-InactivityTimer. It is also a bit odd when the gNB would first wake-up the UE with an LP-WUS in front of the drx-OnDurationTimer and then the gNB has to send LP-WUS one more time because the UE entered sleep mode:

Option 1-2-2, i.e. LP-WUS monitoring during drx-OnDurationTimer timer or a new timer based on the DRX cycle is not further discussed.

Option 1-3

Option 1-3 concerns PDCCH monitoring triggered by LP-WUS during Active Time (except when the drx-InactivityTimer is running):

But there are existing features like PDCCH skipping, SSSG switching and cross slot scheduling that can be used effectively on this very short time scale where light/micro sleep is possible during Active Time. For example with PDCCH skipping the UE may enter micro or light sleep temporarily while sending and receiving data. The pdcch-SkippingDurations that can be configured are between {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100} ms.

RAN1 can continue to discuss PDCCH monitoring features on this very short time scale as they have done before, and capture it in their specifications. RAN2 should not discuss these enhancements on this very short time scale which do not impact MAC:

Option 1-3 is not further discussed in RAN2.

## Configuration

Concerning the configuration RAN1 agreed:

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN2 can confirm:

PDCCH monitoring triggered by LP-WUS is configured via dedicated RRC signaling.

The OFDM-based WUR is expected to provide full coverage, while OOK-based WUR is not expected to do that. As a baseline based on WUR type and/or RRM/RLM/BFD/CSI reporting the gNB can configure or de-configure LP-WUS in the UE via RRCReconfiguration, e.g. when the UE is outside LP-WUS coverage.

As a baseline RAN2 assumes that based on existing measurement reporting the gNB can determine when to (de)configure LP-WUS PDCCH monitoring.

To avoid ping-pong, i.e. frequent reconfigurations the gNB can decide to configure LP-WUS after a certain time period when the UE has reported good coverage, and immediately de-configure LP-WUS when the quality drops below a certain threshold. However this can be left to gNB implementation.

In Idle/Inactive the gNB does not know if the UE is outside LP-WUS coverage, but if the entry condition is not fulfilled the UE monitors the Paging PDCCH directly. If the entry conditions is fulfilled it is up to the UE implementation to monitor LP-WUS or legacy PO directly.

When the UE is in connected mode and has to monitor Paging PDCCH then the UE uses the idle mode procedures to receive e.g. ETWS/CMAS indication. Provided that the LP-WUS entry condition is met the UE can use LP-WUS monitoring for Paging as well. Similarly it can be discussed whether the LR can use PSS/SSS or LP-SS measurements to decide to monitor LP-WUS or PDCCH directly when PDCCH monitoring triggered by LP-WUS is configured:

RAN2 to discuss whether the UE can be configured to use LP-SS or other measurements to determine whether it can monitor LP-WUS or should monitor PDCCH directly.

Such entry/exit condition for LP-WUS in connected mode may provide additional security that PDCCH performance is not impaired when LP-WUS is configured. There could for example be latency in the RRM reporting of the UE.

When LP-WUS triggered PDCCH monitoring is configured there is no need for signalling to indicate whether the UE can use LP-WUS or the UE is using LP-WUS for PDCCH monitoring, because such signalling can create much signalling overhead. Based on existing measurement reporting the gNB can decide when to configure LP-WUS PDCCH, and when configured the gNB assumes that the UE is using LP-WUS:

When LP-WUS triggered PDCCH monitoring is configured the gNB assumes that the UE monitors LP-WUS to trigger PDCCH monitoring and there is no signalling of the UE status.

RAN1 agreed to use UE capability signalling to determine the time offset between LP-WUS and start of PDCCH monitoring:

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

From RAN2 perspective it is possible to convey the minimum time gap via capability signalling. And capability signalling is more suitable than UE assistance, i.e. the minimum time gap of the UE does not change or some time gaps are preferred over others.

The LP-WUS configuration (e.g. time offset, duration and minimum gap until start of drx-onDurationTimer) can be discussed further when there is progress in RAN1/RAN4.

Similar as with DCP it should be possible to configure the UE to report CSI when the drx-OnDurationTimer is not start started because LP-WUS is not detected:

The UE can be configured to transmit periodic L1-RSRP or other CSI reports during the OnDuration when LP-WUS is not detected.

## Paging PDCCH

In connected mode the UE does not receive Paging PDSCH, but the UE is required to monitor Paging PDCCH, e.g. to receive ETWS/CMAS warning messages. If LP-WUS is configured in SIB, then the UE in connected mode can use LP-WUS to monitor Paging PDCCH:

UE in RRC_CONNECTED can use LP-WUS to monitor Paging PDCCH, including indication of Paging message on PDSCH, SI-change and ETWS/CMAS indication.

## Subgrouping

The OnDuration of UEs in connected mode typically do not overlap, or an attempt is made to minimize overlap. During the OnDuration the gNB may want to reach a specific UE, and when the start of the OnDurations coincide, then the LP-WUS can wake-up multiple UEs and cause false alarms.

The number of “individual” LP-WUS occasions that can be configured in connected mode depends on:

- C-DRX cycle length

- Numerology, i.e. number of slot per C-DRX cycle

- LP-WUS duration (1 or more slots)

- LP-WUS bandwidth (up to 5 Mhz)

The number of required LP-WUS occasions depends on the number of UEs in connected mode, and penetration of the feature, i.e. whether the UE supports the feature.

The current best guess is that with high penetration, low numerology and high number of users the UE cannot be configured with an individual LP-WUS occasion. Thus it may be beneficial to have subgroup information in LP-WUS to reduce false alarms, however this should be configurable dependent on demand:

If needed, subgroup information can be configured in LP-WUS.

The use of bits or codepoints to convey subgroup information can be discussed further, i.e. the needs may be different compared to paging.

## Monitoring rules

RAN1 agreed that the UE in connected mode can monitor MOs with a certain periodicity, derived from the DRX cycle or configured separately:

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for Mos

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

There is no need for multiple MOs in a time window associated with an LO as for beam sweeping reason in idle/inactive. RAN2 should wait for further RAN1 progress.

Similar as with DCP for improved reliability and robustness:

The UE can be configured to start drx-OnDurationTimer when LP-WUS is not detected.

There can be cases when the UE is not able to monitor LP-WUS occasions, because other activities need to be performed. In case the UE cannot monitor the LP-WUS occasion before the following OnDuration, then the UE should monitor PDCCH during the following OnDuration, independent whether LP-WUS was transmitted or not:

When the UE is not able to monitor the LP-WUS MO then the UE starts the drx-OnDurationTimer.

LP-WUS can be configured on multiple BWP, but the UE only monitors the LP-WUS on the active BWP:

The UE only monitors LP-WUS on the active BWP.

When LP-WUS is configured it is up to UE implementation to monitor LP-WUS or monitor PDCCH during the OnDuration directly.

## Carrier Aggregation

RAN1 is studying the use of LP-WUS with Carrier Aggregation (CA):

Agreement

Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

  - Option 1: one or more serving cells based on gNB indication/configuration

  - Option 2: all activated serving cells

  - Note: other options are not precluded

DCP is only configured on the PCell and we have some concerns about the complexity and benefit of supporting LP-WUS on SCell. For example how this would work with cross carrier scheduling.

Furthermore SCell dormancy can be used to enable sleep mode when there is no traffic scheduled on the SCell. And SCells are typically activated/deactivated based on traffic demand, while the sPCell remains activated all the time. Thus when the SCell is activated the SCell can be assumed to be in Active Time with the drx-InactivityTimer most of the time, i.e. option 1-1 and 1-2 provide limited power saving gains for SCell (and we are not in favour of option 1-3):

Due to the expected complexity and limited benefit LP-WUS and CA is not further discussed in RAN2.

# Summary

RAN2 is kindly asked to discuss LP-WUS in connected:

Observation 1	Compared to Idle and Inactive mode the expected power saving gains of LP-WUS in Connected mode are (very) limited because ultra-deep sleep cannot be used and most of the power consumption is determined by data transmission and reception.

Proposal 1	The connected mode solution is kept simple with limited specification impact and limited impact on the different working groups.

Proposal 2	RAN2 should discuss LP-WUS triggered PDCCH monitoring based on legacy or new DRX parameters, which are captured in RAN2 specification. RAN2 should not discuss LP-WUS triggered PDCCH monitoring on a very short time scale of 1 or more mseconds like PDCCH skipping which are captured in RAN1 specification.

Proposal 3	The existing RLM/BFM relaxation for normal and (e)RedCap UE can be configured when LP-WUS is configured.

Proposal 4	When discussing the LP-WUS triggered PDCCH monitoring, the RLM/BFD measurements and CSI reporting requirements are discussed as well.

Proposal 5	Option 1-1 is the baseline solution for LP-WUS in connected. Extensions beyond option 1-1 can be discussed further but have to be well motivated, i.e. LP-WUS in Idle and Connected mode is prioritized and should consume the majority of the time units.

Proposal 6	RAN2 to discuss how RLM/BFD measurements and periodic CSI/L1-RSRP reporting can be relaxed with short DRX cycles with minimum specification and RAN4 impact.

Proposal 7	RAN2 to discuss the impact to configure additional periodic LP-WUS occasions outside Active Time relative to the LP-WUS occasion in front of the OnDuration of option 1-1. When an additional LP-WUS occasion outside Active Time is triggered the UE starts either the drx-InactivityTimer or a new timer and enters Active Time.

Proposal 8	Option 1-2-2, i.e. LP-WUS monitoring during drx-OnDurationTimer timer or a new timer based on the DRX cycle is not further discussed.

Proposal 9	Option 1-3 is not further discussed in RAN2.

Proposal 10	PDCCH monitoring triggered by LP-WUS is configured via dedicated RRC signaling.

Proposal 11	As a baseline RAN2 assumes that based on existing measurement reporting the gNB can determine when to (de)configure LP-WUS PDCCH monitoring.

Proposal 12	RAN2 to discuss whether the UE can be configured to use LP-SS or other measurements to determine whether it can monitor LP-WUS or should monitor PDCCH directly.

Proposal 13	When LP-WUS triggered PDCCH monitoring is configured the gNB assumes that the UE monitors LP-WUS to trigger PDCCH monitoring and there is no signalling of the UE status.

Proposal 14	The UE can be configured to transmit periodic L1-RSRP or other CSI reports during the OnDuration when LP-WUS is not detected.

Proposal 15	UE in RRC_CONNECTED can use LP-WUS to monitor Paging PDCCH, including indication of Paging message on PDSCH, SI-change and ETWS/CMAS indication.

Proposal 16	If needed, subgroup information can be configured in LP-WUS.

Proposal 17	The UE can be configured to start drx-OnDurationTimer when LP-WUS is not detected.

Proposal 18	When the UE is not able to monitor the LP-WUS MO then the UE starts the drx-OnDurationTimer.

Proposal 19	The UE only monitors LP-WUS on the active BWP.

Proposal 20	Due to the expected complexity and limited benefit LP-WUS and CA is not further discussed in RAN2.

# References

- TR 38.869, Study on low-power wake up signal and receiver for NR, RAN1, v18.0.0, Rel-18

- RP-240801, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN#103

# Annex: WID and agreements

## WID

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

- Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3.

- Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

## RAN1 agreements

R1-2401862	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

    - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded.

Agreement

For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits

- FFS value X, which is no more than [8 or 16]

Agreement

For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following

- LP-WUS processing time

- MR transition time for ramp up

- Time/frequency synchronization of MR

- FFS whether UE can report supported minimum time gap from candidate values

FFS: Whether the minimum time gap values can be more than one

Agreement

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

R1-2403714	FL summary#3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

Update the following agreement in RAN1#116 in red:

Agreement

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration

    - FFS duty-cycled and/or continuous LP-WUS monitoring

- Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

R1-2405721	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

- FFS: exact value(s) of the minimum time gap

- FFS: support of multiple minimum time gaps

- FFS whether the reported value includes the duration for time/frequency synchronization of MR

R1-2405592	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state

- FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS

- FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET

Agreement

LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode.

- FFS whether to define a time window for Mos

- It is at least supported that a UE can monitor MOs with a periodicity.

  - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured

R1-2405659	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

  - Option 1: one or more serving cells based on gNB indication/configuration

  - Option 2: all activated serving cells

  - Note: other options are not precluded

Agreement

For RRC CONNECTED mode, LP-WUS can be configured without following existing features.

- Rel-16 DCP

- Rel-17 PDCCH skipping

- Rel-17 SSSG switching

- Rel-18 cell DTX

Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

## RAN2 agreements

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

- For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

- For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

- RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

- The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.