3GPP TSG-RAN-WG2 Meeting #127	R2-2407402

Maastricht, Netherlands, 19th– 23rd Aug, 2024

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	discussion on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN#103 meeting, the new WID was approved [1];

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

According to the WID, RAN2 needs to discuss (A) framework of discovery and (re)selection, (C) SRAP and QoS handling, and (D) C-plane procedure. And RAN2 needs to consider forward compatibility while RAN2 discusses one additional hop relay. Considering future compatibility, the same term (e.g. intermediate relay UE) should be used for the new relays (both for first/second relay UEs), but for convenience, we will use the same term as the WID in this document.

In this document, we consider multi-hop systems as following the requirements specified in TS 22.261[2] (the list of KPIs is provided in the Appendix as an example). And we will consider the TR23.700-03[3] as baseline of the discussion.

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

In the WID, it is assumed that the all UEs are in RRC_CONNECTED state before and after forming a multi-hop PC5 link. And candidate first relay UE is in RRC_CONNECTED state via indirect connection. Therefore, the candidate first relay UE is a remote UE in RRC_CONNECTED state.

Proposal 2. RAN2 considers the candidate first relay UE is a multi/single-hop U2N remote UE in RRC_CONNECTED state.

  - mechanism enhancement for Scenario A and B

Scenario A and B are below;

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

Firstly, we assume following procedure can be baseline of scenario A based on current procedure and SA2’s conclusions.

Figure 1. multi-hop to direct path switching procedure

For the measurement reporting event, Event X1 (Serving L2 U2N Relay UE becomes worse than threshold1 and NR Cell becomes better than threshold2) can be used for this scenario since this path switching scenario is switching from source relay UE to target gNB. And event X2 (Serving L2 U2N Relay UE becomes worse than threshold) may also be used for it.

Proposal 3. Current measurement reporting event X1 and X2 can be reused for M2D (multi-hop indirect to direct) path switching procedure.

Secondary, we assume following procedure can be baseline of scenario B.

Figure 2. multi-hop indirect to single-hop indirect path switching procedure

For the measurement reporting event, Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can be used for this scenario since this path switching scenario is switching to target relay UE. And event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) can also be used for this switch. If the target last relay UE is source last relay UE, this event may not be used since the first relay UE may be closer than the last or other intermediate relay UE from remote UE’s perspective. For this case, Y2 should be configured. However, if the target last relay UE is different from source last relay UE Remote UE, this event (Z1) can be used for it since the reporting event having two thresholds is useful to avoid unnecessary reporting. It is up to gNB to decide how to configure Y2 and/or Z1. Also, event X1 can be used for this scenario.

Proposal 4. Current measurement reporting event X2, Y2 and Z1 can be reused for M2I path switching procedure.

Considering discovery transmission for measurement/U2N relay reselection, in Rel-17, remote/relay UE can transmit the discovery message when the threshold condition is fulfilled if configured. We should consider whether the legacy threshold condition can be used for multi-hop U2N remote UE. Considering latency and risk of link failure, it is generally better to use a link with fewer hops to gNB. If the PC5 link between the remote UE and the first relay UE is very good (i.e., they are almost in the same location), it is reasonable for the remote UE to switch to the last relay UE. However, if the remote UE cannot send a discovery message due to the threshold condition, it cannot report to the gNB that the last relay UE is available. On the other hand, sending unnecessary messages when good enough links are available wastes resources. Therefore, legacy lower threshold for discovery transmission is needed. And RAN2 should discuss whether the upper threshold is needed for the case that remote UE and first relay UE are almost in the same location.

Observation 1. If remote UE and first relay UE are almost in the same location, it is reasonable for the remote UE to switch to the last relay UE.

Proposal 5. Legacy lower threshold for discovery transmission can be reused.

Proposal 6. RAN2 discusses whether the upper threshold is needed to allow discovery transmission for the case that remote UE and first relay UE are almost in the same location.

  - . mechanism enhancement for Scenario C and D

Scenario C and D are below;

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

Following procedures are example of the switch procedures of scenario C.

Figure 3. Intra-gNB direct to multi-hop indirect path switching procedure

For the measurement reporting event, Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) and Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can be used for this scenario since this path switching scenario is switching to target relay UE from source PCell.

Proposal 7. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure.

For scenario D, following procedure can be a baseline.

Figure 4. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure

For the measurement reporting event, Event X2 (Serving L2 U2N Relay UE becomes worse than threshold), event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) and event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) can be used for this scenario since this path switching scenario is switching to target relay UE from source relay UE. This scenario is similar to I2I path switching as with M2I path switching procedure.

Proposal 8. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure.

When we discuss X2M scenarios, we need to know who triggered the discovery message and what path it takes. According to the SA’s potential solution, the discovery procedure has the following characteristics;

- Model A: discovery message is firstly transmitted by “last relay UE.” And, similar to U2U relay discovery, intermediate UEs forward the discovery to the remote UE.

- Model B: discovery message is firstly transmitted by “remote UE.” If first relay UE is last relay UE, first relay UE responses to discovery message directly, else, similar to U2U relay discovery, first relay UE forwards the discovery message to the other relay UE and then responses to discovery message when receives response message from the other relay UE.

Based on this, we need to consider following aspects;

- The condition under which the last relay UE initiates discovery Model A

- The condition under which the first (second, …) relay UE forwards discovery message for Model A

- The condition under which the remote UE initiates discovery Model B and/or relay (re)selection

- The condition under which the first (second, …) relay UE forwards discovery message for Model B

For the condition of transmitting model A Discovery, we think it can be same as legacy condition for U2N relay UE since the last relay UE should always be legacy U2N relay UE. And for the condition of transmitting Model B Discovery, to determine whether the UE will be legacy single-hop U2N remote UE or multi-hop U2N remote UE, the UE needs to send discovery message. Also, multi-hop U2N remote UE should always satisfy the condition for legacy single-hop U2N remote UE. Therefore, the condition of transmitting Model B Discovery can be same as the condition for legacy single-hop U2N remote UE.

Proposal 9. The condition under which the last relay UE initiates discovery Model A should be same as the condition of legacy U2N relay UE.

Proposal 10. The condition under which the remote UE initiates discovery Model B should be same as the condition of legacy U2N remote UE.

For the condition of forwarding Model A Discovery, we think the mechanism of U2U relay discovery can be reused, i.e. first relay UE forwards if the RSRP of the received discovery is above than the threshold. However, if the UE who received discovery message meets condition of legacy U2N relay UE, the UE shouldn’t forward the discovery message. Because in that case, data routing from/to last relay UE to/from first relay UE is wasteful, and it is more efficient that the first relay UE becomes the last relay UE, which is one hop less. Therefore, only UEs that meet the legacy U2N remote UE criteria should be allowed to transmit discovery messages. And this condition can be used for the condition of forwarding Model B Discovery.

Proposal 11. The UE who received Model A/B discovery transmits (transfers) discovery message if the RSRP of the received Model A/B discovery is above the threshold and if the UE meets the condition of legacy U2N remote UE.

The gNB needs to find whether the candidate first relay UE reaches the same gNB. In legacy, remote UE reports the serving cell information of the candidate relay UE to the gNB. And the remote UE considers the serving cell of the relay UE as PCell. Same as the legacy, in multi-hop system, remote UE should report the serving cell information of last relay UE connecting with the candidate first relay UE.

Observation 2. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 12. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

We also consider what information gNB needs to select the appropriate PC5 links. We assume the AS layer evaluation metrics that the gNB uses to select optimal PC5 links are PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE. This allows the gNB to select a path with low latency and/or robustness (i.e., less likely to be disconnected) depending on the QoS and radio resource management of the remote UE. According to the current SA2 discussion, the discovery message may include hop counter, ID of last relay UE and accumulated QoS for the PC5 links (i.e. Achievable PDB) in addition to the legacy contents. Therefore, the remote UE knows these metrics. Then, the gNB knows that the reported first relay UE is connecting with the same gNB or not, the number of hops of the PC5 links to which the reported candidate first relay UE belongs, and the QoS profile for the (potential) remote UE. So, for selecting appropriate path (first relay UE), UE should send the information of PC5 link quality of the path the first relay UE belongs. Furthermore, the greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization. So, we consider that the UE needs to report to the gNB the PC5 link qualities of each hops of the path first relay UE belongs.

Observation 3. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 4. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 5. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 13. Candidate remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs.

Conclusion

Proposal 1. RAN2 will take into account the requirement specified in TS22.261 and the solutions and conclusion in TR23.700-03.

Proposal 2. RAN2 considers the candidate first relay UE is a multi/single-hop U2N remote UE in RRC_CONNECTED state.

Proposal 3. Current measurement reporting event X1 and X2 can be reused for M2D path switching procedure.

Proposal 4. Current measurement reporting event X2, Y2 and Z1 can be reused for M2I path switching procedure.

Observation 1. If remote UE and first relay UE are almost in the same location, it is reasonable for the remote UE to switch to the last relay UE.

Proposal 5. Legacy lower threshold for discovery transmission can be reused.

Proposal 6. RAN2 discusses whether the upper threshold is needed to allow discovery transmission for the case that remote UE and first relay UE are almost in the same location.

Proposal 7. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure.

Proposal 8. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure.

Proposal 9. The condition under which the last relay UE initiates discovery Model A should be same as the condition of legacy U2N relay UE.

Proposal 10. The condition under which the remote UE initiates discovery Model B should be same as the condition of legacy U2N remote UE.

Proposal 11. The UE who received Model A/B discovery transmits (transfers) discovery message if the RSRP of the received Model A/B discovery is above the threshold and if the UE meets the condition of legacy U2N remote UE.

Observation 2. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 12. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

Observation 3. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 4. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 5. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 13. Candidate remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs.

Reference

- RP-241609, New WID on NR sidelink multi-hop relay

- TS22.261, Service requirements for the 5G system

- TR23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3

Appendix

- 7.7	KPIs for UE to network relaying in 5G system

In several scenarios, it can be beneficial to relay communication between one UE and the network via one or more other UEs. The functional requirements related to relaying can be found in clause 6.9.2. Performance requirements for relaying in different scenarios can be found in table 7.7-1.

Table 7.7-1: Key Performance for UE to network relaying

| Scenario | Max. data rate (DL) | Max. data rate (UL) | End-to-end latency (note 7) | Area traffic capacity (DL) | Area traffic capacity (UL) | Area user density | Area | Range of a single hop (note 8) | Estimated number of hops |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| InHome Scenario (note 1) | 1 Gbit/s | 500 Mbit/s | 10 ms | 5 Gbit/s/ home | 2 Gbit/s /home | 50 devices /house | 10 m x 10m – 3 floors | 10 m indoor | 2 to 3 |
| Factory Sensors (note 2) | 100 kbit/s | 5 Mbit/s | 50 ms to 1 s | 1 Gbit/s /factory | 50 Gbit/s /factory | 10000 devices /factory | 100 m x 100 m | 30 m indoor / metallic | 2 to 3 |
| Smart Metering (note 3) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /hectare | 200 x 100 bytes / 15 mins /hectare | 200 devices /hectare | 100 m x 100 m | > 100 m indoor / deep indoor | 2 to 5 |
| Containers (note 4) | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 15000 x 100 bytes / 15 mins /ship | 15000 x 100 bytes / 15 mins /ship | 15000 containers /ship | 400 m x 60 m x 40 m | > 100 m indoor / outdoor / metallic | 3 to 9 |
| Freight Wagons | 100 bytes / 15 mins | 100 bytes / 15 mins | 10 s | 200 x 100 bytes / 15 mins /train | 200 x 100 bytes / 15 mins /train | 120 wagons /train | 1 km | > 100 m outdoor / tunnel | 10 to 15 |
| Public Safety (note 5) | 12 Mbit/s | 12 Mbit/s | 30 ms | 20 Mbit/s /building | 40 Mbit/s /building | 30 devices /building | 100 m x 100 m – 3 floors | > 50 m indoor (floor or stairwell) | 2 to 4 |
| Wearables (note 6) | 10 Mbit/s | 10 Mbit/s | 10 ms | 20 Mbit/s per 100 m2 | 20 Mbit/s per 100 m2 | 10 wearables per 100 m2 | 10 m x 10 m | 10 m indoor / outdoor | 1 to 2 |
| NOTE 1: Area traffic capacity is determined by high bandwidth consuming devices (e.g. ultra HD TVs, VR headsets), the number of devices has been calculated assuming a family of 4 members. NOTE 2: Highest data rate assumes audio sensors with sampling rate of 192 kHz and 24 bits sample size. NOTE 3: Three meters (gas, water, electricity) per house, medium density of 50 to 70 houses per hectare. NOTE 4: A large containership with a mix of 20 foot and 40 foot containers is assumed. NOTE 5: A mix of MCPTT, MCVideo, and MCData is assumed. Average 3 devices per firefighter / police officer, of which one video device. Area traffic based on 1080 p, 60 fps is 12 Mbit/s video, with an activity factor of 30% in uplink (30% of devices transmit simultaneously at high bitrate) and 15% in downlink. NOTE 6: Communication for wearables is relayed via a UE. This relay UE can use a further relay UE. NOTE 7: End-to-end latency implies that all hops are included. NOTE 8: 'Metallic' implies an environment with a lot of metal obstructions (e.g. machinery, containers). 'Deep indoor' implies that there can be concrete walls / floors between the devices. NOTE 9: All the values in this table are example values and not strict requirements. |  |  |  |  |  |  |  |  |  |