3GPP TSG-RAN-WG2 Meeting #127	R2-2407403

Maastrihit, Netherlands, 19th– 23rd Aug, 2024

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	discussion on C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN#103 meeting, the new WID was approved [1];

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

In this paper, we discuss how to introduce multi-hop relay. As needed, we’ll refer to the SA2’s conclusion [2], use case of multi-hop relaying [3] and requirement of multi-hop relaying [4].

Discussion

According to the WID, RAN2 need to discuss (A) framework of discovery and (re)selection, (C) SRAP and QoS handling, and (D) C-plane procedure. And RAN2 needs to consider forward compatibility while RAN2 discusses one additional hop relay. Considering future compatibility, the same term (e.g. intermediate relay UE) should be used for the new relays (both for first/second relay UEs), but we will use individual UE names used in the WID where appropriate.

We also understand from Ref. 4 that, in principle, multi-hop relays are deployed to save power and extend coverage. Then, the required QoS should be satisfied.

C-plane procedures for Scenarios (A/B/C/D)

Firstly, we assume following procedure can be baseline of scenario A and B based on current procedure and Ref. 2.

Figure 1. Intra-gNB multi-hop indirect to direct path switching procedure

Figure2-a. Intra-gNB multi-hop indirect to single-hop indirect path switching by remote UE

Figure2-b. Intra-gNB multi-hop indirect to single-hop indirect path switching by first relay UE

Firstly, RAN2 needs to decide an overall procedure to discuss details. So, we propose that figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure.

Proposal 1. Figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure (scenario A).

For scenario B, we can consider two procedures, one is the procedure which the remote UE switches the path as shown in Fig.2-a, another one is the procedure which the first relay UE switches the path while keeping the path with the remote UE as shown in Fig.2-b. The procedure shown in Fig.2-a is used, for example, in case of public safety, and the procedure shown in Fig.2-b is used, for example, in case of wearable. Furthermore, most of the steps in the fig.2-b are supported by legacy I2D (indirect to direct) path switching procedure. Therefore, RAN2 should consider these two procedures for scenario B.

Proposal 2. Figure 2-a and Figure 2-b should be considered as baseline for the M2I (multi-hop indirect to single-hop indirect) path switching procedure (scenario B).

Another point of contention, in fig.2-a, is whether the target last relay UE and the source last relay UE can be the same UE. This type of case occurs when relay UEs are arranged in a straight line, and such a case is included in Ref. 4 as one of the use cases. Also, even if the target last relay UE were the source last relay UE, the gNB would know that based on the measurement report. Therefore, there is no problem at this time with the target last relay UE being the source last relay UE.

Proposal 3. The target last relay UE and the source last relay UE can be the same UE

In fig.1 and fig.2-a, we need to consider whether to release the first relay UE from the path. According to WID, “The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.” Based on this, we assume that first relay UE (intermediate relay UEs) is in RRC_CONNECTED state throughout MH relay operation and only has RRC connection through the last relay UE. It means the first relay UE (intermediate relay UEs) is acting as a relay UE and a remote UE. Therefore, gNB doesn’t need to release the PC5 connection between the last relay UE and the first relay UE when the remote UE switches the path.

Proposal 4. The last relay UE and other intermediate relay UEs are always in RRC_CONNECTED state throughout MH relay operation.

Proposal 5. The other intermediate relay UEs (first relay UE, second relay UE, …) are acting as a U2N relay UE and as a U2N remote UE.

Proposal 6. No need to release the PC5 connection between the last relay UE and the first relay UE when the remote UE switches the path.

Secondary, we assume following procedure can be baseline of scenario C and D based on current procedure and SA2’s conclusions.

Figure 3. Intra-gNB direct to multi-hop indirect path switching procedure

Figure 4-a. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by remote UE

Figure 4-b. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by relay UE

To clarify the scenario D, we consider whether the source last relay UE can be the target first relay UE. In a U2N relay use case, the remote UE may be a low-power device, e.g. wearable. In this case, remote UE may move with the source last relay UE. Therefore, there exists a scenario where the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE. Not covering this scenario would limit the use cases of Rel-19 multi-hop relay.

Figure 5. One possible case of scenario D

Proposal 7. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

We illustrated the case that the source last relay UE is the target first relay UE in fig.4-b. In fig.4-b, most of the procedures are supported by legacy D2I (direct to indirect) path switching procedure. If P4 is confirmed, RAN2 should consider fig. 3/4-a/4-b as baseline of scenario C and D.

Proposal 8. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

Proposal 9. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

Configuration for multi-hop relaying

RRC connection maintenance

As before, the appropriate PC5/RRC connections are released upon the path switching. Then, RAN2 need to clarify which UE establishes a connection between UEs. Firstly, in legacy L2 U2N relaying, there are three connections, i.e. remote UE-to-relay UE, remote UE-to-gNB, relay UE-to-gNB. These connections are needed for multi-hop relaying since these connections are the minimum required.

Observation 1. Remote UE-to-last relay UE RRC connection, remote UE-to-gNB RRC connection and relay UE-to-gNB RRC connection are needed for multi-hop relaying.

In multi-path relaying, first relay UE joins to the path between remote UE and last relay UE. We can consider following alternatives;

- Alt 1: Remote UE has a PC5-RRC connection between remote UE and first relay UE and an RRC connection between remote UE and gNB.

- Alt 2: Remote UE has a PC5-RRC connection between remote UE and first relay UE, a PC5-RRC connection between remote UE and last relay UE (using U2U relay architecture) and an RRC connection between remote UE and gNB.

Figure 5. possible scenarios of RRC connections

As a premise, E2E RRC connection (green(#1) and light blue(#2) lines) is essential for measurement reporting and RRC reconfiguration. Also, a PC5-RRC connection is required for each hop to manage each hop. The advantage of Alt.2 is that it can reuse the procedures of U2U relay, but there are too many RRC connections and low scalability to additional two hops since there is no U2U multi-hop relaying related specification. Furthermore, Alt.2 implicitly supports U2U multi-hop relay. For Alt.1, there are minimum required RRC connections and high scalability to more than one hops. Also, in Alt.1, the mechanism of U2U relaying cannot be completely reused, but some of U2U relay's procedures and messages can be reused. Therefore, Alt 1 is reasonable scenario.

Proposal 10. As with legacy, Remote UE only has a PC5-RRC connection between remote UE and first relay UE and an E2E RRC connection between remote UE and gNB.

For supporting SIB/paging delivery, we should discuss whether the current mechanism can be reused. Firstly, current paging delivery is supported by following mechanism [5];

- Method 1: The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space

- Method 2: The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).

Method 1 can be available if the relay UE can receive paging message. For that purpose, the relay UE should be the last relay UE in multi-hop relay, it means that PC5-RRC connection between remote UE and the last relay UE is required. Furthermore, if RAN2 supports PC5-RRC connection between remote UE and the last relay UE, RAN2 implicitly support U2U multi-hop relay (i.e. violate the WID definition). Therefore, Method 1 cannot be used for multi-hop relay.

Method 2 can be available if the first relay UE is in RRC_CONNECTED. If the first relay UE is in IDLE/INACTIVE, second/last relay UE needs to monitor first relay UE’s (and/or remote UE’s) POs or gNB sends dedicated RRC message including paging message to the second/last relay UE, and then gNB sends dedicated RRC message including paging message to the second/last relay UE. It seems wasteful from the perspective of latency and radio resource. Therefore, last relay UE and other intermediate UEs should always be in RRC_CONNECTED. And Method 2 can be used for multi-hop relay if the first relay UE is in RRC_CONNECTED.

Observation 2. The first relay UE cannot monitor POs of its connected remote UE(s). The last relay UE can monitor POs of its connected remote UE(s), but it needs E2E PC5 RRC connection(s) between the last relay UE and its connected remote UE(s).

Proposal 11. Paging message for remote UE is always delivered by dedicated RRC message to the first relay UE. The first relay UE doesn’t deliver paging message for its connected remote UE via PC5-RRC message.

Secondary, current SIB delivery is supported by following mechanism [5];

- The remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected relay UE and receive the SIB(s) from the relay UE via PC5-RRC message.

- The remote UE in RRC_CONNECTED receives SIB1 and other SIBs by dedicated RRC message and performs on-demand SI request.

- For SIB1 forwarding, for L2 U2N Remote UE, both request-based delivery and unsolicited forwarding are supported by L2 U2N Relay UE. If SIB1 changes, for L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE always forwards SIB1.

The last relay UE may not monitor SIB(s) directly on Uu interface. However, relay UE in RRC_CONNECTED can performs on-demand SI request for the requested SIB(s). We see no particular problems in applying these mechanisms to multi-hop relaying. So, legacy SIB delivery mechanism can be reused for multi-hop relaying.

Proposal 12. The legacy SI retrieval mechanisms (including SIB delivery) can be reused for multi-hop relaying.

If RAN2 determines that legacy SIB/paging delivery works in multi-hop system, remote UE can be in any RRC states.

Proposal 13. remote UE can be in any RRC states if legacy SIB/paging delivery works in multi-hop system.

In U2U relay, there is no rule regarding the order of initiating establishment of connections. Also, in U2N relay, it is up to gNB and remote UE. By the way, in both cases, E2E connection is considered as established after hop-by-hop connections are established. Therefore, in MH U2N relay, E2E RRC connection is connected after path switch procedure.

In R17, RAN2 introduced the Timer (T420) to manage whether the path switch is successful or not. We think T420 can be reused for MH U2N relay path switch. In U2U relay, the need for a Timer (T400) with double the existing value was recognized to consider the E2E delays caused by the increased number of hops. However, stop condition of T420 is sending RRCReconfigurationComplete message, i.e. when remote UE establishes PC5-RRC connection with relay UE and complete configuring PC5 RLC channel. It means that the increase in hop count has no relation to the value of T420.

Proposal 14. T420 can be reused for path switch to multi-hop indirect path without value expansion.

For path failure handling, we can consider legacy mechanism as baseline. In legacy, there are some failure cases;

- U2N relay UE detects Uu RLF/ detects RRC failure/ receives HO command/ initiates cell-reselection

- U2U relay UE detects PC5 RLF

- U2N remote UE detects PC5 RLF/ detects RRC failure

- U2U remote UE detects PC5 RLF/ detects PC5 RRC failure

In case of multi-hop relay, intermediate relay UE is like a U2N remote UE and a U2U relay UE. Therefore, there are following failure case from remote UE’s point of view;

- Last relay UE detects Uu RLF

- Last relay UE detects RRC failure

- Last relay UE receives HO command

- Any of the intermediate UEs detects PC5 RLF

- Any of the intermediate UEs detects RRC failure

- Any of the intermediate UEs receives path failure notification

- Remote UE detects PC5 RLF

- Remote UE detects RRC failure

- Remote UE receives path failure notification

In case of intermediate UE’s failure, legacy NotificationMessageSidelink can be reused. If the intermediate UEs detecting any failure needs to notify remote UE(s) of the failure, there should be E2E PC5-RRC connections for each UE combination. However, as mentioned above, establishment of E2E PC5-RRC connections for each UE combination will increase maintenance costs and complicate the system. Therefore, UE should use hop-by-hop PC5-RRC message in order to send a notification to remote UEs.

Proposal 15. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 16. NotificationMessageSidelink should be sent hop-by-hop if needed.

Here, we consider the case where failure information is notified for all remote UEs if last relay UE or intermediate UE detects failure. Firstly, the UE detecting failure notifies the failure to the nearest UE located on the branch side of the tree, and then the nearest UE notifies the failure to other nearest UE(s). Thus, all remote UEs in the outer branch of the tree to which the UE detected the failure belongs will perform RRC re-establishment. This behavior needs huge radio resource instantaneously. In addition, remote UE configured with multi-hop relay is unlikely to discover a new cell, and is also unlikely to discover any relay UE other than the currently connected relay UE. Therefore, there are cases where it is effective for a remote UE configured for multi-hop relay to remain connected to the relay and wait for the relay to reconnect to the gNB. It means that the remote UE doesn’t have to perform RRC re-establishment if a connected (intermediate) relay UE initiates RRC re-establishment procedure.

Observation 3. There are cases where it is effective for a remote UE configured for multi-hop relay to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 17. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

We also consider the content when UE received the notification send a notification to another UE(s). One option is setting the same indicationType with the received notification. However, UE cannot understand whether the event indicated by the indicationType occurring in the UE sent the notification, or the UE simply reflecting it. If UE doesn’t have to perform RRC re-establishment, such information is important to determine UE behavior. Then, another option is setting specific indicationType (e.g. receiving_Notification, perform_reestablish) for multi-hop.

Proposal 18. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

SRAP configuration

Before discussing protocol details, RAN2 needs to have a common understanding of the protocol stack. In our understanding, the SRAP layer was introduced for packet routing. Thus, it should be reused for multi-hop relay as like the legacy U2N relay.

Figure 7. candidate protocol stack for C-plane in multi-hop relay

Proposal 19. RAN2 considers protocol stack as shown in fig. 7 is baseline for discussing protocol details.

SRAP layer supports the routing function, i.e. determination of egress RLC channel/link, determination of UE/BEARER ID field and data transfer. For routing function, gNB configures relay UE(s) and remote UE with local UE ID and mapping rule. Then, the relay UE(s) and remote UE determines the route of the packet based on the SRAP header of the packet. There are two types of SRAP header, one is for U2N SRAP data PDU and another one is for U2U SRAP data PDU. The both of SRAP header has common fields, i.e. D/C field, BEARER ID field and UE ID field(s). D/C field is exactly same. So, it can be reused for multi-hop relay. The value of BEARER ID refers appropriate bearer identity. It means that, in case of U2N relay, the value refers srb-Identity or drb-Identity (minus 1) and, in case of U2U relay, the value refers SL-SRB identity or slrb-PC5-ConfigIndex. In the multi-hop case, the U2N relay format can be reused since E2E RB is not sidelink specific bearer. The discussion point is UE ID field. In case of the U2N relay, one UE ID of remote UE’s local ID and, in case of U2U relay, two UE IDs of source remote UE and destination remote UE. In our understanding, two ID is needed for the destination remote UE that receives the SRAP PDU to determine to which PDCP the corresponding SRAP SDU should be transferred if E2E SL-DRBs from multiple source UEs are mapped to one ingress PC5 RLC channel. In case of multi-hop relay, such a situation occurs in gNB. Then, gNB understands who source UE is based on one UE ID field (i.e. remote UE’s local ID). Also, the intermediate UEs can understand whether the SRAP SDU is UL or DL based on the received RLC even if there is only one UE ID, because one RLC entity cannot be established for multiple destinations. Therefore, SRAP header for legacy U2N relay can be reused.

Proposal 20. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 21. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay, i.e. remote UE’s local ID is set to the UE ID.

And, as with legacy, gNB should allocate local ID for each remote UE for managing sources of UL data and destinations of DL data. And gNB can avoid local ID collisions.

Proposal 22. As with legacy, gNB should allocate local ID for each remote UE.

Then, legacy mapping rule should be configured for each UEs. Therefore, when a remote UE switches to multi-hop path, last relay UE and intermediate UEs should be reconfigured with SRAP configuration for the remote UE. While using the RRC message to allocate the local ID, the gNB can provide the mapping configuration for each UE.

Proposal 23. When a remote UE switches to multi-hop path, last relay UE and intermediate UEs should be reconfigured with SRAP configuration for the remote UE by RRC reconfiguration as with legacy.

RLC channel configuration for each relay

QoS handling

The requirement is important when we consider the QoS handling. Especially, some use cases, e.g. public safety, wearable and V2X, needs very low E2E latency. Therefore, optimization should be done to achieve low latency.

Proposal 24. QoS related optimization should be considered since very low E2E latency is required for some use cases.

When we consider QoS related optimization, it is important to know achievable QoS at each hop. Here, gNB can guarantee at least one hop (i.e. last relay hop) QoS if gNB allocate sidelink and/or uplink grant for the last relay UE using resource allocation mode 1. Thus, we propose that the principles of resource allocation for U2N relay operation follow the legacy mechanism.

Proposal 25. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 4. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

In U2U relay, U2U relay UE split the E2E QoS for each hop, and then each UE configures each hop configuration to guarantee the split QoS. Because U2U relay UE knows quality of both hops and can configure appropriate values. In multi-ho relay, there is no special UE like a U2U relay UE. However, if gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality. Thus, we propose that the gNB sets the appropriate (split) QoS to be met for each hop and UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Observation 5. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 26. The gNB sets the appropriate (split) QoS to be met for each hop.

Proposal 27. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

For QoS parameters to be split, same as U2U relay, only PDB should be split. For example, split PER extremely decrease as the number of hops increases, making it difficult to divide it accurately. Therefore, we consider it a compromise in the multi-hop relay system to assume that gNB sets appropriate values for each hop.

Proposal 28. PDB should be split by the gNB, and it is up to gNB how to split the QoS parameters including PDB.

Conclusion

Proposal 1. Figure1 should be considered as baseline for the M2D (multi-hop indirect to direct) path switching procedure (scenario A).

Proposal 2. Figure 2-a and Figure 2-b should be considered as baseline for the M2I (multi-hop indirect to single-hop indirect) path switching procedure (scenario B).

Proposal 3. The target last relay UE and the source last relay UE can be the same UE

Proposal 4. The last relay UE and other intermediate relay UEs are always in RRC_CONNECTED state throughout MH relay operation.

Proposal 5. The other intermediate relay UEs (first relay UE, second relay UE, …) are acting as a U2N relay UE and as a U2N remote UE.

Proposal 6. No need to release the PC5 connection between the last relay UE and the first relay UE when the remote UE switches the path.

Proposal 7. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

Proposal 8. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

Proposal 9. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

Observation 1. Remote UE-to-last relay UE RRC connection, remote UE-to-gNB RRC connection and relay UE-to-gNB RRC connection are needed for multi-hop relaying.

Proposal 10. As with legacy, Remote UE only has a PC5-RRC connection between remote UE and first relay UE and an E2E RRC connection between remote UE and gNB.

Observation 2. The first relay UE cannot monitor POs of its connected remote UE(s). The last relay UE can monitor POs of its connected remote UE(s), but it needs E2E PC5 RRC connection(s) between the last relay UE and its connected remote UE(s).

Proposal 11. Paging message for remote UE is always delivered by dedicated RRC message to the first relay UE. The first relay UE doesn’t deliver paging message for its connected remote UE via PC5-RRC message.

Proposal 12. The legacy SI retrieval mechanisms (including SIB delivery) can be reused for multi-hop relaying.

Proposal 13. remote UE can be in any RRC states if legacy SIB/paging delivery works in multi-hop system.

Proposal 14. T420 can be reused for path switch to multi-hop indirect path without value expansion.

Proposal 15. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 16. NotificationMessageSidelink should be sent hop-by-hop if needed.

Observation 3. There are cases where it is effective for a remote UE configured for multi-hop relay to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 17. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

Proposal 18. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

Proposal 19. RAN2 considers protocol stack as shown in fig. 7 is baseline for discussing protocol details.

Proposal 20. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 21. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay, i.e. remote UE’s local ID is set to the UE ID.

Proposal 22. As with legacy, gNB should allocate local ID for each remote UE.

Proposal 23. When a remote UE switches to multi-hop path, last relay UE and intermediate UEs should be reconfigured with SRAP configuration for the remote UE by RRC reconfiguration as with legacy.

RLC channel configuration for each relay

Proposal 24. QoS related optimization should be considered since very low E2E latency is required for some use cases.

Proposal 25. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 4. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

Observation 5. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 26. The gNB sets the appropriate (split) QoS to be met for each hop.

Proposal 27. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Proposal 28. PDB should be split by the gNB, and it is up to gNB how to split the QoS parameters including PDB.

Reference

- RP-241609, New WID on NR sidelink multi-hop relay

- TR23.700-03, Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 3

- TR23.866, Study on system improvements for energy efficiency

- TS22.261, Service requirements for the 5G system

- TS38.300, NR; NR and NG-RAN Overall description; Stage-2