3GPP TSG-RAN WG2 Meeting #127	R2-2407404

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.7.2

Source:	Nokia

Title:	Multi-modality support

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Multi-modality support has been discussed in several RAN2 meetings and the following agreement was made in RAN2 #126:

Multi-modality awareness at RAN - benefits - Support Multi-Modality awareness in RAN in Rel-19 for UL and DL. - Ask SA2 whether/what information could be provided to RAN - Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows. - Can consider while drafting the LS to indicate the potential benefits - Put SA4, RAN3 in cc - Remove “Although discussions are still ongoing in RAN2 about the potential RAN mechanisms and its gains“ - Add “and their gains in next RAN2 meeting” in the second paragraph - Change “to continue the discussions about the potential RAN” to “to continue the discussions which potential RAN” - We clarify that we are still in study phase and that we need to understand whether/what information we can get from SA2 (e.g. MMSID, synchronization thresholds) to conclude the study. - Change “In this meeting, as part of this study, RAN2 has agreed that RAN awareness of multi-modality is supported for both UL and DL.” To “In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL.” - Change “MMSID or synchronization thresholds” to “MMSID and/or synchronization thresholds” - Remove “CC SA4, RAN3” from actions - With the changes above, the LS is approved unseen in R2-2405782

# 2	Discussion

## 2.1	Multi-modality information to RAN

In RAN2#126, an LS has been sent to SA2 [R2-2405782] with the questions on whether and what information can be provided from SA2 to RAN, e.g., MMSID and/or synchronization thresholds. It is worth to mention that extensive study on coordinated/synchronized delivery has been done in SA2 during Rel-18 SA2 study item. In the end, SA2 agreed to only introduce MMSID which can be used by PCF to derive the correct PCC rules and appropriate QoS policies for example packet delay budget, partially due to the relatively relaxed synchronization requirements according to SA1 [Table 6.43.1-1, TS 22.261]. In SA2#163, it has been discussed if any multi-modality information can be provided to RAN based on the LS from RAN2. However, no consensus has been made in SA2, which is the same situation as Rel-18 SA2 study item.

Observation 1: In SA2, there is no consensus reached in providing MMSID to RAN, which is the same situation as Rel-18 SA2 study item.

For UL, one can still argue that multi-modality information should be reported by the UE so that the gNB utilizes such information for uplink scheduling. However, it is questionable what kind of RAN enhancement is necessary given the statement in TS 23.700-600 that There is no need for millisecond level synchronized delivery within RAN/5GS. Currently, even if the gNB does not know the multi-modality information, as long as multi-modal flows data are generated around the same time (even with some jitter), those data will also likely be scheduled together – via the same or different MAC PDUs – based on BSR/SR. It would be a bad implementation or strange assumption that the gNB starts scheduling of some data (e.g., video stream) but intentionally delays scheduling of the other data (e.g., haptic stream). In addition, 'synchronization requirement' is unclear from RAN2 perspective – e.g., How can it be measured in RAN? Therefore, it is difficult to set the criteria or understand the target of utilizing multi-modality information in RAN, which may be the reason why we have not yet seen any tangible gain by RAN enhancement so far.

For DL, all the scheduling and policy control is up to gNB implementation. Like UL, there has been no clear issue identified for DL.

Observation 2: In RAN2, there has not been sufficient study on identifying any issue caused by lack of multi-modality information in RAN. In addition, there has been no tangible gain shown by any RAN enhancement using multi-modality information.

Proposal 1: As a conclusion of the study of multi-modality, RAN2 confirm that the study on the need of having multi-modality information at RAN side is not enough to continue the discussion in work phase.

## 2.2	DRX enhancement

It is proposed to enhance the DRX operation, e.g., to support multiple active DRX configurations per cell so that the data from multi-modal flows with different periodicity are scheduled via corresponding on-duration. Some simulation results are shown in [R2-2404512]. The simulation results, however, seems to only show that there is a trade-off between power saving gain and the number of satisfied UEs, i.e., the less power is saved, the larger the number of satisfied UEs. Moreover, the active time is determined by various timers in DRX, e.g., drx-InactivityTimer and drx-RetransmissionTimerDL/UL. With these timers, once the scheduling starts, the UE stays in activate time until no further scheduling is done/expected. Therefore, it is not necessary for scheduling to align the on-duration to every occurrence of multi-modal flow data.

Observation 3: Scheduling of different periodicities of multi-modal data does not require additional on-duration via different DRX configurations since they can be scheduled once the UE is in active time by any DRX timer.

Proposal 2: As a conclusion of the study of multi-modality, RAN2 confirms that multiple active DRX is not much studied and justified for support of multi-modal data, and thus it is not enough to continue the discussion in work phase.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: In SA2, there is no consensus reached in providing MMSID to RAN, which is the same situation as Rel-18 SA2 study item.

Observation 2: In RAN2, there has not been sufficient study on identifying an issue caused by lack of multi-modality information in RAN. In addition, there has been no tangible gain shown by any RAN enhancement using multi-modality information.

Proposal 1: As a conclusion of the study of multi-modality, RAN2 confirm that the study on the need of having multi-modality information at RAN side is not enough to continue the discussion in work phase.

Observation 3: Scheduling of different periodicities of multi-modal data does not require additional on duration via different DRX configurations since they can be scheduled once the UE is in active time by any DRX timer.

Proposal 2: As a conclusion of the study of multi-modality, RAN2 confirm that multiple active DRX is not much studied and justified for support of multi-modal data, and thus it is not enough to continue the discussion in work phase.