3GPP TSG-RAN WG2 Meeting #127	R2-2407406

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.4.4

Source:	Nokia

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

For LP-WUS in RRC_CONNECTED, RAN1 made some progress in RAN1#117:

Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for Mos - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

In RAN2#126, RAN2 also discussed LP-WUS for RRC_CONNECTED and agreed on the followings:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

# 2	Discussion

## 2.1	Option 1-2-2

In RAN1, the option 1-2-2 is clarified as follows:

PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS.

During study phase in RAN2, the option 1-2 was considered for LP-WUS operation, where the UE starts active time upon receiving LP-WUS outside C-DRX active time [TR 38.869]. In RAN2#126, there seemed to be various understanding on option 1-2-2, as shown in the meeting munities [R2-2405702].

?? RAN2 will further discuss Option 1-2-2 based on the following understanding for it: - [option A] LP-WUS does not impact PDCCH monitoring in legacy active time other than legacy drx-onDurationTimer. Or?? - [option B] For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, LP-WUS only impacts the PDCCH monitoring outside C-DRX Active Time and during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than drx-onDurationTimer, even no LP-WUS has been detected. Or?? - [option C] Upon the detection of LP-WUS, the UE starts active time regardless of C-DRX config. How exactly this is done is FFS. Once UE starts PDCCH monitoring, it follows legacy active time definition.

Option A generally describes the potential impact on On duration, but it seems unclear how On duration is impacted.

Option B provides a more detailed explanation of the impact on On duration by introducing LP-WUS monitoring. In detail, option B aims at replacing On duration by LP-WUS monitoring occasions since On duration is considered the most uncertain for whether the UE will actually receive the PDCCH or not compared to other active time by e.g., retransmission timer [R2-2405935]. Once LP-WUS is received outside C-DRX active time, the UE starts active time by starting e.g., a new timer or drx-InactivityTimer. Once the UE exits active time, e.g., when a new timer or drx-InactivityTimer expires, the UE starts monitoring of LP-WUS. In option B, there seems to be no On Duration where the UE monitors the PDCCH, but On duration may still be necessary for e.g., measurement report.

Option C is similar to option B in a way to start C-DRX active time when detecting LP-WUS outside C-DRX active time. The difference may be how to switch back to LP-WUS monitoring and whether to allow PDCCH monitoring on On Duration. In option C, the scheduling activity may be used as one criterion to switch back to LP-WUS monitoring. For instance, the UE may go back to LP-WUS monitoring if e.g., a Long DRX cycle is used. In this case, different from option B, the UE keeps monitoring the PDCCH in On duration until the UE starts monitoring of LP-WUS.

As a starting point, it would be good to confirm that the UE starts active time if LP-WUS is received outside C-DRX active time.

Proposal 1: For option 1-2-2, the UE starts C-DRX active time if LP-WUS is received outside C-DRX active time. While the UE is in C-DRX active time, the UE follows legacy C-DRX operation.

Currently, active time is controlled by DRX timers. Thus, RAN2 need to discuss which timer is started upon receiving LP-WUS.

-	Option A. drx-InactivityTimer

-	Option B. A new timer

For both timers, the UE would start LP-WUS monitoring when the timer expires. One benefit of introducing a new timer would be that it can be configured by considering the wake-up time, whereas the drx-InactivityTimer is set by considering only the expected scheduling gap between new transmissions. The new timer may be stopped when PDCCH is received because legacy C-DRX operation will be followed by PDCCH reception.

Proposal 2: For option 1-2-2, RAN2 discuss whether to start a new timer or drx-InactivityTimer when the UE detects the LP-WUS outside C-DRX active time.

Once the UE starts PDCCH monitoring, the legacy C-DRX operation will be followed. The question is how or when to go back to LP-WUS monitoring, i.e., to stop PDCCH monitoring.

-	Option A. When the timer (which is related to DRX active time or a new timer) expires

-	Option B. When Long DRX cycle is started

Option A is to switch back to LP-WUS monitoring immediately when the ongoing scheduling ends or the UE does not actually receive any PDCCH after being woke up by LP-WUS. Meanwhile, option B is to give more time to ensure that there will be no further scheduling. Considering the wake-up time required to resume PDCCH monitoring, which is not very short and can be a few milliseconds, it seems preferable to go back to LP-WUS monitoring when it is highly likely that there will be no further scheduling.

Proposal 3: For option 1-2-2, RAN2 discuss how to go back to LP-WUS monitoring, i.e., whether to be based on DRX timers or DRX cycle.

## 2.2	Option 1-2-1

Option 1-2-1 has not yet been discussed in RAN2, but it is simply to provide additional scheduling opportunity by allocating LP-WUS monitoring occasions on top of C-DRX active time. Thus, PDCCH monitoring would be additionally triggered by detecting LP-WUS. RAN1 discussed that option 1-2-1 may need to be considered with option 1-1 to gain some power saving benefits compared to existing C-DRX.

The power saving gain from option 1-2-1 may be marginal because it additionally allocates scheduling opportunities. Meanwhile, option 1-2-1 can be easily discussed once option 1-2-2 and option 1-1 are finalized because option 1-2-2 requires the interaction between LP-WUS and PDCCH monitoring to be specified, and option 1-1 will define how the LP-WUS monitoring occasion can be configured, which can be reused in option 1-2-1.

Therefore, we suggest to discuss/support option 1-2-1 once option 1-1 and option 1-2-2 are finalized.

Proposal 4: Option 1-2-1 can be discussed/supported when option 1-1 and option 1-2-2 are finalized.

## 2.3	Option 1-3

Option 1-3 is understood that the UE monitors LP-WUS inside C-DRX active time. As concluded from the study [TR 38.869], the power saving gain is expected by reducing unnecessary PDCCH monitoring within C-DRX active time. However, there may be limited use of this option 1-3 considering the length of On duration [R2-2404380].

In our view, we already have some mechanism to end or reduce unnecessary PDCCH monitoring during active time, e.g., sending DRX command MAC CE or PDCCH skipping. Thus, it would be desirable to focus more on first two options, i.e., option 1-1 and option 1-2-1.

Proposal 5: Option 1-3 is excluded in RAN2.

## 2.4	gNB awareness

RAN1 has agreed that LP-WUS monitoring by UE is known to gNB and it is FFS whether implicit/explicit indication from UE is necessary. It should be noted that the coverage of LP-WUS can be different than the NR coverage and devices may equipped with different type of LR receiver with different performance. For these reasons NW is not always aware whether the UE is monitoring the LP-WUS or not. If the UE is not monitoring LP-WUS then NW should be aware of this, because otherwise NW resources are wasted for LP-WUS, which is not monitored by the UE.

Proposal 6: RAN2 to discuss explicit UE indication on LP-WUS monitoring status.

## 2.5	SPS/CG

For RRC_CONNECTED, RAN1 agreed that LP-WUS indicates to wake-up PDCCH monitoring. It was further agreed that RLM/BFD/CSI as well as RRM measurements are performed by the MR. However, it is unclear whether the MR keeps the SPS and CG resources activated while monitoring the LP-WUS.

If SPS and CG are assumed deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the MR and then activates the SPS/CG via DCI, i.e., the activation command. This would introduce latency and power consumption because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in the MR, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power.

Thus, RAN2 needs to discuss how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 7: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.

# 3	Conclusion

In this contribution, we discuss each option – what needs to be decided and which option need to be discussed first:

Proposal 1: For option 1-2-2, the UE starts C-DRX active time if LP-WUS is received outside C-DRX active time. While the UE is in C-DRX active time, the UE follows legacy C-DRX operation.

Proposal 2: For option 1-2-2, RAN2 discuss whether to start a new timer or drx-InactivityTimer when the UE detects the LP-WUS outside C-DRX active time.

Proposal 3: For option 1-2-2, RAN2 discuss how to go back to LP-WUS monitoring, i.e., whether to be based on DRX timers or DRX cycle.

Proposal 4: Option 1-2-1 can be discussed/supported when option 1-1 and option 1-2-2 are finalized.

Proposal 5: Option 1-3 is excluded in RAN2.

Proposal 6: RAN2 to discuss explicit UE indication on LP-WUS monitoring status.

Proposal 7: RAN2 to discuss the LP-WUS impact on SPS and CG Type 2.