3GPP TSG-RAN-WG2 Meeting #127	R2-2407407

Maastricht, Netherland, 19th– 23rd Aug, 2024

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for enhancement mobility was approved and RAN2 started discussion on the WI. Also, RAN2 started discussion and reached some agreements as follows [2];

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. - Xn-based inter-CU LTM is prioritized in Rel-19. - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). - The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. - For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. - The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations . - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. - FFS on conditional LTM. - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. - Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. - The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this paper, we discuss inter-CU LTM related potential issues.

Discussion

Scenario

In RAN2#126, RAN2 agreed that;

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

This agreement allows UE to be configured with (1) both of inter-CU MCG LTM and intra-CU SCG LTM and (2) both of intra-CU MCG LTM and inter-CU SCG LTM simultaneously. For case (2), inter-CU SCG LTM should be SCG LTM with MN involved. Thus, there is no problem with mobility where the MN does not change. However, for case (1), there are potential issues since intra-CU SCG LTM is SCG LTM without MN involved. When UE executes inter-CU MCG LTM cell switch, the UE doesn’t have to keep the SCG LTM configuration. In Rel-18, MCG LTM with SCG release and MCG LTM without SCG change are supported. Therefore, RAN2 should discuss whether the SCG is released when inter-CU MCG LTM is execute if SCG is configured.

Observation 1. According to current agreements, inter-CU MCG LTM and intra-CU SCG LTM, and intra-CU MCG LTM and inter-CU SCG LTM can be configured for a UE simultaneously.

Observation 2. For SCG LTM, only SCG LTM without MN involved is supported in Rel-18.

Proposal 1. RAN2 discusses whether the SCG is released when inter-CU MCG LTM is execute if configured.

Stage-2 signalling flow

According to the WID, RAN2 should resolve inter-CU specific issues for LTM enhancement. So, RAN2 should start by identifying potential issues, and therefore, consider the overall procedure of inter-CU LTM. We think Rel-18 LTM procedure and Rel-18 SCPAC architecture can be a baseline, and we have considered procedures Figure 1 based on legacy procedures.

Figure 1 Signalling procedure for inter-CU LTM

Proposal 2. Firstly, RAN2 should clarify overall procedure and then identify the potential issues based on the overall procedure.

At LTM decision, source DU should send an LTM CSC MAC CE to a UE. And PDCCH for early synchronization needs to be triggered by source gNB. In addition, L1 measurement results are reported to source DU. Therefore, source DU should decide to trigger an inter-CU LTM cell switch.

Proposal 3. Source DU decides to trigger an inter-CU LTM.

For service continuity during inter-CU LTM cell switch procedure, SN transfer and path switch of user data flow are needed.

Proposal 4. RAN2 considers SN transfer and PATH SWITCH are needed for service continuity.

In legacy HO, source gNB sends HO command after sending HO request and receiving HO request ACK from target gNB. However, inter-gNB processing delay may be too large. Therefore, we think legacy CHO can be a baseline to reduce the latency between determination of LTM switch and LTM cell switch command. It means that source gNB sends HO(LTM) request on LTM preparation phase but not at LTM decision.

Proposal 5. Same as conditional reconfiguration, RAN2 considers that source gNB sends HO(LTM) request on LTM preparation phase but not at LTM decision.

As with Rel-18 LTM, the UE needs to know which beam to be used in the target cell. The beam to be used in the target cell can be determined by the source gNB based on the L1 measurement results and the prior configurations. Also, the source gNB can provide the determined TCI state to the target gNB and the UE. Therefore, as with legacy, TCI state ID is provided in LTM CSC MAC CE for inter-CU LTM.

Proposal 6. TCI state ID is provided in LTM CSC MAC CE for inter-CU LTM.

Preparation/configuration

It has been agreed in RAN2#126 that the source gNB-CU will start preparation and collect configuration information and CSI resources from each candidate in the preparation phase, as in SCPAC. It is reasonable to create a reference configuration as in Rel-18 LTM to reduce the size of the configuration information. In Rel-18 LTM, since it was intra-CU mobility, there is one reference configuration for one CU. Thus, are multiple reference configurations necessary in the case of inter-CU LTM? Reference configuration was also introduced in SCPAC, where there was one reference configuration for inter-CU mobility. And, RAN2 has also agreed that the number of entries in LTM is 8. Therefore, it is highly likely that creating multiple reference configurations for each CU will not be useful in reducing the information size. For the above reasons, one reference configuration is necessary for inter-CU LTM.

Proposal 7. Only one reference configuration is configured as with SCPAC.

Early UL/DL synchronization

RAN2 agreed to support early DL/UL synchronization for inter-CU LTM. In Rel-18, there are two options for early synchronization, i.e. PDCCH ordered early RACH and UE-based TA measurement. RAN2 already agreed former option. Following steps, as shown in Figure 2, can be considered as baseline of RACH-based early TA acquisition in case of inter CU LTM.

Figure 2 Steps for RACH-based early TA acquisition in case of inter CU LTM

In this procedure, a UE performs DL synchronization and RA to target gNB. Then, target DU can calculate the TA value for the UE. And then, TA value should be sent to the source gNB since TA value should be included in LTM CSC MAC CE. It is reasonable for the source NB to manage the TA since source DU can send a PDCCH for early RACH when the source DU predicts that the TA will no longer be valid.

Proposal 8. Target DU calculates TA value.

Proposal 9. Target DU send the TA value to source DU via CUs and source DU maintains the TA value.

Cell switch execution

In Rel-18 LTM, UE keeps the security key during the LTM cell switching. In RAN2#125bis, one company proposed that UE doesn’t need to update security key even if it’s an inter-CU LTM. In SDT WI, UE can send data to anchor node via non-anchor node. In such a case, UE may not need to update the security key. However, we have concerns on this option. Firstly, UE is in RRC_CONNECTED state while LTM is configured. Therefore, data forwarding from non-anchor node to anchor node is always needed (unlike SDT, many packets are forwarded). It increases backhaul data overhead and Anchor gNB cannot distribute the load. Secondly, when anchor relocation is needed, RRC reconfiguration with sync is needed for security key update. It is a waste of signalling. So, it is better to update the key upon LTM cell switch if security key update is needed.

Proposal 10. UE should update security key upon inter-CU LTM cell switching.

Upon LTM cell switch execution, UE needs to know whether it is intra-CU LTM or inter-CU LTM since RAN2 already agreed that PDCP re-establishment is needed in case of inter-CU LTM but PDCP re-establishment is not needed in case of intra-CU LTM. Thus, a mechanism is needed to determine whether the LTM cell switch is intra-CU or inter-CU. In Rel-18, UE uses “no reset IDs” to determine whether RLC re-establishment is needed for LTM cell switch and uses “security cell set IDs” to determine whether PDCP re-establishment is needed for SCPAC. Therefore, as with legacy, UE can determine whether the LTM cell switch is intra-CU or inter-CU based on CU set ID.

Proposal 11. UE determines whether the LTM cell switch is intra-CU or inter-CU based on CU set ID as with SCPAC.

Security aspects

For security aspects, many companies have made proposals for key update mechanisms and have sent LS to SA3 to ask about their policies. In SA3#116, SA3 approved a WID [3][4] about security aspects for LTM and will start discussion on it. And so far, there has been no progress. Therefore, we propose to postpone discussion on security update mechanism until the conclusion of SA3.

Observation 3. SA3 approved a WID about security aspects for LTM and will start discussion on it.

Proposal 12. Postpone discussion on security key change mechanism until the conclusion of SA3.

Subsequent LTM

The intention of Rel-18 subsequent LTM is to avoid RRC configuration so as to reduce the handover interruption. Thus, subsequent intra-CU was supported. In RAN2#125bis, it was agreed that Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. However, in case that an inter-CU LTM occurs, the gNB-CU is switched from the source gNB-CU to the target gNB-CU, where the candidate CU of the source gNB-CU may not be applicable as the candidate CU of the target gNB-CU. Since LTM preparation phase including RRC reconfiguration (LTM candidate configuration) is neglected in subsequent LTM based on the definition of subsequent LTM [5] and the limited number of LTM candidate configurations (i.e., at most 8 LTM candidate configurations), the RRC configuration transmitted by the source gNB-CU may not be applicable for the target gNB-CU after an inter-CU cell switch. As a result, to ensure the content accuracy of the RRC configuration applicable for the target gNB-CU and avoid RRC configuration during the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM, the validity of the subsequent LTM should be monitored. For example, the number of subsequent cell switches should be limited. The duration of subsequent cell switches to be allowed should be limited.

Proposal 13. The validity of the subsequent LTM should be monitored to ensure the content accuracy of the RRC configuration applicable for the target gNB-CU and avoid RRC configuration during the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

- Scenario

Observation 1. According to current agreements, inter-CU MCG LTM and intra-CU SCG LTM, and intra-CU MCG LTM and inter-CU SCG LTM can be configured for a UE simultaneously.

Observation 2. For SCG LTM, only SCG LTM without MN involved is supported in Rel-18.

Proposal 1. Intra-CU SCG LTM configuration is released when inter-CU MCG LTM is execute if configured.

- Stage-2 signalling flow

Proposal 2. Firstly, RAN2 should clarify overall procedure and then identify the potential issues based on the overall procedure.

Proposal 3. Source DU decides to trigger an inter-CU LTM.

Proposal 4. RAN2 Consider SN transfer and PATH SWITCH are needed.

Proposal 5. Same as conditional reconfiguration, RAN2 considers that source gNB sends HO(LTM) request on LTM preparation phase but not at LTM decision.

Proposal 6. TCI state ID is provided in LTM CSC MAC CE for inter-CU LTM.

- Preparation

Proposal 7. Only one reference configuration is configured as with SCPAC.

- Early synchronisation

Proposal 8. Target DU calculates TA value.

Proposal 9. Target DU sends the TA value to source DU and source DU maintains the TA value.

- Execution

Proposal 10. UE should update security key upon inter-CU LTM cell switching.

Proposal 11. UE determines whether the LTM cell switch is intra-CU or inter-CU based on CU set ID as with SCPAC.

- Security

Observation 3. SA3 approved a WID about security aspects for LTM and will start discussion on it.

Proposal 12. Postpone discussion on security key change mechanism until SA3 conclusion is made.

- Subsequent LTM

Proposal 13. The validity of the subsequent LTM should be monitored to ensure the content accuracy of the RRC configuration applicable for the target gNB-CU and avoid RRC configuration during the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-24xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- S3-242400, Reply LS to RAN2 on security handling for inter-CU LTM in non-DC cases

- S3-242401, New WID on security aspects of NR mobility enhancement

- TS 38.300 V18.1.0 (2024-05), 5G; NR; NR and NG-RAN Overall description; Stage-2