3GPP TSG-RAN WG2 Meeting #127　　　　　　　                                     R2-2407408

Maastricht, NL, 19th – 23rd Aug, 2024

Agenda item:	8.6.3

Source: 	Sharp

Title: 	Discussion on issues for supporting measurement event evaluation

Document for:	Discussion and Decision

Introduction

WID for enhancement mobility was approved and RAN2 started discussion on the WI. Also, RAN2 started discussion and reached some agreements follows in RAN2#126 [2].

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper, we discussed measurement event evaluation related potential issues.

Discussion

First, it is needed to discuss which sublayer evaluate measurement event to report measurement results to NW. We can use UCI and MAC CE to report measurement result. One advantage of using MAC CE is that HARQ can be used to confirm arrival. On the other hand, UCI is not protected by HARQ. Therefore, we proposed MAC CE can be used to report measurement result [2]. If RAN2 agree to use MAC CE to report measurement result to NW, it is reasonable that MAC sublayer evaluate measurement event to report measurement results to NW.

Proposal 1. MAC sublayer evaluates events to trigger the L1 measurement for LTM if MAC CE is used for event-triggered measurement reporting.

Next, it is needed to discuss whether cell level measurement result is used to evaluate measurement event or not. In legacy, UE performs beam level measurement at PHY layer and cell level measurement at RRC layer. If cell level evaluation is supported in MAC sublayer, UE needs to derive cell level quality at PHY or MAC sublayer. This can complicate the measurement system and should be avoided. Therefore, UE should evaluate events only at the beam level.

Proposal 2. MAC sublayer evaluates at the beam level, not at the cell level.

Next, we discuss whether Event LTM1 is supported or not. LTM is the like A1 event which is triggered when serving beam(s) become better than threshold configured by the NW. If measurements are reported by the LTM1, NW can know that serving beams is better and decide to release or modify the LTM configuration for the UE. However, it was agreed that event triggered L1 measurement should be designed for selecting the candidate beam/cell to trigger early synchronization and selecting the target beam/cell and trigger LTM cell switch. Therefore, the benefit of supporting LTM1 are not in line with these design concepts.

Observation 1: The benefit of supporting LTM1 are not in line with these design concepts.

Proposal 3. Event LTM1 is not supported.

Conclusion

In this contribution, we made the following observations and proposals:

Observation 1: The benefit of supporting LTM1 are not in line with these design concepts.

Proposal 1. MAC sublayer evaluates events to trigger the L1 measurement for LTM.

Proposal 2. MAC sublayer evaluates at the beam level, not at the cell level.

Proposal 3. Event LTM1 is not supported.

Reference

[1] R2_126_R19 MOB_Session Notes (Kyeongin Jeong)_EoM.docx

[2] R2-2407409 Discussion on issues for supporting measurement event evaluation, Sharp