3GPP TSG-RAN WG2 Meeting #127　　　　　　　                                     R2-2407409

Maastricht, NL, 19th – 23rd Aug, 2024

Agenda item:	8.6.4

Source: 	Sharp

Title: 	Discussion issues on related to measurement reporting

Document for:	Discussion and Decision

Introduction

RAN2 started discussion and reached some agreements follows in RAN2#126 [1].

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this paper, we discussed measurement reporting related potential issues.

Discussion

How to report measurement results?

It is needed to discuss how to report measurement results. UE can use UCI and MAC CE to report measurement result. There are some different points between UCI and MAC CE. One important advantage of using MAC CE is that HARQ can be used to guarantee that the measurement report has reached the NW.

Observation 1. If measurement reports are carried by a MAC CE, it is guaranteed that the measurement report has reached the NW.

Proposal 1. MAC CE can be used to report measurement result.

What contents are included in Measurement report?

It is needed to discuss what contents are included in measurement report. NW needs the following information to select the candidate beam/cell to trigger early synchronization and select the target beam/cell and trigger LTM cell switch procedure.

- candidate cell ID

- candidate beam ID of the candidate cell

- L1-RSRP of the candidate beam

- report config ID.

In addition, NW may use the following information to select the candidate beam/cell to trigger early synchronization and select the target beam/cell and trigger LTM cell switch procedure. The following information is not absolutely necessary, but may be helpful for proper LTM execution and early sync. Therefore, these may be considered as optional.

- RSRP per cell (if UE has derived the cell level RSRP)

- current beam RSRP

Proposal 2. MAC CE may include following contents:

- Mandatory: candidate cell ID, candidate beam ID, L1-RSRP, report config ID,

- Optional: RSRP per cell, current beam RSRP

It is needed to discuss max number of beams and candidate cells included in a MAC CE to report measurement results. In legacy LTM, max number of beams and candidate cells included in UCI to report measurement result are both 4. Therefore, we think consider that 4 is sufficient for both the maximum number of cells and the maximum number of beams.

Proposal 3. Maximum number of candidate cells and beams are both 4 in MAC CE to report measurement results.

In addition, we discuss how UE chooses candidate cells and beams to include in MAC CE. It is needed to discuss whether the MAC CE can include the measurement result of a candidate beam which did not trigger the event if MAC CE is used to carry measurement reports and MAC CE can include more than one measurement reports of each candidate beam. If measurement report can include the measurement result of the beam which did not trigger the event, it is needed to discuss which beams should be reported.

Proposal 4. RAN2 discuss whether measurement report can include the information of candidate beam did not trigger the event or not.

Conclusion

In this contribution, we made the following observations and proposals:

Observation 1. If measurement reports are carried by a MAC CE, it is guaranteed that the measurement report has reached the NW.

Proposal 1. MAC CE can be used to report measurement result.

Proposal 2. MAC CE may include following contents:

- Mandatory: candidate cell ID, candidate beam ID, L1-RSRP, report config ID,

- Optional: RSRP per cell, current beam RSRP

Proposal 3. Maximum number of candidate cells and beams are both 4 in MAC CE to report measurement results.

Proposal 4. RAN2 discuss whether measurement report can include the information of candidate beam did not trigger the event or not.

Reference

[1] R2_126_R19 MOB_Session Notes (Kyeongin Jeong)_EoM.docx