3GPP TSG-RAN-WG2 Meeting #127	R2-2407410

Maastricht, Netherlands, 19th– 23rd Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4950 | rev | 0 | Current version: | Ver.18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LTM related procedure for UE-based TA measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sharp |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is an issue that when the ltm-Candidate doesn’t include the ltm-UE-MeasuredTA-ID, the RRC layer inform lower layers nothing, the lower layers don’t understand whether the UE-based TA measurements is applied for the ltm-Candidate. This issue is resolved in subclause 5.3.5.18.6 (LTM cell switch execution), but not yet in subclause 5.3.5.18.3 (LTM candidate configuration addition/modification procedure). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The RRC informs lower layers that the UE is not configured with UE-based TA measurements for the ltm-Candidate if the ltm-Candidate doesn’t include ltm-UE-MeasuredTA-ID when LTM candidate configurations are added and/or modified. Impact analysis Impacted 5G architecture options:NR SA, MR-DC Impacted functionality: L1/L2 Triggered Mobility Inter-operability: If the network implements this CR but not the UE, the UE cannot perform UE-based TA measurements to the appropriate cells when LTM candidate configurations are added and/or modified. If the UE implements this CR but not the network, there is no inter-operabiilty issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The UE’s lower layers don’t understand whether the UE-based TA measurements is applied for the ltm-Candidate if the ltm-Candidate doesn’t include ltm-UE-MeasuredTA-ID when LTM candidate configurations are added and/or modified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

##### 5.3.5.18.3	LTM candidate configuration addition/modification

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToAddModList:

2>	if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value:

3>	reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate;

2>	else:

3>	add the received LTM-Candidate;

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate;

3>	else:

4>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

2>	else:

3>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

End of Change