3GPP TSG-RAN-WG2 Meeting #127	R2-2407411

Maastricht, Netherlands, 19th– 23rd Aug, 2024

Agenda item	:		7.9.3

Source	: 	Sharp

Title	: 	Discussion on U2U relay related issues

Document for	:	Discussion and Decision

Introduction

In 126 meeting, RAN2 had FFS points but there is no discussion for the following issues due to lack of time[1];

No discussion for the following issues due to lack of time: - (R2-2404252 OPPO) Proposal 1 In clause 5.8.9.1.2, remove the “according to association between User Info and L2 ID as specified in TS 23.304 [65],” for local ID pair setting procedure. Discussion: Apple: ok with the change, but want to clarify. P3: remove the “according to association between User Info and L2 ID as specified in TS 23.304 [65],” for local ID pair setting procedure On SLRB index in SUI for L2 U2U relay - (R2-2404663 Apple)Option 1: RAN2 concludes that L2 U2U Relay UE shall set the same value of SLRB index in SUI from what it received from remote UE. No more spec change is needed for this issue. - (R2-2405238 Huawei, R2-2405628 Sharp) Option 2: : U2U relay UE is allowed to set a different value of the SLRB index in SUI. (It can be left to implementation how to maintain the association between SLRB-PC5-ConfigIndex and SLRB-Uu-ConfigIndex.) (R2-2405351 ZTE) Relay UE traffic pattern reporting in UAI - Proposal 1: For periodic U2U relay services, U2U relay UE could report sidelink traffic pattern for periodic resource allocation for the second hop transmission. - Proposal 2: For U2U relay UE sidelink traffic pattern reporting, QoS flow ID is reused (with the modification of the field description) as an index in the scope of the relay UE to associate the traffic pattern with an SLRB-level QoS profile reported in SUI.

In this paper, we discuss;

- Whether to allow relay UE setting different value of SLRB index in SUI

Discussion

SLRB index

In RAN2#125bis, RAN2 made an agreement;

For an E2E SLRB, source remote UE configures the same value of SLRB index to Relay UE and target Remote UE. No change is made now in relation to allowing relay UE setting different value of SLRB index in SUI; whether to allow this, and if there would be any spec impact to do so, can be discussed next meeting based on contributions.

According to this FFS point, it means whether relay UE can use a value different from PC5-SLRB-ConfigIndex for Uu-SLRB-ConfigIndex. Firstly, we analysed the pros/cons for each option.

|  | Pros | Cons |
| --- | --- | --- |
| Use same value | No spec changes. If source remote UE uses same value for Uu index and PC5 index, gNB may determine U2U configuration based on two SUIs. | If same value is used for other sidelink communications, relay UE has to change the index. |
| Allow to use different value | Special handling is not needed even if same value is used for other sidelink communication. | Relay UE needs to maintain the indexes mapping. Relay UE needs to determine the mapping between PC5 index and RLC based on the mapping between a Uu index and an RLC entity sent by RRC signalling, referring to the mapping between the Uu index and the PC5 index. |

According to the above analysis we think it is slightly better to allow relay UE to use different value. However, current spec mandates U2U relay UE to use same value. Therefore, we need to study if there are any spec impacts. One concern is that Relay UE configures SRAP entity in accordance with SL-SRAP-ConfigU2U provided in dedicated configuration. SL-SRAP-ConfigU2U provides mapping between Uu-SLRB-ConfigIndex and RLC channel. At first glance, it seems necessary to configure SRAP according to the Uu index. However, legacy sidelink UE can associate Uu-SLRB-ConfigIndex with PC5-SLRB-ConfigIndex since sidelink UE doesn’t need to use same value for Uu index and PC5 index in Rel-16. So, we think it is easy to determine the mapping between PC5 index and RLC based on the mapping between Uu index and RLC sent by RRC signalling, referring to the mapping between Uu index and PC5 index. Therefore, RAN2 should allow relay UE to use different value in SUI.

Observation 1. If UE doesn’t use different value, special handling is needed in a case that same value is used for other sidelink communication.

Observation 2. it is easy to determine the mapping between PC5 index and RLC based on the mapping between Uu index and RLC sent by RRC signalling, referring to the mapping between Uu index and PC5 index since sidelink UE doesn’t need to use same value for Uu index and PC5 index in Rel-16.

Proposal 1. In SUI, Relay UE is allowed to use a Uu-SLRB-ConfigIndex value different from PC5-SLRB-ConfigIndex value provided by source remote UE.

- 2.3 others

For ASN.1 related clarification, sl-SplitPacketDelayBudget-r18 has no field description but other parameter related PDB has field description. Especially, the time unit of the PDB is important. If there is no definition of parameter, UE can select any time unit for the split PDB. Therefore, we suggest to add the field description as shown in TP#1.

Proposal 2. To clarify time unit of split PDB, RAN2 add the field description of sl-SplitPacketDelayBudget-r18 as shown in TP#1.

Conclusion

Observation 1. If UE doesn’t use different value, special handling is needed in a case that same value is used for other sidelink communication.

Observation 2. it is easy to determine the mapping between PC5 index and RLC based on the mapping between Uu index and RLC sent by RRC signalling, referring to the mapping between Uu index and PC5 index since sidelink UE doesn’t needed to use same value for Uu index and PC5 index in Rel-16.

Proposal 1. In SUI, Relay UE is allowed to use a Uu-SLRB-ConfigIndex value different from PC5-SLRB-ConfigIndex value provided by source remote UE.

Proposal 2. To clarify time unit of split PDB, RAN2 add the field description of sl-SplitPacketDelayBudget-r18 as shown in TP#1

Reference

- R2-2405876, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- R2-2403813, RRC corrections for Rel-18 SL relay enhancements

Annex

- 5.1 Text Proposal #1

#### –	UEInformationResponseSidelink

The UEInformationResponseSidelink message is used to deliver UE information in sidelink, e.g. the split QoS information for L2 U2U Relay operation.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: L2 U2U Relay UE to L2 U2U Remote UE

UEInformationResponseSidelink message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSESIDELINK-START

UEInformationResponseSidelink-r18 ::=       SEQUENCE {

rrc-TransactionIdentifier-r18               RRC-TransactionIdentifier,

criticalExtensions                          CHOICE {

ueInformationResponseSidelink-r18           UEInformationResponseSidelink-r18-IEs,

criticalExtensionsFuture                    SEQUENCE {}

}

}

UEInformationResponseSidelink-r18-IEs ::=  SEQUENCE {

sl-SplitQoS-InfoListPC5-r18                SEQUENCE (SIZE (1.. maxNrofSL-QFIs-r16)) OF SL-SplitQoS-InfoPC5-r18 OPTIONAL, -- Need N

lateNonCriticalExtension                   OCTET STRING                                                OPTIONAL,

nonCriticalExtension                       SEQUENCE {}                                                 OPTIONAL

}

SL-SplitQoS-InfoPC5-r18 ::=                SEQUENCE {

sl-QoS-FlowIdentity-r18                 SL-QoS-FlowIdentity-r16,

sl-SplitPacketDelayBudget-r18           INTEGER (0..1023)

}

-- TAG-UEINFORMATIONRESPONSESIDELINK-STOP

-- ASN1STOP

UEInformationResponseSidelink field descriptions

sl-SplitQoS-InfoListPC5 Indicates the split PDB on the first PC5 hop between L2 U2U Relay UE and the L2 U2U Remote UE for a list of QoS flow indicated by sl-QoS-FlowIdentity for one or more end-to-end PC5 connections. sl-SplitPacketDelayBudget indicates upper bound value for the delay that a packet may experience expressed in unit of 0.5ms.