3GPP TSG-RAN WG2 Meeting #127	R2-2407413

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.1.4

Source:	InterDigital Inc., Ericsson, Nokia, T-Mobile USA Inc., BT PLC., Deutsche Telekom, Verizon, NTT Docomo, Charter Communications Inc.

Title:	Data collection for the training of a UE-sided model

Document for:	Discussion

# 1. Introduction

During RAN2-125bis, the issue of data collection for the training of a UE-sided model was discussed but there was no consensus [1], as companies seem to have different interpretation of the different solutions identified during the Rel-18 study phase [2]. An email discussion was carried out after the meeting, with the aim of reaching a common understanding of the different solutions [3].

In parallel to the discussion of the clarification of the different solutions, RAN2 needs to also identify the main requirements for the training of a UE-sided model, which can then be used in the selection of a solution(s).

# 2. Discussion

For an efficient collection of training data for the UE-sided model training, it is important that the data being collected is relevant. For example, the UEs from which the data is being collected must have the measurement configuration by the network (i.e., RAN/CN). The network is responsible for configuring UEs to report CQI, best beam information, and location information, and thus it is in the unique position to determine feasible and useful data collection configurations. For example, the network might avoid configurations that are overly burdensome, e.g., significantly different RS configurations per UE for BM or overly dense RS configurations for CQI, BM, and positioning. The only way to avoid requiring RS configurations with high overhead to support AI/ML functionalities is for models to be trained with reasonable datasets which resemble reasonable RS transmission configurations from the network.

Collecting the data without network awareness/control could lead to an order of magnitude increase in the amount of data that needs to be collected to be able to gather enough relevant information (i.e., most of the data that is being gathered could end up being useless to the model training). Additionally, even if a UE has been provided with the required measurement configuration that is relevant for data collection for UE side model training, it does not necessarily mean that the UE will always perform the configured measurements. For example, to save UE battery, a UE is not required to perform neighbouring cell measurements if the serving cell’s signal level is above the s-measure (if configured), even if the UE has been provided with intra or inter-frequency measurement configurations.

Observation 1: For the training of a UE-sided model, collecting the data from UEs without network control/awareness is highly inefficient, as a UE may not have the relevant/required information (e.g., UE not configured with the proper measurement configuration).

The amount of data to be collected also needs to be considered, even if the data is being collected from UEs that have the relevant information. This will be an even bigger problem in the future where there will likely be more AIML use cases to be considered, more AIML capable UEs, and more models to be trained. If the data collection is done completely transparently from the network, it may cause network congestion and degrade the performance of other services of the UEs collecting the data and the performance of the UEs that are not involved in the data collection process (i.e., network will not be able to perform proper traffic management and/or admission control for the data being collected)

Observation 2: Collecting large amount of data for the training of a UE-sided model transparently from the RAN/CN may lead to traffic congestion and the operator may not be able to ensure that the required QoS is provided to the UEs in the network.

If the network is part of the pipeline for the collected data, then network control of the data collection process implies that the network is responsible for routing the collected data to a network-controlled entity or to a third-party entity via a network entity. In short, the network would be in control of the flow of collected data, which would be up to network implementation.

Considering the above observations, we propose that the network should have full control of the data collection process, which also includes the data transfer. Controllability of the data transfer is defined in the latest TP for the TR [4]:

Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer

Proposal 1: The network should have full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

# 4. Conclusion

In this contribution, the issues of data collection for UE-sided model training are discussed, and the following observations and proposals are made:

Observation 1: For the training of a UE-sided model, collecting the data from UEs without network control/awareness is highly inefficient, as a UE may not have the relevant/required information (e.g., UE not configured with the proper measurement configuration).

Observation 2: Collecting large amount of data for the training of a UE-sided model transparently from the RAN/CN may lead to traffic congestion and the operator may not be able to ensure that the required QoS is provided to the UEs in the network.

Proposal 1: The network should have full control of the data collection process for the training of a UE-sided model, including the initiation, termination and management of the data collection and data transfer.

# 5. References

[1]	RAN2-116bis Chairman Notes, Changsha, China, April 2024

[2]	TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Release 18, 2023/12

[3]	R2-2404476, Report of [POST125bis][020][AI/ML PHY] UE side data collection, MediaTek Inc., RAN2-126, Fukuoka, Japan, May 2024

[4]	R2-2407209, RAN2 inputs to TR 38.843, Ericsson, RAN2-127, Maastricht, Netherlands, August 2024