3GPP TSG-RAN WG2 Meeting #127	R2-2407414

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.5.2

Source:	Sharp

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

# Introduction

In RP#102 meeting, the following objective was approved as part of the WID [1].

- Specify procedures and signalling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Moreover, the following agreements were made by RAN1’s meetings [2-4].

Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported. - Frequency of the on-demand SSB - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst - Periodicity of the on-demand SSB - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE - FFS: Whether the RRC is newly introduced or existing RRC is reused Agreement For a cell supporting on-demand SSB SCell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement

In this contribution, we are going to discuss on-demand SSB based L3 measurement.

# Discussions

According to the discussion in RAN1, on-demand SSB based measurement can serve different functionalities for identified scenarios. When SCell is configured but deactivated (i.e., Scenario #2), the network can determine whether to activate the configured SCell (that is not activated) based on the L3 measurement reports. Moreover, by sending a valid measurement report (as specified in TS 38.133), the configured SCell is identified to be known, which can accelerate SCell activation procedure. During the SCell activation procedure (i.e., Scenario #3A), the SSB within SMTC measurement window is used for AGC gain setting. After SCell is activated, (i.e., Scenario #3B), L3 measurement results can be reported so that the network can identify the SCell channel condition and further determine whether to deactivate or release the activated SCell. Moreover, RAN1 has agreed that L3 measurement based on on-demand SSB may be supported, but L3 measurement is RAN2's and/or RAN4's expertise. Therefore, we suggest RAN2 WG discuss specification impacts on L3 measurement due to on-demand SSB operation in RAN2 point of view.

Proposal 1: RAN2 WG to discuss specification impacts on L3 measurement due to on-demand SSB operation.

Based on the current specifications, whenever the UE is configured with the measurement configuration (i.e., measConfig), for a serving cell associated with a measurement object which is associated with reporting configuration(s), if the measurement object is configured with ssb-ConfigMobility and at least one associated reporting configuration includes the rsType set to SSB, the UE shall perform SSB based measurement (e.g., RSRP and RSRQ measurement) as well as derive the corresponding measurement results. Moreover, if the measurement object is configured with ssb-ConfigMobility and at least one associated reporting configuration contains SINR as the trigger quantity and/or the report quantity, the UE shall perform SSB based measurement and derive the corresponding cell SINR.

Assuming on-demand SSB based L3 measurement is supported for a cell supporting on-demand SSB operation, it is noted on-demand SSB may be activated/deactivated dynamically on the cell. However, for Case #1 (no always-on SSB on the cell), if the UE follows legacy measurement behaviour as mentioned above, it may introduce unnecessary measurement when there is no SSB transmission on the cell, which further leads to unnecessary UE power consumption. To tackle this issue, L3 measurement on the cell may be adjusted based on whether on-demand SSB is transmitted. Specifically, the UE does not expect to measure SSB when on-demand SSB is deactivated. In other words, the UE expects to measure SSB when on-demand SSB is activated. We then have the following proposal.

Proposal 2: For Case #1, the UE does not expect to measure SSB when on-demand SSB is deactivated. In other words, the UE expects to measure SSB when on-demand SSB is activated.

It is also noted that L3 measurement report can be configured as event-triggered or periodic. When the reporting configuration associated with a measurement identity is configured as event triggered, to evaluate whether the triggering (e.g., entering or leaving) condition of the reporting configuration is fulfilled, the UE may firstly determine which (serving and/or neighboring) cell is applicable for evaluation. Afterwards, if the triggering condition is fulfilled for the applicable cell(s) for all measurement results (after L3 filtering) during the period defined by timeToTrigger configured in the reporting configuration, the UE may initiate a reporting procedure for this measurement identity in order to report measurement results to the network.

However, assuming on-demand SSB based L3 measurement is supported for a cell supporting on-demand SSB operation, when on-demand SSB is deactivated for the cell, the event-triggered measurement report may be falsely initiated. More specifically, as shown in Figure 1, at Time X0, on-demand SSB is activated for serving cell Y. At Time X1, the UE may derive a serving cell Y measurement result (i.e., M1) that fulfils the triggering condition. Afterwards, on-demand SSB is deactivated for serving cell Y at Time X2. At Time X3, UE may determine all derived measurement results (including the serving cell Y measurements M1, M2) during the timeToTrigger period fulfil the triggering condition, and thus initiate a corresponding reporting procedure. However, since there is no measurement result derived for serving cell Y after on-demand SSB is deactivated (i.e., the yellow area), the triggered measurement report may not properly indicate the real channel condition for serving cell Y, which may cause the network to make wrong decisions. Therefore, we suggest RAN2 discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

Figure 1. Event triggered measurement reporting after on-demand SSB is deactivated.

Observation 1: When a (serving) cell supporting on-demand SSB is configured, the UE may initiate a measurement reporting after on-demand SSB is deactivated for the cell, where the report may not properly indicate the real channel condition for this cell.

Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

In the current specifications, the measurement configuration (i.e., measConfig) may include measurement objects, reporting configurations, and measurement identities, where each measurement identity associates one measurement object with one reporting configuration. For serving cell measurement purpose, when a cell is configured, the UE may be further configured with a measurement object indication (i.e., servingCellMO) indicating the cell is associated with the measurement object indicated by servingCellMO. Then the UE may perform serving cell measurement based on the parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object, derive measurement results based on the L3 filtering parameters provided in the associated quantity configuration as well as perform measurement reporting based on the parameters (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s).

For Case #2, always-on SSB is SSB supported in Rel-18 specifications, and the UE may perform always-on SSB based measurement as mentioned above. However, after on-demand SSB is activated, the UE may incorrectly perform on-demand SSB based measurement since the legacy parameters (e.g., smtc1, ssb-ToMeasure) provided in the associated measurement object are configured for always-on SSB instead of on-demand SSB. To resolve this, one option is to configure additional corresponding parameters (e.g., smtc1, ssb-ToMeasure) which can be enabled when on-demand SSB is activated. Thus, we suggest RAN2 discuss what additional parameter(s) needs to be provided in the measurement object for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated. Similarly, whether the additional parameter(s) (e.g., reportInterval, reportAmount, timeToTrigger) provided in the associated reporting configuration(s) and additional L3 filtering parameters provided in the associated quantity configuration are needed for on-demand SSB can be further studied.

Proposal 4: For Case #2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Proposal 1: RAN2 WG to discuss specification impacts on L3 measurement due to on-demand SSB operation.

Proposal 2: For Case #1, the UE does not expect to measure SSB when on-demand SSB is deactivated. In other words, the UE expects to measure SSB when on-demand SSB is activated.

Observation 1: When a (serving) cell supporting on-demand SSB is configured, the UE may initiate a measurement reporting after on-demand SSB is deactivated for the cell, where the report may not properly indicate the real channel condition for this cell.

Proposal 3: RAN2 WG to discuss the issue of false measurement report triggering due to no SSB transmission when on-demand SSB is deactivated.

Proposal 4: For Case #2, RAN2 WG to discuss what additional parameter(s) needs to be provided in the measurement object/report configuration/quantity configuration for on-demand SSB and how UE adapts the additional parameter(s) when on-demand SSB is activated/deactivated.

# References

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson (Moderator).

- R1-2401937, “Final Report of 3GPP TSG RAN WG1 #116 v1.0.0 ​(Athens, Greece, February 26th - March 1st, 2024)”, Chairman (Samsung).

- R1-2403821, “Final Report of 3GPP TSG RAN WG1 #116b v1.0.0 ​(Changsha, China, April 15th – 19th, 2024)”, Chairman (Samsung).

- “Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0 ​(Fukuoka, Japan, May 20th – 24th, 2024)”, Chairman (Samsung).