3GPP TSG-RAN WG2 Meeting #127	R2-2407415

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	UE behaviour for MBS related procedures

Document for	:	Discussion and Decision

Introduction

In the last RAN2 #126 meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - For MBS broadcast service, both EFC and EMC are supported. - RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. - MBS broadcast intended service area is provided via system information - For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS - When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area) Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

In this paper, we’d like to discuss UE behaviour for MBS related procedures.

Discussion

In RAN2#126, RAN2 agreed that;

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

This agreement enables UE to physically receive MBS related information and data, if serving cell broadcasts them, even if UE is not located within the intended service area.

Observation 1. If serving cell broadcasts MBS related information and data, UE can physically receive them even if UE is not located within the intended service area.

In this case, based on the current specification, UE can perform MBS related procedures (e.g. MRB establishment, MRB release, MII (MBS interest indication), cell reselection priorities handling). If these procedures are performed by UEs which are not intended to receive the service, there are negative impacts for NW operation, especially if UL resources are used by the procedure. For example, if UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should limit UE behaviours for MBS related procedures.

Observation 2. If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service.

Proposal 1. UE behaviour considering intended service area should be specified for MBS related procedures.

Therefore, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area. RAN2 already suggested that it is up to UE implementation how the UE detects it’s not in the intended service area, and this intension can be applied to this MII procedure.

Proposal 2. MBSInterestIndication message should not be sent by UE located outside of the intended service area. (up to UE implementation how the UE detects it’s not in the intended service area)

As a similar case, in IDLE mode, cell reselection priorities handling is performed considering the interest of MBS service. NW configures cell reselection priorities for each carrier frequencies and/or cells based on the cell layout, load balancing, and so on. If UEs located outside of the intended service area are allowed to change the cell reselection priorities for receiving MBS service, the intended cell reselection priorities by NW are ignored by them.

Observation 3. If UEs located outside of the intended service area are allowed to change the cell reselection priorities for receiving MBS service, the intended cell reselection priorities by NW are ignored by them.

Proposal 3. Cell reselection priorities handling for receiving MBS service should be performed by the UE located inside of the intended service area. (up to UE implementation how the UE detects it’s in the intended service area)

On the other hand, MRB establishment/release procedures have less impacts for NW operation than above procedures. However, to align with the above UE behaviour, we think it is preferable UE behaviour considering intended service area for MRB establishment/release procedures are mandated.

Proposal 4. UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Proposal 5. UE should release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. If serving cell broadcasts MBS related information and data, UE can physically receive them even if UE is not located within the intended service area.

Observation 2. If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service.

Proposal 1. UE behaviour considering intended service area should be specified for MBS related procedures.

Proposal 2. MBSInterestIndication message should not be sent by UE located outside of the intended service area. (up to UE implementation how the UE detects it’s not in the intended service area)

Observation 3. If UEs located outside of the intended service area are allowed to change the cell reselection priorities for receiving MBS service, the intended cell reselection priorities by NW are ignored by them.

Proposal 3. Cell reselection priorities handling for receiving MBS service should be performed by the UE located inside of the intended service area. (up to UE implementation how the UE detects it’s in the intended service area)

Proposal 4. UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Proposal 5. UE should release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

Reference

- R2-24xxxx, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan