3GPP TSG-RAN-WG2 Meeting #127	R2-2407416

Maastricht, Netherland, 19th– 23rd Aug, 2024

Agenda item	: 	8.8.4

Source	: 	Sharp

Title	: 	Discussion on MBS service support for NR NTN

Document for	:	Discussion and Decision

Introduction

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

According to the R19 NR NTN WID[1] scope, NTN MBS broadcast service was proposed by RAN2.

And in RAN2#126 meeting, RAN2 made following agreements about Intended Service Area.

Agreements (excerpted ):

- MBS broadcast intended service area is provided via system information.

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this paper, we discuss where/how to contain information about the intended service area.

Discussion

- 2.1 Which message includes the Intended Service Area Information

SIB20 includes scheduling information for MCCH-message. And each of the session information in the MCCH-message includes information on one or more sessions in a list, frequency information used for the session, and TMGI. In the previous meeting, it is proposed that the intended service area information to be included in SIB20 and/or MBSBroadcastConfiguration as one of alternatives. In this case, it is necessary to choose whether to include it in SIB20 or MBSBroadcastConfiguration, by considering these data structures as below [2].

SIB20-r17 ::=	SEQUENCE {

mcch-Config-r17                MCCH-Config-r17,

cfr-ConfigMCCH-MTCH-r17        CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

lateNonCriticalExtension       OCTET STRING            OPTIONAL,

...,

[[

cfr-ConfigMCCH-MTCH-RedCap-r18 CFR-ConfigMCCH-MTCH-r17 OPTIONAL,  -- Need S

mcch-ConfigRedCap-r18          MCCH-Config-r17         OPTIONAL   -- Need S

]]

}

MBSBroadcastConfiguration-r17 ::= SEQUENCE {

criticalExtensions                CHOICE {

mbsBroadcastConfiguration-r17     MBSBroadcastConfiguration-r17-IEs,

criticalExtensionsFuture          SEQUENCE {}

}

}

MBSBroadcastConfiguration-r17-IEs ::= SEQUENCE {

mbs-SessionInfoList-r17               MBS-SessionInfoList-r17                                              OPTIONAL,   -- Need R

mbs-NeighbourCellList-r17             MBS-NeighbourCellList-r17                                            OPTIONAL,   -- Need S

drx-ConfigPTM-List-r17                SEQUENCE (SIZE (1..maxNrofDRX-ConfigPTM-r17)) OF DRX-ConfigPTM-r17   OPTIONAL,   -- Need R

pdsch-ConfigMTCH-r17                  PDSCH-ConfigBroadcast-r17                                            OPTIONAL,   -- Need S

mtch-SSB-MappingWindowList-r17        MTCH-SSB-MappingWindowList-r17                                       OPTIONAL,   -- Need R

lateNonCriticalExtension              OCTET STRING                                                         OPTIONAL,

nonCriticalExtension                  SEQUENCE {}                                                          OPTIONAL

}

Observation 1: As one of alternatives, SIB20 or MBSBroadcastConfiguration can include the Information linking intended service area to each of session information if intended service area is associated with session information.

If information related to intended service area is included in SIB20 or MBSBroadcastConfiguration, it means UE needs to acquire them before considering whether UE is located inside of intended service area or not. In addition, since SIB20 is provided by each cell independently to schedules MCCH, UE may need to receive each cell's SIB20 to check the information for intended service area. This causes the increasing of UE power consumption to determine whether the UE is located inside of intended service area.

Observation 2: If UE needs to acquire SIB20/MBSBroadcastConfiguration to get information related to intended service area, it means UE needs to acquire them that the UE can receive before considering whether UE is located inside of intended service area or not.

1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

2) Either:

- One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.346 [20], or

- SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

- SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

SIB21 is also system information related to MBS, which includes the MBS FSAI. In TS 38.331 [4] states that one or more MBS FSAIs of the frequency on which SIB20 is transmitted is indicated in the SIB21 of the serving cell. And the TS26.346 states that that same FSAI is included in the MBS User Service Description.

5.9.4.3	MBS frequencies of interest determination

The UE shall:

1>	consider a frequency to be part of the MBS frequencies of interest if the following conditions are met:

<omit>

2> for at least one of these MBS sessions, SIB21 acquired from the PCell or a non-serving cell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and

In TS38.331, in the procedure for determining the MBS frequency of interest, the following is stated.

From above, FSAI is associated with MBS session(s), and SIB21 indicates the association between the frequency which the services are provided and FSAI (i.e. the service frequency and (a group of) MBS session(s)). So, we propose that RAN2 will also consider the possibility of linking intended service area information to the FSAI.

Observation 3: FSAI is associated with MBS session(s), and SIB21 indicates the association between the frequency which the services are provided and FSAI (i.e. the service frequency and (an group of) MBS session(s)).

Observation 4: Since one group of MBS sessions which is associated with one FSAI is associated with common frequency/location, intended service area also can be associated with FSAI to indicate the common service area for the group of MBS sessions.

Proposal 1：RAN2 will also consider the possibility of linking intended service area information to the FSAI.

In the current specification, the mapping information between the frequency and FSAIs are included in SIB21.

Observation 5: the mapping information between the frequency and FSAIs are included in SIB21.

Proposal 2: if Proposal 1 is agreed, since FSAI related information is already included in SIB21, intended service area(s) should be included in SIB21. For the same reason, explicit (or implicit) association information should also be included in SIB21.

Conclusion

Observation 1: As one of alternatives, SIB20 or MBSBroadcastConfiguration can include the Information linking intended service area to each of session information if intended service area is associated with session information.

Observation 2: If UE needs to acquire SIB20/MBSBroadcastConfiguration to get information related to intended service area, it means UE needs to acquire them that the UE can receive before considering whether UE is located inside of intended service area or not.

Observation 3: FSAI is associated with MBS session(s), and SIB21 indicates the association between the frequency which the services are provided and FSAI (i.e. the service frequency and (an group of) MBS session(s)).

Observation 4: Since one group of MBS sessions which is associated with one FSAI is associated with common frequency/location, intended service area also can be associated with FSAI to indicate the common service area for the group of MBS sessions.

Proposal 1：RAN2 will also consider the possibility of linking intended service area information to the FSAI.

Observation 5: the mapping information between the frequency and FSAIs are included in SIB21.

Proposal 2: if Proposal 1 is agreed, since FSAI related information is already included in SIB21, intended service area(s) should be included in SIB21. For the same reason, explicit (or implicit) association information should also be included in SIB21.

Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3

- R2-2405876, Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

- TS38.331 v18.0.0

- TS38.304 v18.2.0