3GPP TSG-RAN WG2 Meeting #127	                                    R2-2407417

Maastricht, Netherlands, Aug. 19th – Aug. 23rd, 2024

Agenda item:		8.1.3

Source:			Sharp

Title:		Discussion on NW-side Data Collection for Positioning

Document for:	 	Discussion

- Introduction

During Rel-18, data collection for the training of NW-side models was discussed with the following agreements in TR 38.843 [1]. The following principles apply to all the currently identified used cases (i.e., the beam management, the positioning, and the CSI compression and prediction use cases).

- A set of general data collection principles is expected to be considered for network-side model training. These include: - UE to support data logging - UE to report the collected data periodically, event-based, and on-demand - The UE memory, processing power, energy consumption, signalling overhead should be considered

In RAN2#125bis, agreements for general principles for data collection for NW-side model have been made as follows [2]. The other agreements for specific use cases (i.e., the beam management use case) are omitted.

We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection

In RAN2#126, no further agreements were made for the positioning use case, nor for general principles. However, from the principles in the TR 38.843, it is necessary to have some basic enhancements for the LPP protocol. To avoid frequent measurement reporting, it is essential for the data collection mechanism for NW-side model to support the logging of measurement results. However, the current LPP only supports a one-shot report. For the positioning use case, it is not defined how the UE logs the measurement result and what exactly the UE includes in the measurement report. As a result, in this contribution, we would like to share our view on the enhancement of LPP messages to accommodate the logging feature for NW-side data collection.

- Discussion

UE Capability for Data Collection

First, since the measurement is performed by the UE, the data collection for NW-side model needs the consent and the capability of the UE. That is, whether the UE measures the PRS and reports the result for the purpose of data collection for NW-side data collection is determined and indicated by the UE. The NW can obtain the measurement results for NW-side model only when the UE supports the function of data collection and is able to perform the function (e.g., the power / the memory can afford the data collection for NW-side model).

Proposal 1: The UE determines and indicates whether the UE measures the PRS and reports the result for the purpose of data collection for NW-side data model.

In our view, the existing LPP messages ProvideCapabilities and RequestCapabilities can achieve the coordination. The NW can first request whether the UE supports the AI/ML-related functions (e.g., whether the UE can perform inference using the UE-side model or whether the UE can collect the measurement result related to the model input for model training/inference/monitoring) via the RequestCapabilities message. In addition, the request for AI/ML-related functions can be a new indication which is separate from the request for legacy positioning methods. For example, an AIML-RequestCapabilities message can be used for the UE to identify that the LMF is requesting the capability of AI/ML-related functions.

The UE can then respond its capability of AI/ML-related functions via the ProvideCapabilities message. Similarly, the ProvideCapabilities message can be a new message such as AIML-ProvideCapabilities and can include different indications to indicate the respective support of different AI/ML-related functions. In addition, if the UE supports data collection for NW-side model, the UE can indicate a maximum number of logged measurement results based on its memory status. The maximum number can be used as a reference by the LMF to determine the configuration related to logging. Moreover, if the ground truth label is available at the UE side (i.e., the UE can obtain its location by other non-AI/ML based positioning methods), the UE can indicate the information in the AIML-ProvideCapabilities.

Proposal 2: The LMF can request the UE’s capabilities related to AI/ML positioning via RequestCapabilities with an indication of AIML-RequestCapabilities.

Proposal 3: The UE can provide its capabilities related to AI/ML positioning via ProvideCapabilities with an indication of AIML-ProvideCapabilities. The UE can further include (a) a maximum number of logged measurement results as a reference for the LMF and (b) an indication whether the ground truth is available at the UE side.

Measurement and Report Configuration for AI/ML Positioning

In our view, the LMF is capable of flexibly switching between different NW-side models corresponding to different positioning methods (e.g., TDOA, AOD, Multi-RTT). As a result, LMF needs to collect data with different quantities in different additional conditions. To allow the LMF to dynamically configure the quantity to be measured, we propose to use a new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) to configure the UE what to measure. Similar to the legacy ProvideAssistanceData message, the AIML-ProvideAssistanceData message contains PRS resource configurations; additionally, it also configures whether one or more of the measurement quantities (i.e., RSTD, RSRP, RTT) need to be measured by the UE.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different quantities in different additional conditions.

Proposal 4: A new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure one or multiple measurement quantities based on the demand of data types for training different NW-side models.

As for the report configuration, we think the legacy RequestLocationInformation message can be reused with some extension for 1) configuring the logging-related configuration and 2) configuring the measurement report contents; as a result, we propose a new type of RequestLocationInformation message for the AI/ML positioning.

Observation 2: The legacy LPP message “RequestLocationInformation” can be enhanced to configure the measurement report contents and the logging-related configurations.

Proposal 5: A new type of RequestLocationInformation message (e.g., AIML-RequestLocationInformation) is used for the LMF to configure the logging and measurement reporting configurations regarding the data collection for NW-side model.

The logging configuration can indicate how and when the UE logs the measurement results. For example, a basic periodical logging configuration can include a logging interval and a threshold value for the number of logged measurement results. In addition, to precisely collect the data with certain additional conditions, and to reduce the measurement overhead at the UE side, we propose to log the measurement results in an event-triggered manner. For example, the LMF can configure an association ID (in case it represents a NW-side or UE-side additional condition) in the RequestLocationInformation message. The UE only measures and logs the measurement result at the certain NW-side or UE-side additional condition.

Observation 3: Logging the measurement results in an event-triggered manner is beneficial for precise collection of the data with certain additional conditions and reduction of the measurement overhead at the UE side.

Proposal 6: The LMF can provide the condition (e.g., a certain additional condition) in the new type of RequestLocationInformation message to enable the UE to perform event-triggered logging.

The measurement report configuration can also be enhanced to allow the UE to perform event-triggered measurement report and on-demand reporting to avoid frequent measurement report that congests the wireless channel. For event-triggered measurement report, the LMF can configure an indication that the UE triggers the measurement report when 1) the logging counter reaches a threshold, or 2) the leaving condition for event-triggered logging is met. As for on-demand measurement report, an indication from LMF is needed to inform that a measurement report is requested. The indication could be an existing LPP message such as RequestLocationInformation with an indication IE for the UE to identify the message is for configuration or for on-demand measurement report request; alternatively, a new type of LPP message is needed to be defined to trigger the UE to initiate a measurement report.

Observation 4: To avoid frequent measurement report, the measurement report configuration can be enhanced to accommodate the event-triggered and on-demand measurement report.

Proposal 7: For event-triggered measurement report, the LMF can configure the UE to report when 1) the logging counter reaches a threshold or 2) the leaving condition for event-triggered logging is met.

Proposal 8: For on-demand measurement report, an indication from LMF is needed to inform the UE of the request of measurement result. The indication can be 1) a legacy LPP message with an indication IE for the UE to identify the on-demand request, or 2) a new LPP message.

As for the content of the measurement report for model training, what the UE should include in the report can be configured also via the new type RequestLocationInformation message. The LMF can request the UE for providing the ground truth label and the associated additional condition for better data classification, which would be beneficial for training specific models. In our view, if the LMF requires a ground truth label, it can provide a validity period to the UE. The UE then associates the measurement results with the ground truth label whose timestamp difference is less than the validity period. By doing so, the timestamps of the measurement result and the ground truth label are not needed to be reported.

Observation 5: The legacy RequestLocationInformation message can be extended to configure the report content.

Proposal 9: Use the new type RequestLocationInformation message to indicate whether the UE needs to report the ground truth label and the associated additional conditions.

Proposal 10: Use a validity period for the UE to associate the measurement result and the ground truth label by itself.

In case on-demand measurement report is supported, a UE may face the problem that the logged measurement results increase upon every logging instance while the LMF does not request the measurement results. To prevent the UE from the overwhelming size of logged measurement results, a dropping rule is needed. In our view, although this can be up to UE’s implementation, an upper bound of logging time (like 48 hours in logged MDT) can be discussed.

Observation 6: A dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 11: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: LMF should be capable of flexibly switching between different NW-side models leveraging different positioning methods, so it may need to collect data with different quantities in different additional conditions.

Observation 2: The legacy LPP message “RequestLocationInformation” can be enhanced to configure the report contents and the logging-related configurations.

Observation 3: Logging the measurement results in an event-triggered manner is beneficial for the precise collection of the data with certain additional conditions and the reduction of the measurement overhead at the UE side.

Observation 4: To avoid frequent reporting, the reporting configuration can be enhanced to accommodate the event-triggered and on-demand reporting.

Observation 5: The legacy RequestLocationInformation message can be extended to configure the report content.

Observation 6 A dropping method is needed to prevent the UE from the overwhelming size of logged measurement results.

Proposal 1: The UE determines and indicates whether the UE measures the PRS and reports the result for the purpose of data collection for NW-side data model.

Proposal 2: The LMF can request the UE’s capabilities related to AI/ML positioning via RequestCapabilities with an indication of AIML-RequestCapabilities.

Proposal 3: The UE can provide its capabilities related to AI/ML positioning via ProvideCapabilities with an indication of AIML-ProvideCapabilities. The UE can further include (a) a maximum number of logged measurement results as a reference for the LMF and (b) an indication whether the ground truth is available at the UE side.

Proposal 4: A new type of ProvideAssistanceData message (e.g., AIML-ProvideAssistanceData) is used for the LMF to configure one or multiple measurement quantities based on the demand of data types for training different NW-side models.

Proposal 5 A new type of RequestLocationInformation message (e.g., AIML-RequestLocationInformation) is used for the LMF to configure the logging and reporting configurations regarding the data collection for NW-side model.

Proposal 6: The LMF can provide the condition (e.g., a certain additional condition) in the new type of RequestLocationInformation message to enable the UE to perform event-triggered logging.

Proposal 7: For event-triggered reporting, the LMF can configure the UE to report when 1) the logging counter reaches a threshold or 2) the leaving condition for event-triggered logging is met.

Proposal 8: For on-demand reporting, an indication from LMF is needed to inform the UE of the request of measurement result. The indication can be 1) a legacy LPP message with an indication IE for the UE to identify the on-demand request, or 2) a new LPP message.

Proposal 9: Use the new type RequestLocationInformation message to indicate whether the UE needs to report the ground truth label and the associated additional conditions.

Proposal 10: Use a validity period for the UE to associate the measurement result and the ground truth label by itself.

Proposal 11: An upper bound of logging time (e.g., 48 hours in logged MDT case) is specified for the use case of data collection for NW-side model.

Reference

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, December 2023

- R2-2404102 Report of 3GPP TSG RAN WG2 meeting#125bis, April 15-19, 2024