3GPP TSG-RAN WG2 Meeting #127	R2-2407421

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.6.2

Source:	Samsung

Title:	Further Considerations to Support Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, RAN2 reached the following agreements for inter-CU LTM procedure and some details:

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this contribution, we will further address inter-CU LTM in details.

Discussion

Issue 1: CSI measurement resource/report configurations for inter-CU

Rel-18 intra-CU LTM introduced the CSI measurement configurations with different level between LTM CSI resource configuration and LTM CSI report configuration. The LTM CSI resource configuration (LTM-CSI-ResourceConfig) is provided from the source PCell under the LTM-Config, it means this LTM CSI resource configuration includes all CSI resources for the LTM candidate cells as a common configuration. Meanwhile, the LTM CSI report configuration (LTM-CSI-ReportConfig) is configures in CSI-MeasConfig which is configured under the serving cell configuration (i.e. ServingCellConfig) similar as other L1 report configuration. So, each LTM candidate cell configuration provides own LTM CSI report configuration in the pre-configured RRCReconfiguration to support the subsequent LTM.

Since the Rel-18 LTM only operated in inter-CU, the procedure to request/collect the common CSI resource configurations from DUs are handled by the source CU. Then, how to support the CSI resource configurations for all candidate cells which can be located in the different CUs should be determined. Following two options could be considered:

  - Option 1: Common configuration across all CUs (i.e. source CU manage the CSI resource configuration)

  - Option 2: Per CU handling (i.e. Each CU manage LTM-CSI-ResourceConfig and share it to source CU)

From our understanding, Option 1 has more benefit to apply the LTM CSI resource configurations for inter-CU, UE point of view because UE just measure the commonly configured L1 measurement resources which could be L1 resources from all LTM candidate cells, i.e. no need to differentiate the CU which the RS is provided). Option 2 reduces the CSI resource request/response steps in LTM preparation phase because the current LTM preparation steps (between CU and DUs) for CSI resource configuration could be reused, and each CU provide the results to the source CU. However, Option 1 requires more step to collect and provide the final CSI resource configuration by the source CU. In addition, RAN2 already agreed to use the unique LTM candidate ID, so there will be no CU ID.

For CSI report configuration, we think Rel-19 LTM for inter-CU also support the subsequent LTM between CUs so the CSI report configuration should be provided per the candidate cell after inter CU LTM cell switch is performed.

Proposal 1-1: Source CU manage the CSI resource configuration across all CUs.

Proposal 1-2: Each candidate cell provides the CSI report configuration for inter-CU LTM.

Issue 2: single LTM reference configuration vs. multiple LTM reference configuration

In the legacy cell switch case, the RRC reconfiguration in handover command is configured as the delta configuration manner on top of the source cell configuration if the full configuration is not used. However, LTM can be operated with the multiple candidate cells the separate reference configuration is more fit with the signalling reduction. So, we think the multiple LTM reference configurations are more aligned with the purpose of introduction of the reference configuration in Rel-18 LTM. The LTM reference configuration target to reduce the RRC signalling overhead applying the candidate cell configuration (i.e. target cell configuration) by using the delta configuration on top of the reference configuration.

The comparison between two methods are given as below:

- Single reference configuration: The UE is provided one reference configuration, and the configuration of each candidate cell is generated based on this reference configuration. For this case, the delta configuration for each candidate cell may contain more information. We assume the reference configuration has no much common configuration because the RRC configuration from the different CUs might be different. So, the signalling reduction by introducing the single reference configuration seems not good enough. Meanwhile, it may be simple to implementation point of view by reducing the LTM preparation phase (i.e. NW may skip the preparation phase and use the source cell configuration as the reference configuration)

- Multiple reference configuration: The UE is configured with multiple reference configurations, and each candidate cell should be associated with one reference configuration for the full configuration generation. We assume these multiple reference configurations are configured per CU. For this case, the delta configuration for each candidate cell may contain less information than that via single reference configuration. However, the consequence is that the UE needs be configured multiple reference configurations and apply the each reference configuration to the candidate configuration(s) within the indicated (same) CU. Rel-18 LTM preparation procedure could be reused i.e. each CU just share the final reference configuration to the source CU, then source CU provides the received multiple reference configurations to the UE.

From the above comparison, it is difficult to justify which method results in less signalling overhead because there are different signalling reduction points between the single reference configuration and multiple reference configuration.

We think it should be supported to allow network to configure either one or two reference configuration to reduce overhead for their network implementation. If we always use only one reference configuration many configurations of cells belonging to other CU will be full configuration.

Proposal 2: Multiple reference configurations are introduced for inter-CU LTM.

Issue 3: LTM candidate configuration ID determination

In last meeting, the following agreement was reached:

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

To achieve this purpose, a straightforward solution is to let the source gNB-CU assign the LTM candidate ID, and provide it to the LTM candidate gNB-CU during the preparation procedure.

Moreover, when the inter-CU LTM cell switch occurs, the source gNB-CU should share the mapping information between LTM candidate ID and the LTM candidate cell ID to the new target gNB-CU. The intention is to help the new target gNB-CU carry out the subsequent LTM cell switch.

-	In LTM preparation phase, source gNB/CU informs a candidate gNB/CU of the candidate’s own id(s).

-	In LTM cell switch, source gNB/CU informs a target gNB/CU of all candidate ids.

However, sharing the mapping relationship between LTM candidate ID and the LTM candidate cell ID to the target gNB-CU could also performed in LTM preparation procedure. We think it is not clear now so make it to clear.

Proposal 3: RAN2 discuss how/when the source gNB-CU share the mapping relationship between LTM candidate ID and the LTM candidate cell ID to the target gNB-CU.

Issue 4: inter-node message for LTM-candidate

It is natural that the Rel-18 LTM signalling procedure should be used for inter-CU LTM and intra-CU LTM co-existence scenario. It means RAN2 should determine the Stage 2 signalling procedure for inter-CU LTM without changing the legacy intra-CU procedures. For inter-CU LTM, we expect that at least inter-gNB-CU communication between serving gNB-CU and the candidate gNB-CUs with extending the HANDOVER_REQUEST/ HANDOVER_REQUEST_ACK or introducing the new Xn message(s). If we focus on the Xn message, there would be two approaches to deliver the inter-CU LTM pre-configurations based on our understanding.

- Option 1: One LTM configuration request is included in a HANDOVER_REQUEST message and the HANDOVER_REQUEST_ACK message also includes a LTM candidate configuration.

  - Reuse the legacy Handover preparation procedure as much as possible (i.e. each handover preparation procedure is used for one LTM candidate cell preparation). This approach is used in CHO case.

  - LTM cell switch MAC CE only provides the candidate cell ID (no change).

  - Require many interaction of Xn messages (e.g. HANDOVER_REQUEST/ HANDOVER_REQUEST_ACK)

  - Separate structure of LTM preparation. DU-level signalling coordination between source gNB and candidate cells i.e. candidate gNB-CU has less role.

- Option 2: Multiple LTM configuration requests within a gNB-CU could be included in a HANDOVER_REQUEST message.

  - Prepare multiple candidate cells over one handover preparation procedure. Bring some RAN2/RAN3 impacts. For example, each HO req ACK message will contain multiple LTM candidate configurations i.e. how to provide those multiple configurations need to be determined.

  - LTM cell switch MAC CE needs to provide the gNB-CU ID and the associated candidate cell ID (no change if the unique LTM candidate cell ID is used).

  - Less interaction of Xn messages (e.g. HANDOVER_REQUEST/ HANDOVER_REQUEST_ACK)

  - Parallel structure of LTM preparation between gNB-CUs. Each gNB-CU follows the Rel-18 intra-CU LTM signalling procedure.

- Figure 1. Signaling flow on the LTM preparation phase

From our understanding, both options have pros/cons in different aspects (e.g. signalling reduction, simple implementation reusing the Rel-18 intra-CU LTM, etc.) and it has impact on RAN3. However, we think that one LTM configuration request is included in a HANDOVER_REQUEST message and the HANDOVER_REQUEST_ACK message is simple if we introduce the unique LTM candidate cell ID across multiple CUs, so we prefer option 1

Proposal 4: RAN2 request RAN3 to introduce handover message that source gNB can assign unique LTM cell ID for each UE across CUs.

During inter-CU LTM candidate cell preparation procedure, the gNB-CU of LTM candidate cell needs to provide the information in LTM-candidate to the source gNB-CU. In R18, the LTM-candidate IE contains several IEs in addition to RRCReconfiguration container, as shown below:

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...

}

However, during legacy handover preparation procedure, the target gNB-CU only provides the RRCReconfiguration message as the inter-node RRC message to the source gNB-CU. Thus, the issue is how to provide other information in LTM-candidate by the LTM candidate gNB-CU. Two options can be considered:

- Option 1: Define a new inter-node RRC message for LTM-candidate

- This option has RAN2 specification impact. Meanwhile, the source gNB-CU should be able to decode this message since some information inside needs to be sent to the source gNB-DU, e.g., Early UL sync configuration, UE measured TA ID.  Such option can simplify RAN3 signaling design since the Handover Request Acknowledge message only needs include the container of LTM-candidate.

- Work load

  - Rel-19

RAN2 - define new inter-node RRC message;

RAN3 - include such new inter-node RRC message as the response for LTM candidate cell preparation.

  - Beyond R19:

If RAN2 defines new IE in LTM-candidate, the work is only within RAN2 scope;

RAN3 doesn’t need to be aware of such changes.

- Option 2: Define new XnAP IEs to include other information in the LTM-Candidate

- This option does not have RAN2 specification impact. However, it will cause large specification impact in RAN3. The reason is that RAN3 should add IEs in LTM-candidate over Handover Request Acknowledge message.

- Work load

  - Rel-19

RAN2 - no work to do if LTM-candidate is not enhanced in R19;

RAN3 -  needs define new IEs corresponding to IEs in LTM-candidate

  - Beyond R19:

If RAN2 defines new IEs in LTM-candidate,

RAN2 sends LS to RAN3, and RAN3 adds it to XnAP.

Proposal 5: RAN2 is kindly asked to discuss the LTM candidate configuration (ltm-candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

- Option 1: Define a new inter-node RRC message for LTM candidate configuration

- Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration

Issue 5: PDU session admission control for LTM candidate cell preparation

When preparing one inter-CU LTM candidate cell, the candidate gNB-CU should perform the admission control for the PDU sessions. If different inter-CU LTM candidate cells have different admission results (i.e., different LTM candidate cells have different accepted PDU sessions), the UE may need perform the PDU session addition/release during the subsequent LTM cell switch. Fig. 1 shows one example. The source cell admits PDU session 1&2&3. However, when preparing the LTM candidate cells, the LTM candidate cell 1 can only accept PDU session 1&2, while the LTM candidate cell 2 can accept PDU session 1&2&3. During the subsequent LTM cell switch in the order of “source cell  LTM candidate cell 1  LTM candidate cell 2”, the UE encounters a situation that a released PDU session appear in a new cell without establishment, i.e., :

- After UE accesses cell 2 (LTM candidate cell 1), it will release PDU session 3, which may result in the service release at the application layer.

- After UE accesses cell 3(LTM candidate cell 2), the RRC reconfiguration of cell 3 contains the DRB(s) for the PDU session 3. However, the UE already releases PDU session 3 in cell 2. Normally, a PDU session served by the DRB should be established first. However, this case results in that the released PDU session (i.e., PDU session 3) in cell 2 appears in cell 3 without PDU session establishment procedure.

Fig. 1 Example on the appearance of a released PDU session without PDU session setup

The above case where a released PDU session appears in a new cell without PDU session setup should be handled properly at the UE side. Two options can be considered:

  - Option 1: After accessing to a LTM candidate cell, UE automatically releases the PDU session, which has been released before. This introduces specification impact (RAN2).

  - Option 2: The inter-CU LTM candidate cell is prepared only if all LTM candidate cells have the same admission result. This seems to be the principle applied in intra-CU LTM.

Proposal 6: RAN2 is kindly asked to discuss whether different LTM candidate cells can have different admission results (i.e., different LTM candidate cells accept different PDU sessions).

Issue 5: LTM configuration during L3 handover or PSCell addition/change

For intra-CU LTM, RAN2 agrees that the LTM candidate configurations can be kept/added/modified/released at the UE side when performing L3 handover, and it is the main objective of subsequent LTM in intra-CU. For inter-CU case, RAN2 needs to discuss whether this is also applicable or not. In our understanding, by keeping/adding/modifying/releasing the LTM candidate configurations, the signalling overhead can be saved. Moreover, after the L3 handover, the UE can continuously perform the LTM cell switch. Thus, it is beneficial to follow the design in Rel-18. Similarly, for PSCell addition/change, the MCG/SCG LTM candidate configurations can be also kept/added/modified/released.

Proposal 7: During L3 HO or PSCell addition/change, the inter-CU/intra-CU LTM candidate configurations can be kept/added/modified/released.

Conclusion

Proposal 1-1: Source CU manage the CSI resource configuration across all CUs.

Proposal 1-2: Each candidate cell provides the CSI report configuration for inter-CU LTM.

Proposal 2: Multiple reference configurations are introduced for inter-CU LTM.

Proposal 3: RAN2 discuss how/when the source gNB-CU share the mapping relationship between LTM candidate ID and the LTM candidate cell ID to the target gNB-CU.

Proposal 4: RAN2 request RAN3 to introduce handover message that source gNB can assign unique LTM cell ID for each UE across CUs.

Proposal 5: RAN2 is kindly asked to discuss the LTM candidate configuration (ltm-candidate IE) transfer from candidate gNB-CU to the source gNB-CU during the LTM cell preparation procedure by considering the following two options:

- Option 1: Define a new inter-node RRC message for LTM candidate configuration

- Option 2: Define new XnAP IEs to include other information in the LTM candidate configuration

Proposal 6: RAN2 is kindly asked to discuss whether different LTM candidate cells can have different admission results (i.e., different LTM candidate cells accept different PDU sessions).

Proposal 7: During L3 HO or PSCell addition/change, the inter-CU/intra-CU LTM candidate configurations can be kept/added/modified/released.

References