3GPP TSG-RAN WG2 Meeting #127	R2-2407422

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining Issues for Measurement Event Evaluation

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, RAN2 reached the following agreements for event-triggered L1 measurement enhancements for LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we will further address event triggered L1 measurement evaluation in detail.

Discussion

Introduction

Issue 1: Need of event LTM1

In L3 measurement framework, A1 event is defined as below

- Event A1:	Serving becomes better than absolute threshold;

From our understanding, Event A1 is not used to determine the mobility (e.g. handover) but it is helpful for the network to operate the measurement/mobility operations. For example, the network could stop the measurement of Inter-frequency or Inter-RAT measurements when UE report the Event A1 so the UE measurements could be optimized by this Event A1 configuration.

- Event LTM1: Beam of serving cell better than absolute threshold;

We also think this Event LTM1 is not used for the LTM cell switch but it could be used for the network to operate the L1 based measurement configuration based on the results of the Event LTM1 by UE. Information based on Event LTM1 may already be reported using beam management reporting specified for Rel-19 MIMO. So we can focus on events/reports which involve neighbour cell measurements for mobility.

Proposal 1: Event LTM1 (i.e. Beam of serving cell better than absolute threshold) is not defined for LTM.

Issue 2:  Beam(s) of the serving cell and neighboring cell used for event evaluation

All events for LTM measurement which RAN2 agreed use the beam(s) of the serving cell and neighbouring cell but what beam(s) is used for evaluation is not determined. This issue was treated in the Rel-19 MIMO WI in RAN1, and some agreements were made.

In short, we summarize how the ‘current beam’ (beam for the serving cell) and the ‘new beam’ (beam for the neighbouring cell) are used based on RAN1 agreements below.

- Beam for the serving cell

  - The beam for the serving cell is implicitly derived from a QCL RS of indicated TCI state

  - Since the measurement quality is related with DL channel quality, the RS QCL RS of indicated TCI state is the indicated TCI for PDSCH.

- Beam for the neighbouring cell

  - The beam for the neighbouring cells are explicitly configured by RRC

  - CSI resource configuration provides the list of beams for the neighbouring cells

  - Detail signalling support will be further discussed

Agreement in RAN1#116bis (Event 2, quality metric: L1-RSRP, new beam) On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement in RAN1#116bis (current beam: QCL RS of indicated TCI state) On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured Agreement in RAN1#117 (two schemes for current beam determination) Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. Agreement in RAN1#117 (further determination for current beam) Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded. Agreement in RAN1#117 (new beam determination) Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list Agreement in RAN1#117 (new beam determination) Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

We think the above rule is quite reasonable and it could be easily applicable to the event evaluation for LTM measurement.

Proposal 2: The beam for the serving cell is implicitly derived from a QCL RS of indicated TCI state associated with the PDSCH reception.

Proposal 3: The beam for the neighbouring cells are explicitly configured by RRC i.e. CSI resource configuration provides the list of beams for the neighbouring cells.

RAN2 agreed to support the both SSB and CSI-RS for L1 measurement resource configuration in LTM configuration. Then, the next question is how to guarantee the same type of RSes between the current beam and the new beam because the indicated TCI state is dynamically changed. We think some tricky network configuration solve the issues, see the below options.

- Option 1: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

- Option 2: Only one type of RS is configured for the new beam as a NW restriction, and UE perform the L1 measurement when RS type of the current beam is same with the new beam for event L1 measurement.

We think Option 1 achieves the better performance than Option 2 but only have small signalling overhead than Option 2. For Option 1, UE determines the type of the target beam based on the current UE beam i.e. if the indicated beam is SSB then UE compare the SSB of the target beam from both configured beams.

Proposal 4: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

Issue 3:  Need of cell level measurement result for event evaluation

Based on the RAN2 discussion in RAN#126, companies think it is better to evaluate the LTM event using not only the single best beam but also the multiple beams because of the concerns on the ping-pong between the source cell and target cell. Here, we think the cell-level measurement means the multiple beams are used for the L1 event evaluation for cell quality but we think it is not really needed for LTM cell switch. The legacy LTM L1 measurement report only report the L1 RSRP values to the gNB, then gNB use this report for the decision of cell switch. Since this LTM L1 event-triggered measurement report is also focusing on the beam level quality and mobility based on the best beam, RAN2 only defines the LTM event in terms of beam level.

We think further enhancements to avoid the ping-pong problem on the LTM cell switch can be solved by the multiple beam report in the event triggered L1 measurement report. For details, how to configure the multiple beams could be considered.

The basic assumption is that the current beam which is indicated by NW is always the best beam from the serving cell, and this beam is dynamically changed considering the channel situation. If we apply the LTM events as below the beam measurement of serving cell could be used for Event LTM2/3/5 (yellow highlight), to compare and the evaluate the condition, only one beam of serving cell is simple. Single beam in neighbor cell is also the basic and I think it can be further enhanced to extend it to the multiple beams.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

In addition, LTM event report is used to determine the cell switch by NW. From our understanding, the cell switch is determined based on the “current beam” we think it is the “indicated TCI state by NW i.e. one beam” if we allow the multiple beams in serving cell, some other beams are not currently used so it should not be considered for cell switch decision.

Proposal 5: Do not support cell-level measurement for event evaluation.

In addition, the configured TTT, hysteresis, beam offset can be also applied upon the current beam change because we think this parameters could be configured by network to consider the beams in the source cell i.e. they are not the beam specific parameters. So, the network needs to configure these parameters semi-static manner by RRC.

Proposal 6: Each Parameter to avoid ping-pong i.e. TTT, hysteresis, beam offset and L1 filtering is configured with one value under the event based LTM measurement configuration i.e. do not introduce the multiple values for multiple beams.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 define the Event LTM1 (i.e. Beam of serving cell better than absolute threshold).

Proposal 2: The beam for the serving cell is implicitly derived from a QCL RS of indicated TCI state associated with the PDSCH reception.

Proposal 3: The beam for the neighbouring cells are explicitly configured by RRC i.e. CSI resource configuration provides the list of beams for the neighbouring cells.

Proposal 4: NW configures the both types of RS (SSB and CSI-RS) for the new beam(s), UE could select the same type of beam when UE evaluate the L1 event measurement.

Proposal 5: Do not support cell-level measurement for event evaluation.

Proposal 6: Each Parameter to avoid ping-pong i.e. TTT, hysteresis, beam offset and L1 filtering is configured with one value under the event based LTM measurement configuration i.e. do not introduce the multiple values for multiple beams.