3GPP TSG-RAN WG2 Meeting #127	R2-2407423

Maastricht, Netherlands, Aug. 19th~23rd , 2024

Agenda item:	8.6.4

Source:	Samsung

Title:	Support of Event Triggered L1 Measurement Reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, RAN2 reached the following agreements for event-triggered L1 measurement enhancements for LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we will further address event triggered L1 measurement reporting in detail.

Discussion

Introduction

Issue 1: What information in the measurement report

We think the functionality of L1 measurement report in R19 mobility WI and MIMO WI is similar so it would be better to align the L1 measurement report procedure. Otherwise, we have two different signalling (MAC CE and DCI) for the similar functions and the network selects one of them for their implementation but UE needs to implement both because UE have no idea which signalling is used by NW.

NOTE: ICBM (inter cell beam management) in MIMO and LTM are not the exactly same features. However, we think that common frame (i.e. measurement can be for neighbour cells with different PCI from serving cell) can be applicable because it is still common that L1 measurement reporting based on event is needed. Further aspects can be added e.g. additional events for LTM. If larger reporting is needed for LTM, MAC CE can be used for LTM while UCI is used for MIMO, but we think there are no big differences between the event reporting for LTM and MIMO (i.e. contents of the measurement reporting could be same; no additional contents are really needed for LTM)

First, the event triggered L1 measurement reporting is the work scope in both Rel-19 MIMO and mobility enhancement WIs and RAN1 made good progress to support event based L1 measurement reporting in Rel-19 MIMO WI. The main agreements impacting on RAN2 issues on the LTM are provided below.

- UCI is used for the event based L1 measurement reporting

  - Mode A (dynamically scheduling UCI by gNB)

    - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report

    - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report.

    - Step 3: Beam report is transmitted in second UL channel.

  - Mode B (UCI in pre-configured resource(s) for second UL channel

    - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

    - Step 2: UE transmits the beam report in the second UL channel.

- Define the L1 measurement event

  - Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - Additional L1 events (similar as LTM events which RAN2 agreed but there are some differences)

- Signalling contents

  - Includes at least support DL RS resource indicator and L1-RSRP

  - N ≥ 1 beam(s) are reported in the report instance (N is configured by gNB)

    - At least one of N reported beam(s) should satisfy the condition

Based on above summary of RAN1 agreements, about UCI based L1 measurement report could applied to LTM as well and we also think using the UCI as the event based L1 measurement reporting is consistent approach because we have UCI based L1 reporting for the periodic L1 measurement report. It also has a latency advantage than MAC CE approach, and we think it is very important aspect to determine the signalling support for the event based L1 measurement reporting.

For the contents of the event based L1 measurement reporting, we think the multiple beams of the neighbouring cell could be reported based on the network configuration along with the current beam. In addition, the information to associate the measurement event ID could be added in order to identify for the NW to know what event is triggered. We think the additional information could be added and further discussed, but we prefer to add only the essential information is enough. The legacy CSI measurement report (i.e. legacy L1 report and LTM report) includes the L1 RSRP measurement report for the configured CSI resources but the event L1 measurement report don’t need to report all configured RSes. UE only need to report the L1 measurement report with some selected CSI resources (i.e. RSes which meet the event condition)

Proposal 1: RAN2 confirm the RAN1 agreements using UCI L1 measurement reporting to align the event based L1 measurement reporting is used for LTM.

Proposal 2: Event triggered L1 measurement reporting for LTM includes following information:

- L1 RSRP of the current beam

- L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

- DL RS resource index (SSB or CSI-RS)

Issue 2: Need of measurement reporting once leaving condition is met

In the legacy L3 measurement reporting, measurement report once the leaving condition is met is supported and it is very helpful for the network to operate the mobility. However, we think the L1 measurement report is not L3 filtered information and the quality of the best beam(s) could be changed dynamically even though the additional methods (e.g. TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset) will be introduced. Frequent ping-pong L1 measurement report is one of the concerns to introduce the event triggered L1 measurement report. We think the frequent L1 measurement report will be estimated if we introduce the “leaving condition” for the additional report.

Proposal 3: Do not support the event triggered L1 measurement reporting once leaving condition is met.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: RAN2 confirm the RAN1 agreements using UCI L1 measurement reporting to align the event based L1 measurement reporting is used for LTM.

Proposal 2: Event triggered L1 measurement reporting for LTM includes following information:

- L1 RSRP of the current beam

- L1 RSRP of the N ≥ 1 beams for the neighbouring cell (at least one of N reported beam(s) should satisfy the condition)

- DL RS resource index (SSB or CSI-RS)

Proposal 3: Do not support the event triggered L1 measurement reporting once leaving condition is met.