3GPP TSG-RAN WG2 Meeting #127	R2-2407428

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	6.1.3.1

Source:	Nokia

Title:	Discussion on indication of reserved SN-side NR-DC and aggregated BW resources

WID/SID:	NR_NewRAT-Core, NR_eMIMO-Core, NR_BCS4-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

For inter-node coordination, the specifications restrict the usage of the field CG-Config > configRestrictModReq to SN-initiated requests to modify the SCG configuration. However, in Rel-17 two parameters were added within the ConfigRestrictModReqSCG IE referenced by this field that appear to allow them to be used to indicate the actual reserved or configured resources of the SCG in response to an MN-initiated configuration, i.e. in apparent contradiction to the restrictions on configRestrictModReq.

CG-Config was also extended in Rel-18, and now that Rel-18 is frozen, there is not necessarily a “clean” way to correct this discrepancy. This contribution discusses a couple of options for correcting this and proposes the “cleaner” of the two.

# 2	Discussion

## 2.1	Current CG indications

The relevant CG-Config IEs and fields are shown below.

The IE ConfigRestrictModReqSCG is only referenced by the field configRestrictModReq. Per the field description, configRestrictModReq can only be used when SN initiates a request to modify the CG configuration and not in response to a MN-initiated CG configuration (i.e. not in response to a CG-Config-Info message).

Two fields were added to ConfigRestrictModReqSCG in Rel-17: reservedResourceConfigNRDC and aggregatedBandwidthSN. For both of those parameters, the field description states that they can be used to “request or indicate” the resources at the SN-side. The use of the term “indicate” here seems to imply that these fields can also be used to indicate the actual reserved/configured resources at the SN-side, i.e. in response to an MN-initiated CG configuration. Hence there seems to be a contradiction between the intended use of reservedResourceConfigNRDC and aggregatedBandwidthSN and the fact that they can be indicated within ConfigRestrictModReqSCG only.

Note that, based on their field descriptions, all other fields within ConfigRestrictModReqSCG are only used for requesting configuration changes (i.e. in SN-initiated requests), which is in line with the field description of configRestrictModReq.

Observation 1: The intended use of the reservedResourceConfigNRDC and aggregatedBandwidthSN fields to indicate the actual reserved/configured resources at the SN-side contradicts the restriction that configRestrictModReq can only be used by the SN to request a modification to the CG configuration.

CG-Config-IEs ::=                   SEQUENCE {

scg-CellGroupConfig                 OCTET STRING (CONTAINING RRCReconfiguration)    OPTIONAL,

scg-RB-Config                       OCTET STRING (CONTAINING RadioBearerConfig)     OPTIONAL,

configRestrictModReq                ConfigRestrictModReqSCG                         OPTIONAL,

drx-InfoSCG                         DRX-Info                                        OPTIONAL,

candidateCellInfoListSN             OCTET STRING (CONTAINING MeasResultList2NR)     OPTIONAL,

measConfigSN                        MeasConfigSN                                    OPTIONAL,

selectedBandCombination             BandCombinationInfoSN                           OPTIONAL,

fr-InfoListSCG                      FR-InfoList                                     OPTIONAL,

candidateServingFreqListNR          CandidateServingFreqListNR                      OPTIONAL,

nonCriticalExtension                CG-Config-v1540-IEs                             OPTIONAL

}

< omitted for clarity >

CG-Config-v1730-IEs ::=             SEQUENCE {

fr1-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

fr2-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

nonCriticalExtension                CG-Config-v1800-IEs                             OPTIONAL

}

CG-Config-v1800-IEs ::=             SEQUENCE {

candidateServingFreqRangeListNR-r18    CandidateServingFreqRangeListNR-r18             OPTIONAL,

candidateServingFreqListNR-r16         CandidateServingFreqListNR-r16                  OPTIONAL,

idc-TDM-AssistanceConfig-r18           ENUMERATED {enabled}                            OPTIONAL,

candidateCellInfoListSubsequentCPC-r18 CandidateCellInfoListCPC-r17                    OPTIONAL,

scpac-ReferenceConfigurationSCG-r18    ReferenceConfiguration-r18                      OPTIONAL,

subsequentCPAC-Information-r18         CandidateCellInfoListCPC-r17                    OPTIONAL,

successPSCell-Config-r18               SuccessPSCell-Config-r18                        OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                     OPTIONAL

}

< omitted for clarity >

ConfigRestrictModReqSCG ::=         SEQUENCE {

requestedBC-MRDC                    BandCombinationInfoSN                              OPTIONAL,

requestedP-MaxFR1                   P-Max                                              OPTIONAL,

...,

[[

requestedPDCCH-BlindDetectionSCG    INTEGER (1..15)                                    OPTIONAL,

requestedP-MaxEUTRA                 P-Max                                              OPTIONAL

]],

[[

requestedP-MaxFR2-r16               P-Max                                              OPTIONAL,

requestedMaxInterFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                    OPTIONAL,

requestedMaxIntraFreqMeasIdSCG-r16  INTEGER(1..maxMeasIdentitiesMN)                    OPTIONAL,

requestedToffset-r16                T-Offset-r16                                       OPTIONAL

]],

[[

reservedResourceConfigNRDC-r17      ResourceConfigNRDC-r17                             OPTIONAL

]],

[[

aggregatedBandwidthSN-r17           AggregatedBandwidthSN-r17                          OPTIONAL

]],

[[

requestedMaxLTM-CandidateIdSCG-r18  INTEGER(0..maxNrofLTM-Configs-r18)                 OPTIONAL

]]

}

CG-Config field descriptions

aggregatedBandwidthSN Used to indicate or request the maximum aggregated bandwidth at the SN side if the supportedAggBW-FR1 was reported for the requestedBC-MRDC. This field is only used in NR-DC. - aggBW-FDD-DL/UL-r17 indicates the aggregated bandwidth across FDD DL/UL CCs in SCG; - aggBW-TDD-DL/UL-r17 indicates the aggregated bandwidth across TDD DL/UL CCs in SCG; - aggBW-TotalDL/UL-r17 indicates the aggregated bandwidth across all DL/UL CCs in SCG.

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can be used to request configuring an NR band combination whose use MN has previously forbidden. SN only includes this field in SN-initiated procedures.

reservedResourceConfigNRDC Used to request or indicate the maximum number of resources reserved for the SCG. This field is only used in NR-DC.

## 2.2	Resolving the contradiction

At first glance, one might be tempted to remove the restriction on configRestrictModReq. However, it should be noted that the reason why the a restriction was introduced to configRestrictModReq in the first place was to avoid the occurrence of nested reconfiguration procedures between MN and SN (see the CR R2-2102440): MN can always reject an SN-initiated request to support another band combination (or a request to modify the configuration of an existing BC). Therefore, keeping the existing restriction in place simplifies handling at both sides and should not be removed.

Ideally, a Rel-17 extension could be added CG-Config with another field for indicating the actual reserved NR-DC resources and actual configured aggregated bandwidth at the SN side (re-using the Rel-17 fields), similar to the selectedBandCombination field defined within CG-Config-IEs. The problem, however, is that a Rel-18 extension has already been defined on top of CG-Config-v1730-IEs, so an additional Rel-17 extension cannot just be inserted between the v1730 and v1800 IEs. Based on this limitation, we discuss two options:

Approach A: Dummify the Rel-18 CG-Config IE and add a Rel-17 extension, i.e. dummify the v1800 parameters, then add a new v17xy IE with a field for indicating the actual reserved/configured resources at the SN-side, and then extend it again with the v18xy parameters. The result would look something like below.

CG-Config-v1730-IEs ::=             SEQUENCE {

fr1-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

fr2-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

nonCriticalExtension                dummy-IEs                                       OPTIONAL

}

dummy-IEs ::=                       SEQUENCE {

dummy1                              CandidateServingFreqRangeListNR-r18             OPTIONAL,

dummy2                              CandidateServingFreqListNR-r16                  OPTIONAL,

dummy3                              ENUMERATED {enabled}                            OPTIONAL,

dummy4                              CandidateCellInfoListCPC-r17                    OPTIONAL,

dummy5                              ReferenceConfiguration-r18                      OPTIONAL,

dummy6                              CandidateCellInfoListCPC-r17                    OPTIONAL,

dummy7                              SuccessPSCell-Config-r18                        OPTIONAL,

nonCriticalExtension                CG-Config-v17xy-IEs                             OPTIONAL

}

CG-Config-v17xy-IEs ::=             SEQUENCE {

configuredResourcesSCG-r17          ConfiguredResourcesSCG-r17                      OPTIONAL,

nonCriticalExtension                CG-Config-v18xy-IEs                             OPTIONAL

}

CG-Config-v18xy-IEs ::=             SEQUENCE {

candidateServingFreqRangeListNR-r18    CandidateServingFreqRangeListNR-r18             OPTIONAL,

candidateServingFreqListNR-r16         CandidateServingFreqListNR-r16                  OPTIONAL,

idc-TDM-AssistanceConfig-r18           ENUMERATED {enabled}                            OPTIONAL,

candidateCellInfoListSubsequentCPC-r18 CandidateCellInfoListCPC-r17                    OPTIONAL,

scpac-ReferenceConfigurationSCG-r18    ReferenceConfiguration-r18                      OPTIONAL,

subsequentCPAC-Information-r18         CandidateCellInfoListCPC-r17                    OPTIONAL,

successPSCell-Config-r18               SuccessPSCell-Config-r18                        OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                     OPTIONAL

}

ConfiguredResourcesSCG-r17 ::=      SEQUENCE {

aggregatedBandwidthSN-r17           AggregatedBandwidthSN-r17                          OPTIONAL,

reservedResourceConfigNRDC-r17      ResourceConfigNRDC-r17                             OPTIONAL,

...

}

Technically, Approach A works, but it means that dummy bits would always be wasted whenever the CG-Config extensions past v1730 are to be used, which is really not a clean solution.

Observation 2: Dummifying the Rel-18 CG-Config IEs to add a late Rel-17 extension will result in wasted bits being signalled whenever CG-Config extensions past v1730 are used.

On the other hand, if we can accept that the SN indicating it’s actual reserved/configured resources is not always a critical behaviour, then the extension could instead be delayed until Rel-18. The SN must anyway follow the allowable configuration restrictions set by the MN in CG-Config-Info, so the main advantage to allowing SN to also indicate the actual reserved/configured resources at the SN side is so the MN can know what resources from among the allowable SN resource pool were not used, for example, in case the MN wants to reconfigure the MCG with additional resources. Hence, we have the other Approach B:

Approach B:

- For Rel-17, update aggregatedBandwidthSN-r17 and reservedResourceConfigNRDC-r17 so the fields can only be used to request a modification to the corresponding resources (i.e. remove the statement that SN uses these fields to indicate the actual configured/reserved aggregated bandwidth and NRDC resources, respectively), which is aligned with the actual supported inter-node behaviour in Rel-17. This would require the following modification to the Rel-17 field descriptions and no further ASN.1 change.

CG-Config field descriptions

aggregatedBandwidthSN Used to indicate or request the maximum aggregated bandwidth at the SN side if the supportedAggBW-FR1 was reported for the requestedBC-MRDC. This field is only used in NR-DC. - aggBW-FDD-DL/UL-r17 indicates the aggregated bandwidth across FDD DL/UL CCs in SCG; - aggBW-TDD-DL/UL-r17 indicates the aggregated bandwidth across TDD DL/UL CCs in SCG; - aggBW-TotalDL/UL-r17 indicates the aggregated bandwidth across all DL/UL CCs in SCG.

reservedResourceConfigNRDC Used to request or indicate the maximum number of resources reserved for the SCG. This field is only used in NR-DC.

- For Rel-18, add a new Rel-18 CG-Config extension to allow the SN to indicate the actual reserved/configured NR-DC and aggregated BW resources (from reservedResourceConfigNRDC-r17 and aggregatedBandwidthSN-r17, respectively) in response to an MN-initiated CG configuration. The resulting ASN.1 would look something like below:

CG-Config-v1730-IEs ::=             SEQUENCE {

fr1-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

fr2-Carriers-SCG-r17                INTEGER (1..32)                                 OPTIONAL,

nonCriticalExtension                CG-Config-v1800-IEs                             OPTIONAL

}

CG-Config-v1800-IEs ::=             SEQUENCE {

candidateServingFreqRangeListNR-r18    CandidateServingFreqRangeListNR-r18             OPTIONAL,

candidateServingFreqListNR-r16         CandidateServingFreqListNR-r16                  OPTIONAL,

idc-TDM-AssistanceConfig-r18           ENUMERATED {enabled}                            OPTIONAL,

candidateCellInfoListSubsequentCPC-r18 CandidateCellInfoListCPC-r17                    OPTIONAL,

scpac-ReferenceConfigurationSCG-r18    ReferenceConfiguration-r18                      OPTIONAL,

subsequentCPAC-Information-r18         CandidateCellInfoListCPC-r17                    OPTIONAL,

successPSCell-Config-r18               SuccessPSCell-Config-r18                        OPTIONAL,

nonCriticalExtension                   CG-Config-v18xy-IEs                             OPTIONAL

}

CG-Config-v18xy-IEs ::=             SEQUENCE {

configuredResourcesSCG-r18          ConfiguredResourcesSCG-r18                      OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                     OPTIONAL

}

ConfiguredResourcesSCG-r18 ::=      SEQUENCE {

aggregatedBandwidthSN-r17           AggregatedBandwidthSN-r17                          OPTIONAL,

reservedResourceConfigNRDC-r17      ResourceConfigNRDC-r17                             OPTIONAL,

...

}

In our understanding, having Rel-17 SNs not being able to indicate the actual reserved/configured NR-DC and aggregate BW resources only limits some flexibility and does not actually break any functionality (and it’s anyway how this procedure is actually supported in Rel-17 today), so this seems like a cleaner approach than dummifying the Rel-18 parameters to try to accommodate the flexibility in Rel-17.

Observation 3: Not allowing SN to indicate the reserved/configured NR-DC and aggregated BW resources only limits some flexibility in Rel-17, but does not break any functionality, as SN must anyway follow the allowable configurations provided by the MN.

Proposal 1: Agree to following approach for handling indication of reserved SN-side NR-DC and aggregated BW resources:

- For Rel-17, update aggregatedBandwidthSN-r17 and reservedResourceConfigNRDC-r17 so the fields can only be used to request a modification to the corresponding resources (i.e. remove the statement that SN uses these fields to indicate the actual configured/reserved aggregated bandwidth and NRDC resources, respectively), which is aligned with the actual supported inter-node behaviour in Rel-17.

- For Rel-18, add a new Rel-18 CG-Config extension to allow the SN to indicate the actual reserved/configured NR-DC and aggregated BW resources (from reservedResourceConfigNRDC-r17 and aggregatedBandwidthSN-r17, respectively) in response to an MN-initiated CG configuration.

We have created Rel-17 and Rel-18 CRs in R2-2407429 and R2-2407430 for the respective changes in Proposal 1.

Proposal 2: Agree to CRs in R2-2407429 and R2-2407430 confirming the changes in Proposal 1.

# 3	Conclusion

This document has made the following observations:

Observation 1: The intended use of the reservedResourceConfigNRDC and aggregatedBandwidthSN fields to indicate the actual reserved/configured resources at the SN-side contradicts the restriction that configRestrictModReq can only be used by the SN to request a modification to the CG configuration.

Observation 2: Dummifying the Rel-18 CG-Config IEs to add a late Rel-17 extension will result in wasted bits being signalled whenever CG-Config extensions past v1730 are used.

Observation 3: Not allowing SN to indicate the reserved/configured NR-DC and aggregated BW resources only limits some flexibility in Rel-17, but does not break any functionality, as SN must anyway follow the allowable configurations provided by the MN.

And proposed the following:

Proposal 1: Agree to following approach for handling indication of reserved SN-side NR-DC and aggregated BW resources:

- For Rel-17, update aggregatedBandwidthSN-r17 and reservedResourceConfigNRDC-r17 so the fields can only be used to request a modification to the corresponding resources (i.e. remove the statement that SN uses these fields to indicate the actual configured/reserved aggregated bandwidth and NRDC resources, respectively), which is aligned with the actual supported inter-node behaviour in Rel-17.

- For Rel-18, add a new Rel-18 CG-Config extension to allow the SN to indicate the actual reserved/configured NR-DC and aggregated BW resources (from reservedResourceConfigNRDC-r17 and aggregatedBandwidthSN-r17, respectively) in response to an MN-initiated CG configuration.

Proposal 2: Agree to CRs in R2-2407429 and R2-2407430 confirming the changes in Proposal 1.