3GPP TSG-RAN WG2 Meeting #127	R2-2407438

Maastricht, The Netherlands, 19-23 August 2024

Agenda item:	8.5.3

Source:		Lenovo

Title:			Remaining essential issues for study

Document for:	Discussion and Decision

# Introduction

This paper builds on the agreements made in previous (RAN2#126) meeting.

# Discussion

## Cell reselection:

RAN2 should strive to reuse the current principles as much as possible. As an example, we can reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ) to decide if intra/ inter frequency measurements need to be performed.

If the best ranked cell or a higher priority inter-frequency cell is R19 NES Cell UE should go on and request SIB1. Here we assume Network can ensure SIntraSearchP and SIntraSearchQ are sensible to allow time to the UE to acquire SIB1 of NES Cell before the serving cell deteriorates too much.

Proposal 1: For cell reselections, reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ to decide if intra/ inter frequency measurements need to be performed).

Proposal 2: If the best ranked cell or a higher priority inter-frequency cell is a R19 NES Cell, then UE should go on and request SIB1.

## SIB1 validity

UE is on cell A and from System Information broadcast, it would know if the best cell (intra-frequency or same priority inter-frequency) is a NES cell (or not). It will request SIB1 of NES Cell only if this is the best radio cell on that frequency. If the UE obtained SIB1 of a NES cell but is camping on a different cell (say on Cell X which is not a Cell A i.e., not providing WUS information for a NES cell of interest, as shown in Figure 1), how long can it consider the stored SIB1 and the corresponding WUS configuration as valid?

Figure 1: Cell A, Cell X and NES Cell

For SIB1 there’s neither a value Tag nor a validity timer defined, and we believe that introducing a SIB1 validity concept at this late stage of 5G is not desirable. Instead, a configurable WUS validity can be included in the WUS configuration itself. Further, following simple principles would help:

Principle a) Stored SIB1 is considered valid for cell camping (reselection) purpose if the corresponding WUS configuration is considered valid.

Principle b) After reselection, the UE shall initiate SIB1 request again from the serving NES cell i.e., not wait until RRC Connection needs to be established.

Principle c) Else, if WUS configuration is not valid or has not been acquired, UE proceeds with the cell reselection procedure as if the said NES cell was no more the best ranking cell, i.e., shall not consider the NES cell for cell re-selection before obtaining valid WUS config of an NES cell; the intraFreqReselection parameter broadcasted in MIB and/ or SIB1 can be used for intra frequency (= cell X’s frequency) reselection.

Proposal 3: A configurable WUS validity can be included in the WUS configuration itself.

Proposal 4: RAN2 kindly discuss if the following principles suffice for R19 NES on-demand SIB1 based cell reselection:

Principle a) Stored SIB1 is considered valid for cell camping (reselection) purpose if the corresponding WUS configuration is still valid.

Principle b) After reselection, the UE shall initiate SIB1 request again from the serving NES cell i.e., not wait until RRC Connection needs to be established.

Principle c) Else, if WUS configuration is not valid or has not been acquired, UE proceeds with the cell reselection procedure as if the said NES cell was no more the best ranking cell, i.e., shall not consider the NES cell for cell re-selection; the intraFreqReselection parameter broadcasted in MIB and/ or SIB1 can be used for intra frequency (= cell X’s frequency) reselection.

## Msg1 and Msg3 based SIB1 Request

Following agreements were made in RAN2#126:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

Following the above agreements, the UE should be allowed to request for SIB1 of more than one NES Cell in the same request. Therefore, apart from Msg1 based SIB1 Request, RAN2 should also support Msg3 based SIB1 Request. This will avoid multiple hit-and-trial based SIB1 Request of one cell after the previous one has not resulted into successful Reselection and also save UE battery by avoiding so many SIB1 Request Transmissions.

Proposal 5: Msg3 based SIB1 request is supported, and UE may request for SIB1 of more than one NES Cell in the same request.

## WUS configuration in a new separated SIB or in legacy (SIB3/ 4)

Following agreement was made in RAN2#126:

Transmission of WUS configuration

5.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Further, RAN1 simulations have clearly shown that a new SIB in Cell A is bad for Cell A's energy consumption. There is no point in saving energy of one cell (NES Cell) but wasting in another. So, we think the existing neighbouring cell list broadcast in SIB3/ 4 should be used.

Table 1: RAN1 Simulation results (For the evaluation of NES loss on cell A)

For the evaluation of NES loss on cell A, the following is observed: - NES loss of 0%~0.98% from the following 15 sources with UL WUS configuration transmitted in legacy SIB on Cell A (with different assumptions on SIB periodicity and cell loading): - R1-2404408, R1-2404463, R1-2403942, R1-2403979, R1-2405341, R1-2404224, R1-2404859, R1-2404758, R1-2405162, R1-2404122, R1-2404507, R1-2404033, R1-2405363, R1-2404625, R1-2404561 (for Case 2) - One source (R1-2405106) reports for the case with UL WUS configuration transmitted in separated SIB on Cell A where the separated SIB is TDMed with legacy SIB and transmitted in a 320ms period using dedicated resource. - 0.62%~2.65% NES loss for Case 2 (Options 1+B+X) with empty load - 0.73%~3.20% NES loss for Case 3 (Options 2+B+Y) with empty load - One source (R1-2404122) reports - 2.95%(medium load)~15.49%(empty load) NES loss with UL WUS configuration transmitted in separated SIB on Cell - The separated SIB is TDMed with legacy SIB and transmitted in a 20ms period using dedicated resource. - 1.28%(medium load)~3.62%(empty load) NES loss with UL WUS configuration transmitted in legacy SIB on Cell A

Proposal 6: Reuse SIB3/ 4 to provide WUS configuration of neighboring intra and inter-frequency cells. No new SIB is introduced for providing SIB1 WUS configuration.

## Barring legacy (and non R19 NES) UEs

MIB Barring: Use of cellBarred in MIB has been deprecated based on last discussions in R18 background (discussions related to Emergency Calls). Additionally, R19 NES UEs can't really identify if a cell is R19 NES cell providing SIB1 on on-demand basis just by receiving the cellBarred bit.

So, MIB barring alone will not help R19 NES UEs. We need to use something else to indicate that: the field "ssb-SubcarrierOffset" can indicate that SIB1 is absent and further the field pdcch-ConfigSIB1 can be used to indicate that there's no other frequency positions/ range where the UE may find SS/PBCH block with SIB1 - this will help the legacy UEs immensely and these will not only know that SIB1 is not being provided "here" but also not in another frequency (range). So, unnecessary battery consumption in looking for "non-existent" SIB1 will be avoided. If RAN2 agrees, we need to request RAN1 to consider this.

Proposal 7: To bar legacy UEs, the field "ssb-SubcarrierOffset" can indicate that SIB1 is absent and further the field pdcch-ConfigSIB1 can be used to indicate that there's no other frequency positions/ range where the UE may find SS/PBCH block with SIB1.

## Indication that SIB1 is provided on On-Demand

Following the barring proposal, the immediate next question is: should a R19 NES Cell indicate that SIB1 is provided on on-demand basis? Concerning this the following agreements were made earlier:

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

We think that a R19 NES Cell should indicate that SIB1 is provided on on-demand basis since it is not sufficient to only rely on neighbouring Cell A to provide this info for following reasons

First) Relying on serving cell (which may be a Cell A or Cell X) to provide information if a neighbouring cell is a R19 NES on-demand SIB1 Cell might be non-practical given any toggling of NES status of a cell will trigger (Xn/ F1) signalling in a geography – which if not perfectly implemented in network will create issues, at least in the initial phase of deployment.

Second) We think it is important that a R19 NES Cell itself indicates that SIB1 is provided on on-demand basis, since otherwise a R19 NES UE (failing SIB1 acquisition assuming regular broadcast) will erroneously bar a R19 NES Cell in at least two scenarios:

- cell selection on NES Cell and

- if the UE is camping on Cell ‘X’ which is neither Cell A nor NES Cell and during reselection measurement procedure finds a NES Cell.

Observation 8: Relying only on Cell A for indicating every R19 on-demand NES neighbouring Cell may lead to un-necessary barring of NES Cell.

Proposal 8: A R19 NES Cell can itself indicate that SIB1 is provided on on-demand basis to the R19 NES UE.

## How will NES Cell indicate that SIB1 is provided on demand?

Can the above MIB configuration to bar legacy cell can also be used by a R19 NES UE to determine that the MIB is from a NES Cell that provides SIB1 only on on-demand basis? We think that the above combination of MIB fields ssb-SubcarrierOffset and pdcch-ConfigSIB1 is sufficient for the intended indication. If operators indicate that such a configuration may already be in use in current deployment then PCI reserving can also be used for the intended purpose i.e., to indicate that a R19 NES Cell itself indicates that SIB1 is provided on on-demand basis.

Proposal 9: Indication from a R19 NES Cell that its SIB1 is provided on on-demand basis can use the above Cell barring configuration (using existing MIB fields) and additionally, PCI reservation (e.g., PCIs 100, 200…) can be used as well, if required.

The proposals 7, 8 and 9 can be achieved without any changes/ impact to SSB, as desired in the WID. We should still let RAN1 know of RAN2’s decision.

Proposal 10: RAN2 should send LS to RAN1 informing them on our agreement from proposals 7, 8 and 9.

# Conclusion

Remaining essential issues were discussed in the paper for the on-demand SIB1 topic. Following proposals are made:

Proposal 1: For cell reselections, reuse current principles (If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ to decide if intra/ inter frequency measurements need to be performed).

Proposal 2: If the best ranked cell or a higher priority inter-frequency cell is a R19 NES Cell, then UE should go on and request SIB1.

Proposal 3: A configurable WUS validity can be included in the WUS configuration itself.

Proposal 4: RAN2 kindly discuss if the following principles suffice for R19 NES on-demand SIB1 based cell reselection:

Principle a) Stored SIB1 is considered valid for cell camping (reselection) purpose if the corresponding WUS configuration is still valid.

Principle b) After reselection, the UE shall initiate SIB1 request again from the serving NES cell i.e., not wait until RRC Connection needs to be established.

Principle c) Else, if WUS configuration is not valid or has not been acquired, UE proceeds with the cell reselection procedure as if the said NES cell was no more the best ranking cell, i.e., shall not consider the NES cell for cell re-selection; the intraFreqReselection parameter broadcasted in MIB and/ or SIB1 can be used for intra frequency (= cell X’s frequency) reselection.

Proposal 5: Msg3 based SIB1 request is supported, and UE may request for SIB1 of more than one NES Cell in the same request.

Proposal 6: Reuse SIB3/ 4 to provide WUS configuration of neighboring intra and inter-frequency cells. No new SIB is introduced for providing SIB1 WUS configuration.

Proposal 7: To bar legacy UEs, the field "ssb-SubcarrierOffset" can indicate that SIB1 is absent and further the field “pdcch-ConfigSIB1” can be used to indicate that there's no other frequency positions/ range where the UE may find SS/PBCH block with SIB1.

Observation 8: Relying only on Cell A for indicating every R19 on-demand NES neighbouring Cell may lead to un-necessary barring of NES Cell.

Proposal 8: A R19 NES Cell can itself indicate that SIB1 is provided on on-demand basis to the R19 NES UE.

Proposal 9: Indication from a R19 NES Cell that its SIB1 is provided on on-demand basis can use the above Cell barring configuration (using existing MIB fields) and additionally, PCI reservation (e.g., PCIs 100, 200…) can be used as well, if required.

Proposal 10: RAN2 should send LS to RAN1 informing them on our agreement from proposals 7, 8 and 9.

# References

[1]: RAN1 meeting minutes from RAN2#126

[2]: RAN1 meeting minutes from RAN2#117