3GPP TSG-RAN WG2#127		      					R2-2407439

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:	8.6.2

Source: 	Kyocera

Title: 	Discussion on inter-CU LTM

Document for:	Discussion

Introduction

RAN#102 approved the new work item on NR mobility enhancements Phase 4 [1]. The objectives for inter-CU LTM support are stated in the WID as follows [1].

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released - Note: The case that LTM is configured in both MCG and SCG is excluded - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In this contribution, the potential issues for supporting inter-CU LTM are discussed.

Discussion

PDCCH ordered early RACH

RAN2#125-bis agreed that the PDCCH ordered early RACH is supported also for inter-CU LTM [2].

Agreements on early sync phase: - Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. - PDCCH ordered early RACH is supported for inter-CU LTM. - For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

In Rel-18 intra-CU LTM, the CFRA without network response (i.e., PDCCH ordered early RACH) is supported for the early TA process [3]. During the early TA process, the serving cell sends the PDCCH order to the UE and the LTM candidate cell receives the PRACH from the UE. The tight coordination of the serving cell and the LTM candidate cell can be easily achieved since these cells belong to the same CU.

In case of Rel-19 inter-CU LTM, the LTM candidate cell and the serving cell belong to different CUs, so some interactions between the CUs would need to be considered. For the PDCCH ordered early RACH in inter-CU LTM, a couple of issues should be considered as follows and also depicted in Figure 1.

- Currently, the serving gNB does not know whether it needs to send the PDCCH order to the UE, since it does not know whether the LTM candidate gNB configures the UE with the PDCCH ordered early RACH.

- In addition, the serving gNB does not know the configuration of dedicated preamble which needs to be informed via PDCCH order to the UE, if the serving gNB needs to send the PDCCH order.

Figure 1 Issues of PDCCH ordered early RACH in inter-CU LTM

Regarding these issues, the following three approaches should be considered:

- Approach 1: Via Xn interface, whether the UE is configured with the PDCCH ordered early TA and the dedicated preamble configuration are informed by the LTM candidate gNB to the serving gNB.

  - The LTM candidate gNB informs the serving gNB of the necessary configuration for transmitting the PDCCH order to the UE.

  - The LTM candidate gNB needs to inform the serving gNB explicitly that it has configured the PDCCH ordered early RACH to the UE (e.g., HANDOVER REQUEST ACKNOWLEDGE message), because RAN2 assumes that the serving gNB does not comprehend the RRC container.

- Approach 2: Via Uu interface, whether the UE is configured with the PDCCH ordered early TA is informed by the UE to the serving gNB.

  - The LTM candidate gNB configures the RRC Reconfiguration including the necessary configuration (i.e., dedicated preamble) to execute the PDCCH ordered early RACH to the UE in advance. In this case, the serving gNB needs to know whether the UE has been configured with the dedicated preamble by RRC Reconfiguration, in order to determine whether it needs to indicate the dedicated preamble in PDCCH order.

  - If the dedicated preamble is configured to the UE, the serving gNB only needs to send the PDCCH order to the UE at the optimal timing, because the UE is already configured with RACH-Config Generic and RACH-Config Dedicated (i.e., the configuration of PDCCH ordered early RACH) by RRC Reconfiguration.

  - In this case, the serving gNB does not need to include Random Access Preamble index and SS/PBCH index, PRACH Mask index (i.e., the equivalent of RACH-Config Dedicated) within the PDCCH order.  Note that for Rel-18 intra-CU LTM, the PDCCH order uses the DCI format 1_0 [4] with the following dedicated preamble configuration, since the LTM candidate cell does not configure RACH-Config Dedicated for the early TA in RRC Reconfiguration (i.e., LTM Config.)

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - ⌈〖log〗_2 (C+1)⌉ bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. […]

- Approach 3: The early TA is automatically performed by the UE once it’s configured by the LTM candidate gNB, without the PDCCH order from the serving gNB (i.e., PDCCH order-less early TA).

  - As in Approach 2, the LTM candidate gNB configures RRC Reconfiguration including the necessary configuration to execute the PDCCH ordered early RACH to the UE in advance.

  - Approach 3 is different from above two approaches in that it does not need the PDCCH order. Similar to the legacy L3 Handover, upon the UE is configured with RRC Reconfiguration, the UE immediately sends Random Access Preamble to the LTM candidate gNB. In other words, RACH-Config Dedicated that the UE receives from the LTM candidate gNB works as an implicit PDCCH order.

  - Approach 3 would be useful especially in case the LTM cell switch is likely executed immediately. In addition, from the serving gNB perspective, the early TA is performed transparently (i.e., the serving gNB doesn’t need to send the PDCCH order for early TA.)

Approaches 1 and 2 assume that the serving gNB send the PDCCH order to the UE as agreed by RAN2. In our view, Approach 1 is a usual method but has impacts to RAN3 specifications. Approach 2 has the advantage of reducing the signalling overhead related to the PDCCH order.

Although Approach 3 has not been discussed in RAN2, it’s worth discussing since it does not need any inter-gNB coordination related to the PDCCH order for the early TA.

RAN2 should discuss how to assign the dedicated preamble to the UE, e.g., whether to use PDCCH order as Rel-18 LTM did (Approach 1), RRC Reconfiguration (in LTM config) as Rel-15 L3 Handover did (Approach 2) or PDCCH order-less did (Approach 3).

Inter-CU MCG LTM with SN released

RAN2#126 agreed the clarification of DC scenario for inter-CU LTM [5].

Agreements on inter-CU LTM: Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

RAN#104 approved the revised WID, which included the three DC scenarios as follows [6].

- When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

According to the WID, inter-CU LTM with DC has three scenarios. Among them “inter-CU MCG LTM with SN released” has an issue in handling SCG LTM Config. The scenario of inter-CU MCG LTM with SN released is depicted in Figure 2.

Figure 2 Inter-CU MCG LTM with SN released

According to the TS 38.331 [7] below, the UE maintains MCG and SCG LTM Configs in separate VarLTM-Configs and each LTM Config is configured from the MN and the SN respectively. i.e., it is the problem that the MN cannot delete SCG LTM Config. Our understanding is shown in Figure 3.

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE. In NR-DC, the UE may receive two independent ltm-Config: - an ltm-Config associated with the MCG that is included within an RRCReconfiguration message received via SRB1; and - an ltm-Config associated with the SCG that is included within an RRCReconfiguration message either received via SRB3, or, alternatively, embedded in an RRCReconfiguration message received via SRB1.

Based on the RAN2#126 agreement, before “inter-CU MCG LTM with SN released”, the UE is configured with two LTM Configs as follows:

- MCG LTM Config: A mixture of intra-CU cells and inter-CU cells can be configured.

- SCG LTM Config: Only intra-CU (= intra-SN) cells can be configured.

Figure 3 LTM Config stored in UE before inter-CU MCG LTM with SN released

According to the TS38.300 [3] below, for the subsequent LTM, the UE maintains LTM Configs until the NW explicitly indicates to remove it. Therefore, the UE maintains SCG LTM Config after inter-CU MCG LTM execution. Note that the SCG is released upon inter-CU MCG LTM in this scenario.

However, it’s unclear if the target MCG will use a new DC with the same SN (i.e., the source SN for the intra-CU LTM config which the UE stores), i.e., the target MCG may not use the DC or may configure the UE with a new DC with the target SN different from the source SN. Since the SCG is released in the scenario of “inter-CU MCG LTM with SN released”, the stored SCG LTM Config (for intra-SN) is no longer necessary from the viewpoint of the target MCG.  Note that keeping DC with the same SN after inter-CU MCG LTM execution is considered in “inter-CU MCG LTM with SN is unchanged” scenario.

Subsequent LTM is done by repeating the early synchronization, LTM cell switch execution, and LTM cell switch completion steps without releasing other LTM candidate configurations after each LTM cell switch completion.

According to the 38.300 [3] below, the UE should perform L1 measurement as long as LTM Config in the LTM preparation phase is applicable. Therefore, maintaining the unnecessary LTM Config increases the UE power consumption.

- The UE performs L1 measurements on the configured candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

To avoid this issue, the unnecessary SCG LTM Config should be removed upon execution of “inter-CU MCG LTM with SN released”, which can be only done by the SN according to the current specification. However, this SN is already released when inter-CU MCG LTM is executed. In the current specification, neither the UE nor the MN can remove SCG LTM Config. So, RAN2 should discuss how to remove the unnecessary SCG LTM Config, when no SN is configured to the UE.

RAN2 should discuss how to remove SCG LTM Config after executing inter-CU MCG LTM with SN released.

Conclusion

In this contribution, the potential issues for supporting inter-CU LTM are discussed.  RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should discuss how to assign the dedicated preamble to the UE, e.g., whether to use PDCCH order as Rel-18 LTM did (Approach 1), RRC Reconfiguration (in LTM config) as Rel-15 L3 Handover did (Approach 2) or PDCCH order-less did (Approach 3).

Proposal 2	RAN2 should discuss how to remove SCG LTM Config after executing inter-CU MCG LTM with SN released.

References

- RP-234036, “New WID: NR mobility enhancements Phase 4”, Intel (moderator), RAN#102

- R2-2403731 “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#125-bis

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.212, “Multiplexing and channel coding”, 3GPP

- R2-2405701 “Report from session on V2X/SL, R19 NES and MOB”, Vice Chairman (Samsung), RAN2#126

- RP-241515, “Revised Work Item: NR mobility enhancements Phase 4”, Apple Inc, China Telecom, RAN#104

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

Appendix

The representative Rel-18 LTM procedures are depicted in the following figures.

Figure 4	 Signalling procedure for LTM (Figure 9.2.3.5.2-1 in [3])