3GPP TSG-RAN WG2 Meeting #127	R2-2407441

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                    	DENSO CORPORATION

Title:  	Discussion on inter-CU LTM

Document for:        	Discussion and decision

Agenda Item:         	8.6.2	Inter-CU LTM

- Introduction

At RAN2 meeting [1][2], the following was agreed.

RAN2-125bis Agreement

Agreements on scenarios:

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

RAN2-126 Agreement

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this paper, we discussed about more detail procedure for mixture of inter-CU LTM and intra-CU LTM.

- Discussion

In RAN2-126[1], RAN2 agreed Rel-18 LTM procedure and information structure, such as RRC signalling structure and modelling, CSC MAC CE, and LTM cell switch completion procedure, are taken as the baseline for Rel-19 LTM procedure. Furthermore, in RAN2-125bis[2], RAN2 also agreed that UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Based on these agreements, if the UE is configured with a mixture of intra-CU and inter-CU candidate LTM cells, the UE can switch the cell to the intra-CU candidate LTM cell with the same procedure as Rel-18 LTM (without additional procedure) since Rel-19 LTM procedure follows Rel-18 LTM procedure. Therefore, RAN2 to consider that the UE can follow Rel-18 LTM procedure in Rel-19 intra-CU LTM cell switch procedure.

Proposal 1: The UE can follow Rel-18 LTM procedure in Rel-19 intra-CU LTM cell switch procedure.

However, Rel-19 LTM procedure needs to operate the additional procedures for inter-CU LTM procedure in addition to the intra-CU LTM procedure. For example, the UE needs to perform the PDCP re-establishment and the security key update upon inter-CU LTM execution, in addition to MAC reset and RLC re-establishment which operated in intra-CU LTM procedure, as RAN2 agreed in RAN2-125[3].

Observation 1: Rel-19 LTM requires the additional procedures for inter-CU LTM procedure in addition to the intra-CU LTM procedure.

Therefore, in Rel-19, if the UE is configured with only intra-CU LTM candidate cells, the UE can switch the cell by following Rel-18 LTM procedure. And if the UE is configured with only inter-CU LTM candidate cells, the UE needs to operate additional procedure in addition to Rel-18 LTM procedure. Then, if the UE is configured with a mixture of intra-CU and inter-CU candidate LTM cells, the UE needs to distinguish whether the current LTM cell switch operation is for the intra-CU LTM candidate cell or inter-CU LTM candidate cell, (i.e., whether the UE needs to operate additional procedures). However, the UE cannot distinguish the differences between intra-CU LTM candidate cell configuration and inter-CU LTM candidate cell configuration without additional information, as discussed in Rel-18 SCPAC operation. In SCPAC operation, the additional information (i.e., securityCellSetId-r18) is included in RRC signalling for the UE to distinguish whether current operation is intra-gNB CPAC or inter-gNB CPAC. This solution is also effective for the Rel-19 LTM, therefore, in Rel-19 LTM, the UE can distinguish whether the current operation is intra-CU LTM or inter-CU LTM by using the additional information included in RRC signalling.

Proposal 2: In Rel-19 LTM, RAN2 to include the additional information in RRC signalling for the UE to distinguish whether the current operation is intra-CU LTM or inter-CU LTM.

Furthermore, RAN2 agreed that Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. However, for the mixture of subsequent intra-CU LTM after inter-CU LTM switch, it has an issue that the UE cannot update the security key upon LTM execution. For example, after inter-CU LTM switch, the CU of the source cell may be different from the CU of the cell configured as the intra-CU LTM candidate cell in initial RRC Reconfiguration. However, since intra-CU LTM candidate cell configuration does not include the security update information, the UE cannot update the security key and cannot switch to such cells after inter-CU LTM switch. One simple solution is that the additional information (for example, NCC value) could be included in both of inter-CU and intra-CU LTM candidate cell configuration.

Proposal 3: For supporting mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch, RAN2 to consider that the additional information (for example, NCC value) could be included in both of inter-CU and intra-CU LTM candidate cell configuration.

- Summary and proposal

This paper discussed the procedure for mixture of inter-CU LTM and intra-CU LTM. In summary, the followings were proposed:

Observation 1: Rel-19 LTM requires the additional procedures for inter-CU LTM procedure in addition to the intra-CU LTM procedure.

Proposal 1: The UE can follow Rel-18 LTM procedure in Rel-19 intra-CU LTM cell switch procedure.

Proposal 2: In Rel-19 LTM, RAN2 to include the additional information in RRC signalling for the UE to distinguish whether the current operation is intra-CU LTM or inter-CU LTM.

Proposal 3: For supporting mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch, RAN2 to consider that the additional information (for example, NCC value) could be included in both of inter-CU and intra-CU LTM candidate cell configuration.

- References

[1] R2-2406202, “RAN2#126 Meeting Report,” ETSI MCC.

[2] R2-2404102, “RAN2#125bis Meeting Report,” ETSI MCC.

[3] R2-2402102, “RAN2#125 Meeting Report,” ETSI MCC.