3GPP TSG RAN WG2 #127		R2-2407451

Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda Item:	8.3.2

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Simulation results for RRM measurement prediction

Document for:	Discussion

- Introduction

At the RAN 102 meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” was approved. The primary objectives the SID are as follows,

- RRM measurement prediction

- Measurement event predictions

- RLF HO failure prediction

At the RAN2 #125 bis meeting it was agreed to study the following cases for RRM measurement prediction

      - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results

      - Case 2: To directly predict cell level results based on cell level results

      - Case 3: To directly predict cell level results based on beam level results

Further, it was agreed in RAN2 #125 bis [2] and RAN2 #126 [3] to study the implementation of above cases in spatial domain, temporal domain and frequency domains. In continuation to the agreements in the meetings, several simulation assumptions were also agreed for the development of AI/ML models for RRM predictions. This study needs to be extended for the event as well as RLF/HO failure predictions.

Based on above-mentioned agreements and simulation assumptions, this document focuses on the development of AI/ML models for RRM prediction - specifically for FR2 intra frequency intra cell prediction in the spatial domain.

- Generation of data set

This section details the process for generating mobile UE trajectories for RRM prediction as well as the simulation assumptions used in data generation for training/testing ML methods.

The following are the characteristics of the data used in our simulations:

            - UEs move on random tracks in a two-tier layout as shown in Figure 1.

            - We define a track length, which is divided into segments. The segments are generated based on

minimum length of 20, mean length of 40, standard deviation (of length) of 10, curve radius of 20 and turn probability of 0.7

            - Channel measurements are captured at regular snapshots based on the UE speed an a CSI-RS periodicity of 5ms

            - If the UE encounters the boundary it’s track is terminated and a new UE is dropped at random position.

            - At each snapshot, we measure the RSRP values [4] at the UE’s position from the serving cell and all neighbouring cells. These RSRPs form the model input.

Figure 1: Simulated hexagonal 2-tier layout

Table 1: Simulation assumptions

| Parameter | Value |  |
| --- | --- | --- |
| SCS |  | 120 kHz |
| BW |  | 80 MHz |
| ISD |  | 200 m |
| Center frequency |  | 30 GHz |
| Scenario |  | UMi |
| Layout |  | 2 tier hexagonal cell |
| Number of Tx (N_Tx) |  | 32 |
| Tx antenna height |  | 10 m |
| Number of Rx (N_Rx) |  | 4 |
| Rx antenna height |  | 1.5 m |
| UE track length |  | 120 m |
| BS Tx power |  | 30 dBm for 20 RB bandwidth |
| UE speed |  | 120 kmph |
| UE rotation |  | No |
| Number of resource elements |  | 51 RBs*12 |

Other parameters are as per the outcome of the email discussion [5]

- Model Assumptions:

We used an LSTM (Long Short-Term Memory) based architecture for RRM measurement prediction. Details are given below.

Table 2: Model architecture and other experimental parameters

| Assumptions | Number of beams in set A | 32 * 4 |
| --- | --- | --- |
|  | Number of beams in set B | With 1/2, 1/4, 1/8 and 1/16 reduction of set A |
| AI/ML model | Model input | Set B |
|  | Model output | Set A |
|  | Model Description | LSTM models with different hidden sizes |
| Data size | Training | 12000 samples |
|  | Testing | 3000 samples |

LSTM model

Model input

Measurements of Set A

Model output

Predicted RSRP values for beams in Set A

Figure 2: AI/ML model implementation for RRM measurement prediction for FR2 intra cell intra frequency spatial domain.

- RRM prediction Results for FR2 Intra F_C_S:

Evaluation of model performance for different model complexities

Once the RSRP are predicted using the model in Figure 2, the difference between the predicted RSRP and the true RSRP is calculated. Then, the differences are binned as within 1 dB, 2 dB, … 6 dB ranges. Table 3 shows the % of predictions within the 1 dB, 2 dB, … 6 dB ranges. The results are shown for different values of LSTM hidden size. Table 4 illustrates the number of parameters of the models for each hidden size experimented.

Table 3: RRM prediction vs LSTM model complexity  for reduction rate of 1/4

| Hidden size | diff_1db | diff_2db | diff_3db | diff_4db | diff_5db |
| --- | --- | --- | --- | --- | --- |
| 16 | 79.01 | 92.91 | 99.2 | 99.87 | 100 |
| 32 | 82.26 | 94.04 | 99.23 | 99.89 | 100 |
| 64 | 85.26 | 96.04 | 99.23 | 99.89 | 100 |
| 128 | 85.12 | 95.94 | 99.26 | 99.87 | 100 |
| 256 | 85.12 | 96.01 | 99.28 | 99.89 | 100 |

Table 4: LSTM hidden and complexity in terms of number of parameters

| LSTM hidden size | Number of parameters |
| --- | --- |
| 16 | 5792 |
| 32 | 13600 |
| 64 | 35360 |
| 128 | 103456 |
| 256 | 337952 |

Observation 1: In the FR2 intra frequency intra cell spatial domain, a single AI/ML model with RSRP data captured from all cells, generalizes and performs well across all cells.

Observation 2: In the FR2 intra frequency intra cell spatial domain, using AI/ML, 99% of the RRM predictions are within 3dB of the true values.

Reduction rates:

In another experiment we fix the LSTM hidden size and train the model with different reduction rates. The reduction rate indicates the reduction in the number of beams to be measured in relation to the total number of beams to be measured. From Table 5, it can be observed that for reduction rates up to ¼ there is no change in the prediction accuracy. After that the accuracy decreases with the increase in reduction rate for 1 dB difference. However, there is an insignificant drop in the prediction accuracy for the 3 dB difference and beyond even the reduction rate is changed.

Observation 3: In the FR2 intra frequency intra cell spatial domain, with 1/4 reduction (Set B / Set A), there is little to no difference in the accuracy of the model compared to 1/2 reduction.

Observation 4: In the FR2 intra frequency intra cell spatial domain, with reduction rates (Set B/Set A) greater than 1/4, there is a drop in predictions correct to within 2 dB compared to that of 1/4 but there is no change in predictions correct to within 3dB and above.

Table 5: RRM prediction vs reduction rates for LSTM of hidden size 64

| Reduction rate (MRRS) | diff_1db | diff_2db | diff_3db | diff_4db | diff_5db | diff_6db |
| --- | --- | --- | --- | --- | --- | --- |
| 1/2 | 85.07 | 95.99 | 99.23 | 99.89 | 100 | 100 |
| 1/4 | 85.26 | 96.04 | 99.23 | 99.89 | 100 | 100 |
| 1/8 | 80.37 | 94.66 | 99.02 | 99.89 | 99.97 | 100 |
| 1/16 | 79.38 | 94.82 | 98.8 | 99.87 | 99.97 | 99.97 |

- Conclusion

This contribution focused on the development of AI/ML models for RRM prediction - specifically for FR2 intra frequency intra cell prediction in the spatial domain. We make the following observations.

Observation 1: In the FR2 intra frequency intra cell spatial domain, a single AI/ML model with RSRP data captured from all cells, generalizes and performs well across all cells.

Observation 2: In the FR2 intra frequency intra cell spatial domain, using AI/ML, 99% of the RRM predictions are within 3dB of the true values.

Observation 3: In the FR2 intra frequency intra cell spatial domain, with 1/4 reduction (Set B / Set A), there is little to no difference in the accuracy of the model compared to 1/2 reduction.

Observation 4: In the FR2 intra frequency intra cell spatial domain, with reduction rates (Set B/Set A) greater than 1/4, there is a drop in predictions correct to within 2 dB compared to that of 1/4 but there is no change in predictions correct to within 3dB and above.

- References

[1] RP-234055 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR, FS_NR_AIML_Mob

[2] Chair’s Notes, RAN2 #125 bis, April 2024

[3] Chair’s Notes, RAN2 #126, May 2024

[4] 3GPP 38.215

[5] RAN2 #126 Post Email Discussion: R2-2405960 Draft summary of [AT126][030][AIMob] Simulation assumptions