3GPP TSG RAN WG2 Meeting #127	R2- 2407452

Maastricht, Netherlands, 19th - 23th Aug. 2024

Agenda Item:	8.8.5

Source:	ETRI

Title:		Discussion on time-based measurement initiation for regenerative payload

Document for:	Discussion and Decision

1	Introduction

With the transparent payload in Rel-18, feeder link switch results in a change of the serving gNB. On the other hand, with the regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch. Focused on this observation, we discuss impacts of the regenerative payload on the time-based measurement initiation for the feeder link switch.

2	Discussion

2.1	Background of Rel-18 time-based measurement initiation for feeder link switch

Figure 1. Example feeder link switch for a satellite with a transparent payload in an Earth moving cell

As illustrated in Figure 1, the serving gNB is always changed after feeder link switch for a satellite with a transparent payload.

With the transparent payload in Rel-18, feeder link switch always results in a change of the serving gNB.

In Rel-18, to address feeder-link switch in NTN Earth moving system as well as quasi-Earth fixed system, time-based measurement initiation is used with the reused t-Service. According to Rel-18 spec., a UE in either an Earth moving cell or a quasi-Earth fixed cell shall perform intra-frequency, inter-frequency, or inter-RAT measurements before t-Service which is the time information on when the cell is going to stop serving the UE due to feeder link switch.

2.2	Impacts of the regenerative payload on Rel-18 time-based measurement initiation for feeder link switch

Figure 2. Conceptual diagram of example feeder link switch for a satellite with a regenerative payload in an Earth-moving cell

In Rel-19, satellites with regenerative payload are considered. The regenerative payload is a full-function gNB(s). Consequently, as illustrated in Figure 2, the serving gNB is not changed after the feeder link switch.

With the regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch

This observation clearly indicates that the Rel-18 time-based measurement initiation for feeder link switch is not needed for the case of Rel-19 regenerative payload. The motivation for the Rel-18 time-based measurement is for a UE to have enough time for finding the best cell to camp on before feeder link switch. This is because the serving cell abruptly stops serving the area covered by it after the switch. However, with Rel-19 regenerative payload, the serving cell keep serving the area after feeder link switch.

With the regenerative payload in Rel-19, time-based measurement initiation for feeder link switch is not necessary.

Based on above observations, we propose the following:

Time-based measurement initiation for feeder link switch shall not be configured for regenerative payload.

3	Conclusion

In this paper, the following observations are made:

Observation 1	With the transparent payload in Rel-18, feeder link switch always results in a change of the serving gNB.

Observation 2	With the regenerative payload in Rel-19, the serving gNB is not changed after the feeder link switch

Observation 3	With the regenerative payload in Rel-19, time-based measurement initiation for feeder link switch is not necessary.

In this paper, we propose the following:

Proposal 1	Time-based measurement initiation for feeder link switch shall not be configured for regenerative payload.