3GPP TSG-RAN WG2 #127	R2-2407454

Maastricht, Netherland, 19th – 23rd Aug, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on Adaptation of paging occasions

Document for:	Discussion

Agenda Item:	8.5.4

- Introduction

A new WID on NES (Network Energy Saving) has been approved at the RAN#102 meeting and revised at the RAN#104[1]. In the WID, following objectives are included.

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we describe our view for Adaptation of paging occasions.

- Discussion

The following agreement was made in RAN2 #125bis meeting

RAN2 #125bis Agreement - For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

When excluding legacy UEs in paging, it will have an impact on the coverage of legacy UEs, although it will also depend on the cell design. Therefore, as stated in the WID objective, at the very least, a method that does not bar legacy UEs is necessary in order to reduce the negative impact on legacy UEs.

Proposal 1: 	At least, a mechanism is needed that allows legacy UEs and NES UEs to coexist without barring legacy UEs.

The following agreement was made in RAN2 #125bis meeting

RAN2 #125bis Agreement - For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

For adaptation of paging occasion, two solutions have been identified.

Both solutions can increase the number of opportunities to put gNB to sleep. In addition, when coexisting with Legacy UE, we assume that there will be no significant difference between these two solutions.

In this case, we prefer option b) extend the values of N as a simple solution.

Proposal 2:  	For the adaptation of paging occasions, extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF as a baseline.

- Conlusion

Proposal 1: 	At least, a mechanism is needed that allows legacy UEs and NES UEs to coexist without barring legacy UEs.

Proposal 2:  	For the adaptation of paging occasions, extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF as a baseline.

- References

[1] RP-241650, Revised WID: Enhancements of network energy savings for NR