3GPP TSG-RAN WG2 #127	R2-2407455

Maastricht, Netherland, 19th – 23rd Aug, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on on-demand SIB1

Document for:	Discussion

Agenda Item:	8.5.3

- Introduction

A new WID on NES (Network Energy Saving) has been approved at the RAN#102 meeting and revised at the RAN#104[1]. In the WID, following objectives are included.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

In this contribution, we describe our view for on-demand SIB1.

- Discussion

2.1 Scenario clarifications

The following agreements were reached in RAN1 #116-bis and RAN1#117meeting.

RAN1 #116bis Agreement - On-demand SIB1 scenarios - Agreed the following cases for further study - Case 1: Option 1+A+X - Case 2: Option 1+B+X - Case 3: Option 2+B+Y - Where the options 1/2/A/B/X/Y are defined below: - On target cell of UL WUS transmission: - Option 1: UE transmits UL WUS to NES Cell - Option 2: UE transmits UL WUS to Cell A - On configuration provision for UL WUS transmission - Option A: UE obtains the UL WUS configuration from NES Cell - Option B: UE obtains the UL WUS configuration from Cell A - On receiving of SIB1 - Option X: UE receives on-demand SIB1 from NES Cell - Option Y: UE receives on-demand SIB1 from Cell A

RAN1 #117 Agreement For SIB1 in idle/inactive mode, prioritize RAN1 discussions on Case 2 and Case 3 - Case 2 (Option 1+B+X) is feasible from RAN1 perspective. - Further study Case 3, focusing on the additional NES benefits over Case 2, feasibility, complexity, and spec impact.

And following agreements were made in RAN2 #125bis and RAN2 #126 meeting, about Case 2 (i.e. scenario 1a and 2 in RAN2) and Case 1 (i.e. scenario 3 in RAN2).

RAN2 #125bis Agreement - At least RAN2 starts scenario 1a (Cell A SIB assisted intra-cell WUS. And WUS and SIB1 is sent to/from NES cell). Other scenarios are not excluded.

RAN2#126 Agreement - RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3. - RAN2 to wait for RAN1’s progress whether to support scenario 3.

As Case 1 is the most flexible NES cell deployment in commercial environment, there were opinions that many companies would need it in RAN2. However, in RAN1, because some issues were identified with the WUS configuration provisioning, and no efficient solution was found for this. Therefore, RAN1 also agreed that Case1 should be deprioritized.At present, there is no prospect of being able to confirm that Case 1 is feasible, and RAN2 should not proceed its discussions unless there is progress in RAN1.

Proposal 1: 	RAN2 should continue to deprioritize Case 1 until RAN1 makes progress.

Case 3 is a scenario where it is possible to acquire more NES gain than Case 2, which has already started to be discussed in RAN2.

On the other hand, there is concern about the increasing signalling overhead on the Xn and F1 interfaces, due to the need for close coordination between the NES cell and Cell A to provide the SIB1 of the NES cell from Cell A. However, if a centralised deployment of gNBs such as vRAN is considered, the impact of this signalling overhead can be expected to be reduced.

Observation 1: 	The impact of signalling overheads in Case 3 depends on the deployment.

While studies will continue in RAN1, other points exist to be discussed in RAN2, such as how to deliver SIB1. Therefore, there should be continued discussions on the support of Case 3 in RAN2.

Proposal 2: 	RAN2 should support case 3 and start to study it.

2.2 Barring for legacy UE

Following agreements were made in RAN2 #126 meeting.

RAN2#126 Agreement - NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

The following two options are presented.

  - Option1 Using MIB barred

  - Option2 Using ssbSubcarrierOffset

Option 1, like R18 NES barring, prohibits all UEs using cellbarred in the MIB, and allows NES UEs using the newly introduced barring bit in SIB1. It is explicit and clear, and has the advantage of being able to be aligned with the R18 NES approach. On the other hand, for R18 NES UEs (cellDTX/DRX capable UEs), it is a disadvantage that it attempts to deliver SIBs in OD-SIB1.

In option 2, CORESET#0 set in the MIB is not configured to indicate that this cell does not provide SIB1 to the legacy UE. It has the advantage of not affecting legacy UE (including R18 NES UE). On the other hand, it is necessary to check the RAN1 impact because it affects the use of SSB.

We think, at least, when using option 2, it is necessary to check with RAN1, but the potential impact for further enhancement related to SSB when using On-demand SIB cannot be avoided. On the other hand, we think that being able to design R18 NES and barring in the same way will improve readability and is an advantage. For this reason, we prefer Option 1.

Proposal 3: 	To bar for legacy UE, Use cellbarred in MIB to bar legacy UE, and one new bit is introduced in SIB1 to allow OD-SIB1 capable UE.

- Conlusion

Proposal 1: 	RAN2 should continue to deprioritize Case 1 until RAN1 makes progress.

Observation 1: 	The impact of signalling overheads in Case 3 depends on the deployment.

Proposal 2: 	RAN2 should support case 3 and start to study it.

Proposal 2: 	To bar for legacy UE, Use cellbarred in MIB to bar legacy UE, and one new bit is introduced in SIB1 to allow OD-SIB1 capable UE.

- References

[1] RP-241650, Revised WID: Enhancements of network energy savings for NR