3GPP TSG-RAN WG2 Meeting #127												R2-2407456

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.1.2.3

Source: 	Sharp

Title:  	Discussion on Dynamic Capability Report for Positioning

Document for: 	Discussion

Introduction

In RAN#102 meeting, the Rel-19 Work Item on artificial intelligence/machine learning for NR air interface was approved [1], which includes the following objective corresponding to the positioning:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

During the life cycle management for AI/ML positioning, capability report and assistance information report are critical issues. In this contribution, how to perform capability report will be discussed.

Discussion:

Capability Report

In RAN2#126 meeting, the following agreements are made:

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

For NR, in Rel-16, several positioning methods, including UE-based and UE-assisted approaches, were developed and specified. In Rel-17 and Rel-18, these NR positioning methods were further enhanced to improve accuracy. Currently, AI/ML technologies are being applied to NR positioning to enhance accuracy and reliability, especially in NLOS environments. Similar to the enhancements in Rel-17 and Rel-18, existing NR positioning methods are being further developed and refined, rather than introducing new methods, to increase accuracy. AI/ML technologies can be used to enhance existing NR positioning methods such as DL-TDOA, DL-AoD, UL AoA, and Multi-RTT.

AI/ML technologies are used to enhance the existing NR positioning methods instead of introducing new positioning methods.

In accordance with the agreement reached in RAN2#126, the LPP Capability Transfer procedures are employed to indicate the supported AI/ML positioning capabilities. Given that AI/ML technologies are utilized to enhance each existing NR positioning method, the UE can report its support for the respective AI/ML capabilities and functionalities within each NR positioning method capabilities message (e.g., nr-DL-TDOA-ProvideCapabilities).

Supported AI/ML positioning capabilities are indicated in each NR positioning method capabilities message

Dynamic capability report

Currently, UE-supported AI/ML capabilities and functionalities can be reported via LPP Capability Transfer procedures (i.e., RequestCapability/ProvideCapabilities messages). However, the capabilities and functionalities reported by the UE to the LMF may not always be applicable due to factors such as power status, memory status, overheating, and so on. Consequently, the UE-supported AI/ML capabilities and functionalities need to be dynamically adjusted based on changes in the environment or device status.

Given that UE-supported AI/ML capabilities and functionalities may become inapplicable due to changes in the environment or device status, it is necessary to report these variations to the LMF. This assists the LMF in determining whether to continue AI/ML operations for positioning. Both environmental variations and device status changes serve as assistance information that can be provided to the LMF for informed decision-making. For the delivery of assistance information, since the LPP already includes procedures related to Assistance Data delivery (i.e., RequestAssistanceData/ProvidedAssistanceData), the relevant assistance information for AI/ML can be provided to the LMF via these existing procedures.

Reuse the existing procedure (e.g., RequestAssistanceData/ProvidedAssistanceData) for providing AI/ML-related assistance information to the LMF

In addition, since the UE-supported AI/ML capabilities and functionalities may become inapplicable due to changes in the environment and device status, the corresponding assistance information can be provided to the LMF via existing procedures (e.g., RequestAssistanceData/ProvidedAssistanceData). These existing procedures can be utilized to dynamically indicate the UE-supported AI/ML capabilities and functionalities.

The LPP Assistance Data Transfer procedures (RequestAssistanceData/ProvidedAssistanceData) are used for dynamic capability report

Conclusions

Based on the discussion given above, we have the following proposals:

- AI/ML technologies are used to enhance the existing NR positioning methods instead of introducing new positioning methods.

Supported AI/ML positioning capabilities are indicated in each NR positioning method capabilities message

Reuse the existing procedure (e.g., RequestAssistanceData/ProvidedAssistanceData) for providing AI/ML-related assistance information to the LMF

The LPP Assistance Data Transfer procedures (RequestAssistanceData/ProvidedAssistanceData) are used for dynamic capability report

Reference

[1] RP-240774, " Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface ".