3GPP TSG-RAN WG2 Meeting #127	R2-2407458

Masstricht, Netherlands, Aug. 19th – 23rd, 2024

Agenda item:		8.2.4

Source:		Samsung

Title:			Further discussion on Ambient IoT random access

Document for:	Discussion & Decision

# 1 Introduction

The agreements that have been made related to A-IoT random access in the last RAN2 meeting is as follows.

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is. “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Based on the agreements, in this contribution, we discuss remaining issues on A-IoT random access.

# 2 Discussion

RAN2 agreed that slotted-ALOHA is the baseline for A-IoT random access. For contention-based access procedure, a device is supposed to randomly select an access occasion for A-IoT Msg1 transmission. To this end, the reader should provide the total number of access occasions to the devices. Note that access occasion could be time resource and/or frequency resource so we need to wait RAN1 progress for the detailed configuration for for access occasion and its total number.

Proposal 1: For contention-based access procedure, the reader provides the total number of access occasions to the devices, from which each device randomly selects one access occasion for A-IoT Msg1 transmission. FFS on detailed configuration.

In order to transmit A-IoT Msg1 at the randomly selected access occasion, each device needs to know its starting point. The following two options can be considered:

- Asynchronous method: explict R2D signalling for each access occasion, indicating its starting point

- Synchronous method: access occasion with fixed length, whose starting point is determined autonomously by device

Due to quite large sampling frequency offset (SFO), we think that synchronous method does not work properly.

Proposal 2: For contention-based access procedure, the reader explicitly indicates the starting point of each access occasion to the devices by R2D signalling for A-IoT Msg1 transmission.

In 3-step contention-based random access procedure, RAN2 agreed that A-IoT Msg2 is used for contention resolution purpose. The key assumption here was that the size of an ID in A-IoT Msg1 should be sufficient. As analyzed in [1], if the size of random ID is 16 bit as in RFID, the probability that more than one device selecting the same random ID at the same random access occasion would be close to zero with practical number of devices and sufficient total number of random access occasions.

Proposal 3: For 3-step contention-based random access procedure, the size of an ID in A-IoT Msg1 is 16-bit.

RAN2 agreed that the device itself generates an random ID in A-IoT Msg1, but one remaining issue is how to generate it. One may consider to generate a random ID based on A-IoT device ID i.e. via a hash function that takes an input of A-IoT device ID and transforms it into a fixed-length output of random ID in A-IoT Msg1. However, to make contention with quite marginal probability, such approach may further require secure algorithms, which doe not seem reasonable for ultra-low complexity A-IoT devices with limited or no energy storage. The simplest but efficient way is to randomly generate 16-bit ID, which is independent of device ID.

Proposal 4: For 3-step contention-based random access procedure, the device randomly generates 16-bit ID in A-IoT Msg1, which is independent of device ID.

After transmitting A-IoT Msg2, the reader may not receive A-IoT Msg3 from the intended device. In RAN1#117 meeting, RAN1 made the following agreement.

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

Based on the RAN1 agreement above, we understand that the reader can decide A-IoT Msg3 reception failure after a certain time duration. When the reader detects A-IoT Msg3 reception failure we think that it should have a mechanism that instructs the device to retransmit A-IoT Msg3 as new transmission; otherwise, it has no option but to trigger new random access procedure. Hence, we propose that the device retransmits A-IoT Msg3 as new transmission if A-IoT Msg2 including the same ID in A-IoT Msg1 is received again.

Proposal 5: The device retransmits A-IoT Msg3 as new transmission if A-IoT Msg2 including the same ID in A-IoT Msg1 is received again.

If the reader receives A-IoT Msg3 successfully from the device, we need to discuss whether the subsequent R2D transmission is always sent in all use cases. For inventory + command use case (or command only use case if supported), the command message is directly sent to the device. Note that it could be used as an implicit indicator of successful transmission of A-IoT Msg3 to the device. For inventory only use case, we think that the corresponding R2D transmission needs to be sent to the device to indicate the successful transmission of A-IoT Msg3. Otherwise, the device has no idea whether the reader decided to abort the on-going 3-step contention-based random access procedure or received the A-IoT Msg3 successfully. Hence we think that the corresponding R2D transmission following A-IoT Msg3 is always needed.

Proposal 6: The device considers A-IoT Msg3 transmission as successful if the subsequent R2D transmission to this device is received. Subsequent R2D transmission is either for sending the command or for indicating the successful transmission of A-IoT Msg3.

In 2-step contention-based random access procedure, one remaining issue is whether an additional random ID in A-IoT Msg1 is needed. Considering RAN2 assumption that the size of random ID in A-IoT Msg1 in 3-step contention-based access procedure serves contention resolution properly, we do not see any compelling reason not to just reuse it.

Proposal 7: For 2-step contention-based random access procedure, the device sends an additional randomly generated 16-bit ID in A-IoT Msg1, which is independent of device ID.

Similar to 3-step contention-based random access procedure, A-IoT Msg2 is needed for contention resolution purpose. But we think that it is not always sent to the device i.e. the reader directly sends the command message to the device.

Proposal 8: For 2-step contention-based random access procedure, the device considers the contention resolution as successful if the A-IoT Msg2 including the same random ID in A-IOT Msg1 is received or the command to this device is received.

# 3	Conclusion

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: For contention-based access procedure, the reader provides the total number of access occasions to the devices, from which each device randomly selects one access occasion for A-IoT Msg1 transmission. FFS on detailed configuration.

Proposal 2: For contention-based access procedure, the reader explicitly indicates the starting point of each access occasion to the devices by R2D signalling for A-IoT Msg1 transmission.

Proposal 3: For 3-step contention-based random access procedure, the size of an ID in A-IoT Msg1 is 16-bit.

Proposal 4: For 3-step contention-based random access procedure, the device randomly generates 16-bit ID in A-IoT Msg1, which is independent of device ID.

Proposal 5: The device retransmits A-IoT Msg3 as new transmission if A-IoT Msg2 including the same ID in A-IoT Msg1 is received again.

Proposal 6: The device considers A-IoT Msg3 transmission as successful if the subsequent R2D transmission to this device is received. Subsequent R2D transmission is either for sending the command or for indicating the successful transmission of A-IoT Msg3.

Proposal 7: For 2-step contention-based random access procedure, the device sends an additional randomly generated 16-bit ID in A-IoT Msg1, which is independent of device ID.

Proposal 8: For 2-step contention-based random access procedure, the device considers the contention resolution as successful if the A-IoT Msg2 including the same random ID in A-IOT Msg1 is received or the command to this device is received.

# 4	Reference

[1] R2-2405950	Report of [AT126][022][AIoT] CB on 4 step RA, Huawei