3GPP TSG-RAN WG2 Meeting #127	R2- 2407459

Maastrict, Netherlands, 19th – 23rd Aug, 2024

Title	Clarification on configuring RACH partition in RedCap-specific initial BWP for Msg1-based SI request

Source	LG Electronics

Document for	Discussion and Decision

Agenda Item	6.1.3.1

1 	Introduction

In RAN2#126 meeting[1], one issue regarding whether/how to configure legacy RACH partition should be always configured in order to support Msg1-based SI request in RedCap-specific initial BWP. Based on the discussion, due to NBC issue, it is not agreed to modify the UE procedure to select the RedCap partition if there is no legacy RACH partition configured in the Redcap-specific initial BWP.

Confirm for SI-request without Msg1 repetition in RedCap-specific initial BWP, the UE selects the RACH-ConfigCommon that not associated with any feature (no change to R17 MAC spec). FFS if possible and if spec change needed to allow that, for a RedCap specific initial uplink BWP, whether the same CB preambles in “RA partition not associated with any feature” can be configured in another RA partition sharing the ROs. (can be discussed in next meeting)

Then, based on the understanding that the network needs to configure legacy RACH partition in RedCap-specific initial BWP, we provide our views to clarify how to configure the legacy RACH partition on RedCap-specific initial BWP for Msg1-based SI request case.

2	Discussion

In RAN2#126 meeting, based on discussion based [2] and [3], it was discussed and concluded that the R17 RedCap UE should select the set of Random Access resources which is not associated with any feature (hereinafter, referred to as legacy RACH partition), for Msg1-based SI request in RedCap-specific initial BWP, as in the current MAC specification [4]:

Confirm for SI-request without Msg1 repetition in RedCap-specific initial BWP, the UE selects the RACH-ConfigCommon that not associated with any feature (no change to R17 MAC spec).

In other words, if the Msg1-based SI request is supported in RedCap-specific initial BWP, the network should always configure the legacy RACH partition for RedCap-specific initial BWP, in order to allow the initialization of RA parameters from corresponding RACH-ConfigCommon.

Observation 1. In order to support the Msg1-based SI request in RedCap-specific initial BWP, the network should always configure the RACH partition which is not associated with any feature in RedCap-specific initial BWP.

Meanwhile, according to current RRC spec for RACH partitioning framework [5], legacy RACH partition is defined as CBRA preambles which can be used by Rel-16 UEs, i.e., contention-based preamble range which is not associated with FeautreCombinationPreambles IE. The RA preambles for legacy RACH partition is configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB within RACH-ConfigCommon IE, as depicted in Figure 1. Note that CFRA preambles are not defined as ‘legacy RACH partition’ or ‘feature specific RACH partition,’ since the UE selects different RACH partition for RA initialization for different cases, if the UE performs CFRA using CFRA preambles.

Figure 1. An example of RACH partition configuration

Observation 2. RACH partition which is not associated with any feature is defined as CBRA preambles not associated with FeatureCombinationPreambles IE, which can be configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

On the other hand, in RedCap-specific initial BWP, there are some cases when the legacy RACH partition (i.e., RA preambles which is not associated with any feature) will never be used by (e)RedCap UEs.

For the RACH partition configuration in Redcap-specific initial BWP, the network should configure at [RedCap] partition, if there is any other feature configured in the RedCap-specific initial BWP [5]:

| Conditional Presence | Explanation |
| --- | --- |
| AdditionalRACH | The field is mandatory present if the RACH-ConfigCommon is included in an AdditionalRACH-Config. When included in initialUplinkBWP-RedCap to indicate other feature(s) than redcap and eRedCap, this field is mandatory present with at least FeatureCombinationPreambles list entries: the list entry/entries indicating only redcap or eRedCap and the other(s) indicating both redcap or eRedCap and one or multiple other feature(s) (e.g., smallData, nsag or msg3-Repetitions). Otherwise, it is optional, Need R. |

For example, if RA-SDT is supported on RedCap-specific initial BWP, [RedCap] partition and [RedCap+SDT] partition should be configured, based on the current network restriction. In this case, if Msg1-based SI request is supported, the legacy RACH partition should be additionally configured as followings:

- RACH partition 1: [RedCap]

- RACH partition 2: [RedCap+Msg1 repetition]

- RACH partition 3: [legacy RACH partition]

Then, given that Redcap-specific initial BWP is used by RedCap UE or eRedCap UEs only, if a UE in RedCap-specific initial BWP performs the CBRA procedure, only the Partition 1: [RedCap] and Partition 2: [RedCap+SDT] would be used. In other words, the legacy RACH partition (i.e., Partition 3) would never be used on CBRA purposes.

Observation 3. In some cases, the CBRA preambles for RACH partition which is not associated with any feature is never used by UEs in RedCap-specific initial BWP.

Based on observation 1, in order to configure legacy RACH partition in RedCap-specific initial BWP, following options are possible:

- Option 1: Reserve the separated Random Access preambles for legacy RACH partition, which is not used by any other feature in all cases.

- Option 2: Configure the Random Access preambles for legacy RACH partition, which can be duplicated with the RACH partition associated with any feature(s), if the legacy RACH partition is never be used by CBRA purposes.

For Option 1, it is straightforward to configure the legacy RACH partition, and it is already supported with no issues. However, as explained in Observation 3, in some cases, the configured RA preambles would not be used for any CBRA procedure, which causes a resource waste.

For Option 2, for the case that legacy RACH partition is not used for CBRA purposes, it is possible that the legacy RACH partition can be configured using the same preamble and same RO with RACH partition for a feature (e.g., RedCap partition). An example of configuration for legacy RACH partitions and other RACH partitions in RACH-ConfigCommon IE is shown in Figure 2.

Figure 2. an example of configuration for Option 2.

Then, if the legacy RACH partition is not used by CBRA purpose as in the aforementioned example, there would be no issue on the RA procedure from the UE perspective:

- If CBRA is initiated, the feature-specific partition (e.g., [RedCap] partition or [RedCap+SDT] partition) is selected, and the RA parameter is initialized based on the selected partition.

- If RA procedure for Msg1-based SI request is initiated, legacy RACH partition is selected, and the RA parameter is initialized based on the legacy RACH partition.

Therefore, in order to avoid the resource waste for the case explained in observation 3, Option 2 should also be allowed, i.e., configuring legacy RACH partition duplicated with the RACH partition associated with any feature(s), if the legacy RACH partition is never be used by CBRA purposes.

However, during the discussion on RACH partitioning framework, it is agreed that feature-specific RACH partition can be defined in a) separated time frequency resource or b) separated preambles from the preambles configured for legacy RACH partition:

Agreement in RAN2#115 3. New feature and/ feature combination specific preambles can be defined in a) Separate time-frequency resources, not defined through legacy RRC signalling, b) Within the Contention free preamble resources (i.e. within the preambles not used for contention based) defined through legacy RRC signalling. FFS on c) Within the “not available” preambles defined at the end of a RO through the legacy Agreement in RAN2#116 - No new feature and/ feature combination specific preambles are defined within the “not available” preambles defined at the end of a RO through the legacy totalNumberOfRA-Preambles

In other words, according to the above agreement, the RACH partitioning for any feature should be configured by a) separated RO or by b) separated RA preambles. Therefore, if nothing is captured as an agreement or in specification, it would be ambiguous if it is possible to configure legacy RACH partition duplicated with the RACH partition associated with any feature(s) (i.e., Option 2).

Therefore, in order to allow the Option 2, RAN2 should clarify that for RedCap-specific initial BWP, the legacy RACH partition and the feature-specific RACH partition may have same RA preambles with shared ROs, in order to avoid the confusion caused by agreement made in RACH partitioning discussion.

In our view, the spec impact is not foreseen since the current RRC specification does not describe any restriction on configuring preamble range in RACH partition.

Proposal 1. RAN2 confirms that it is possible that the same CB preambles in “RA partition not associated with any feature” can be configured in another RA partition sharing the ROs in RedCap-specific initial BWP, in order to support the Msg1-based SI request.

3	Conclusion

In this paper, we discussed an issues on how to configure RACH partition which is not associated with any feature in RedCap-specific initial BWP, in order to support Msg1-based SI request. The discussion includes the following observations:

Observation 1. In order to support the Msg1-based SI request in RedCap-specific initial BWP, the network should always configure the RACH partition which is not associated with any feature in RedCap-specific initial BWP.

Observation 2. RACH partition which is not associated with any feature is defined as CBRA preambles not associated with FeatureCombinationPreambles IE, which can be configured by ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

Observation 3. In some cases, the CBRA preambles for RACH partition which is not associated with any feature is never used by UEs in RedCap-specific initial BWP.

Based on the above observations, followings are proposed:

Proposal 1. RAN2 confirms that it is possible that the same CB preambles in “RA partition not associated with any feature” can be configured in another RA partition sharing the ROs in RedCap-specific initial BWP, in order to support the Msg1-based SI request.

4	Reference

[1] Draft Report of 3GPP TSG RAN WG2 meeting #126 Fukuoka, Japan, 20 - 24 May, 2024

[2] R2-2405554 Correction on the RACH resource selection for Msg1 based SI request on the RedCap-specific initial BWP

[3] R2-2405835 Report of [AT126][756][Maint] PDCCH order based CFRA and SI request (ZTE)

[4] TS 38.321 v.18.2.0

[5] TS 38.331 v18.1.0