3GPP TSG-RAN2 Meeting #127	R2- 2407460

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.7.4 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on scheduling enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to study the scheduling enhancement with delay/deadline information, considering LCP complexity.

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105

This document continues discussion on further details of scheduling enhancement for XR.

2.	Discussion

Aspects on scheduling enhancement for delay-critical data for XR

In Rel-18 XR, DSR procedure is defined in order to report the amount of delay-critical data when the remaining time until the PDCP discard timer expiry is less than the predetermined threshold. In order to ensure the transmission of data with low remaining time and to achieve the purpose of DSR, in Rel-19, it is agreed to study the delay-aware LCP enhancement for XR. Regarding some details of delay-aware LCP, followings are agreed in RAN2#126[2]:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

In RAN2#126, it is agreed to avoid the dynamic indication to apply the delay-aware LCP, in order to minimize the RAN1 impacts. However, if the delay-critical LCP is always applied based on configuration, additional UE complexity is expected without any specific benefits, since the UE needs to check whether there is delay-critical data in each logical channel whenever the UE receives uplink grant. In general, current LCP procedure with PBR has enough efficiency for scheduling, since the data with stringent QoS requirement would have higher logical channel priority. Therefore, in normal situation (e.g., when the remaining time of data is large enough), existing LCP procedure is sufficient to meet the QoS requirement on average, so it is not needed to consider any enhancement on LCP procedure, e.g., considering the remaining time or importance.

On the other hand, in some cases, the network may not be able to allocate the sufficient UL grant instantly, causing the delay-critical data and triggering DSR procedure. In this case, as mentioned above, prioritizing the data with short remaining time would avoid the unnecessary discard operation due to the transmission of data with large remaining time in the highest priority logical channel. Therefore, given that the DSR is transmitted in order to request the uplink grant for delay-critical data, the prioritization of data with short remaining time should be performed only if it is needed, i.e., after DSR is transmitted. After the transmission of DSR, i.e., to request uplink grant for urgent data, the delay-aware LCP should be applied until the corresponding delay-critical data is transmitted.

Proposal 1. The LCP enhancement to prioritize delay-critical data should be applied after DSR is transmitted.

Regarding the details of delay-aware LCP, it is agreed to define additional LCH priority as a baseline:

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH LCP prioritization within a logical channel will not be considered in RAN2 discussions

For additional LCP priority, based on the network configuration, it is possible to include data in LCH with the delay-critical prior to other LCHs by adjusting/using higher priority for LCH with delay-critical data. Regarding how to apply the additional priority for LCHs with delay-critical data, there are two options:

- Option 1: the additional priority is applied for delay-critical data within the LCH

- Option 2: the additional priority is applied for all data within the LCH

For Option 1, it would be better to prioritize delay-critical data over other data, when there are more than one LCHs with delay-critical data. However, if there is delay-critical data and non-delay critical data for one LCH, the LCP operation would be more complicated, since the same LCH should be processed with two priorities for each delay-aware LCP operation; the LCH is firstly applied with the adjusted LCP for the delay critical data and then the LCH is applied for the legacy LCP for the remaining data. Given that the scheduling enhancement is to prioritize the delay-critical data with minimized LCP complexity, it would be better to prioritize all the data in the LCH with delay-critical data, i.e. Option 2.

Proposal 2. For adjustment the priority of LCH with delay-critical data, the adjusted priority is applied to all data in the same LCH.

In RAN2#126 meeting, some companies suggested to prioritize the delay-critical data over other data within the LCH, in order to ensure the transmission of delay-critical data if both delay-critical data and other data are buffered in the same LCH. However, due to additional complexity of current LCP procedure, it is agreed that LCP prioritization within a logical channel is not considered.

In our view, the intended behaviour to transmit the delay-critical data first within the LCH can be handled in RLC entity, i.e., by prioritize delay-critical data over the other data. Given that some prioritization rules are already defined in RLC layer (e.g., prioritize retransmission of RLC SDU/SDU segments over other RLC SDUs), the prioritization rule to prioritize delay-critical data could be simply added as well. Therefore, in order to ensure the transmission of delay-critical data within the same LCH, RLC entity should prioritize the delay-critical data over other data.

Proposal 3. In RLC layer, delay-critical data is prioritized over the other data.

Meanwhile, according to the current LCP procedure, only PBR of each LCH is included in the first round. That is, if the LCH1 has delay critical data and LCH2 has not delay-critical data, even though LCH1 has higher priority over LCH2 due to additional LCP for delay-critical data in LCH1, the MAC entity would assembly the data from LCH1 only up to its PBR, then the data in LCH2 will be included before including the remaining data in LCH1. In this case, if the amount of buffered delay-critical data in LCH1 is larger than the PBR, non-delay data in LCH2 will be included prior to the delay-critical data in LCH1.

Therefore, in order to ensure that all the delay-critical data in a logical channel can be included prior to non-delay critical data in other logical channels, it should be possible to set the PBR of LCH with delay-critical data to infinity.

Proposal 4. For adjustment of LCP of LCH with delay-critical data, the PBR should be set to infinity.

During the RAN2#125bis meeting, some companies suggested to additionally consider the PDU set importance for scheduling enhancements for XR. It is true that low importance data should not be prioritized in congestion scenario, since the transmission of low importance data is not essential although it has less remaining time due to reduced PDCP discard timer value. However, since the main purpose of the scheduling enhancements is to avoid the unnecessary performance degradation due to discard operation of the data with short remaining time, the PDU set importance does not need to be carefully considered for scheduling enhancement in general (e.g., other than congestion scenario). Given that delay-critical data is already defined in Rel-18 XR and it does not include the low importance data in congestion scenario, it would be sufficient to prioritize delay-critical data for scheduling enhancement. Then, there is no need to additionally consider the PDU set importance.

Proposal 5. Reuse the definition of delay-critical data for the delay-aware LCP.

Regarding an issue on whether the separated delay threshold is needed for delay-aware LCP, if the separated delay threshold for delay-aware LCP is defined, additional definition for delay-critical data for delay-aware LCP is needed, e.g., in order to exclude low importance data in congestion scenario as explained in proposal 4. However, there is no critical reason to define the separated delay threshold to identify delay-critical data for delay-aware LCP. Specifically,

- If the delay threshold for delay-aware LCP is larger than the delay threshold for DSR trigger, the principle of existing LCP procedure would not be applied even though the corresponding data is not urgent, which unnecessarily increase the UE complexity of LCP procedure.

- If the delay threshold for delay-aware LCP is less than delay threshold for DSR trigger, the DSR would be triggered again even though the delay-critical data for LCP is transmitted based on the delay-aware LCP.

Rather, if the new delay threshold and delay-critical data is defined for delay-aware LCP, the additional UE complexity is expected since the UE should identify two types of delay-critical data, i.e., delay-critical data for DSR and delay-critical data for delay-aware LCP. In this sense, it would be simpler to reuse the existing definition used in DSR operation, in order to avoid the additional work and the UE complexity to determine two types of delay-critical data.

Proposal 6. No need to define the additional delay threshold for delay-aware LCP.

In RAN2#126 meeting, for LCH restriction, it is agreed that LCH restriction-based solution should not disallow other LCH from using an UL grant.

For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant.

Given that the additional LCH priority can be defined to prioritize LCHs with delay-critical data, the remaining issue is whether the additional LCH restriction is needed. In our view, if the additional LCH priorities for LCHs with delay-critical data are configured with higher than other LCHs and the PBR is adjusted to infinity, the data from LCH with delay-critical data will be included anyway prior to the data from other LCH. In this sense, if the proposal 3 is agreed, additional LCH restriction to include only data from LCH with delay-critical channel is not needed since the result would be similar to additional LCP priority.

On the other hand, some companies suggested that data from LCH with delay-critical data can be included even though it is not allowed to include the data based on the existing LCH restriction. However, the existing LCH restriction can be configured with various reason, e.g., network policy, it is not reasonable to use the uplink grant even though it is not permitted by the network. In addition, if the network receives the DSR MAC CE due to delay-critical data, the network would allocate uplink grant which can include the corresponding delay-critical data, so there is no critical reason to define additional enhancement for LCH restriction.

Proposal 7. No additional enhancement on LCH restriction is needed for XR.

In RAN2#125bis meeting, it was proposed to study whether there is an impact on transmission of SRB signalling or MAC CEs. Since the SRB signalling and MAC CEs are used to handle the radio condition or UE status in general, e.g., measurement report, handover, beam failure recovery, buffer status, etc., the transmission of SRB signalling and MAC CEs should be prioritized over the data, regardless of remaining time of the data. Therefore, there is no need to define a mechanism to prioritize delay-critical data over SRB signalling and MAC CEs.

Proposal 8. No need to define a mechanism to prioritize delay-critical data over SRB signalling and MAC CEs

Aspects on DSR enhancement

In RAN2#125bis meeting[3], it is agreed to enhance the DSR enhancement to include additional information.

RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

In RAN2#125bis meeting, it was suggested to include non-delay critical data in DSR, given that DSR also include the amount of buffered data. However, it should be noted that the same discussion was held in Rel-18 XR, when the DSR is defined. Specifically, there was long discussion on whether to associate the DSR procedure and existing BSR procedure. As a result, it was agreed to define DSR operation independently from the existing BSR operation, due to complexity on mixing DSR and BSR procedure, e.g., impact on trigger/cancel BSR based on DSR operation, handling the case for LCGs not configured with DSR, etc.

On the other hand, if the non-delay critical data is included in DSR MAC CE, the additional discussion is needed including:

- whether the reported non-delay critical data may trigger DSR if its discard timer value becomes less than the threshold; and

- whether the reported non-delay critical data can be considered for delay-aware LCP if the uplink grant is received after the transmission of DSR MAC CE.

Therefore, given that there is no additional requirement for Rel-19 XR on this aspect, there is no critical need to repeat the same discussion.

Proposal 9. Do not consider the non-delay critical data in DSR MAC CE.

In addition, in RAN2#126 meeting, it is agreed to enhance the DSR enhancement to include multiple pairs of remaining times:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

Given that the multiple delay level of information can be included in the DSR MAC CE, it was remained as an FFS on whether there is an impact on existing DSR triggering condition. However, if the DSR is triggered multiple times for the same data, the additional overhead is expected due to additional transmission of DSR MAC CE. Given that DSR is triggered when the data is not transmitted until the data becomes delay-critical data, e.g., in congestion scenario, it is not desirable to cause additional overhead due to transmission of DSR MAC CE multiple times for the same data. In this sense, the DSR trigger should not be modified for DSR MAC CE with multiple level of remaining times.

Proposal 10. DSR triggering should not be impacted for the multiple pairs of remaining times and data for an LCG.

Aspects on intra-UE prioritization on remaining time

In NR Rel-16, the intra-UE prioritization mechanism is introduced to support tighter latency and reliability. In the Rel-16 intra-UE prioritization, when the two UL grants are overlapped, the priority of each UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, i.e., logical channel priority is only considered to determine which UL grant is prioritized.

However, the Rel-16 intra-UE prioritization does not consider the remaining time of data, and it may have trouble to satisfy strict delay requirement for XR service because one UL grant which is used for data from the highest priority of logical channel may be always prioritized over any other UL grant. For example, when two UL grants are given for XR service and one of UL grants has the highest priority logical channel with the data having long remaining time till the PDCP discard timer expiry but another UL grant has the relatively lower priority with the data having short remaining time (i.e., delay-critical data), the Rel-16 intra-UE prioritization prioritizes the UL grant having data from the highest priority logical channel. However, from XR service perspective, another UL grant for delay-critical data should be prioritized in this condition because it is not urgent to transmit the data with long remaining time in the highest priority logical channel and it can be transmitted later due to sufficient remaining time. Thus, to support tight XR latency requirement, the uplink grant for the delay-critical data should be prioritized in intra-UE prioritization.

Proposal 11. RAN2 should consider enhancement on intra-UE prioritization in order to prioritize the uplink grant for delay-critical data.

3.	Conclusion

In this contribution, it is discussed for the scheduling enhancement for XR. This document includes following observations.

Proposal 1. The LCP enhancement to prioritize delay-critical data should be applied after DSR is transmitted.

Proposal 2. For adjustment the priority of LCH with delay-critical data, the adjusted priority is applied to all data in the same LCH.

Proposal 3. In RLC layer, delay-critical data is prioritized over the other data.

Proposal 4. For adjustment of LCP of LCH with delay-critical data, the PBR should be set to infinity.

Proposal 5. Reuse the definition of delay-critical data for the delay-aware LCP.

Proposal 6. No need to define the additional delay threshold for delay-aware LCP.

Proposal 7. No additional enhancement on LCH restriction is needed for XR.

Proposal 8. No need to define a mechanism to prioritize delay-critical data over SRB signalling and MAC CEs

Proposal 9. Do not consider the non-delay critical data in DSR MAC CE.

Proposal 10. DSR triggering should not be impacted for the multiple pairs of remaining times and data for an LCG.

Proposal 11. RAN2 should consider enhancement on intra-UE prioritization in order to prioritize the uplink grant for delay-critical data.

4.	Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #126 Fukuoka, Japan, 20 - 24 May, 2024

[3] Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China,15 - 19 April, 2024