3GPP TSG-RAN2 Meeting #127	R2- 2407461

# Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Discussion on Random Access procedure for SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

This document shows the initial view on RAN2 impact on RA procedure for SBFD.

2.	Discussion

RA procedure using the RA resource within SBFD symbol

In RAN1#117 meeting [2], for RA aspects on SBFD, it is agreed to support Option 1 with Alt 1-1 and Option 2 for the configuration of RA resource in SBFD symbol:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. Conclusion For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1: - The legacy ROs that are valid for non-SBFD aware UEs are also valid for SBFD aware UEs. - It is up to the network to configure ROs in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon. - If legacy ROs are configured in SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon, network ensures the ROs are within the UL usable PRBs. Agreement For SBFD-aware UEs in RRC CONNECTED state, and for RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, consider whether additional condition(s) on top of what was agreed in RAN1#116bis for RO validation are necessary. For example, - Condition#1: A valid RO starts at least X>=0 (FFS the value) symbols after a last downlink non-SBFD symbol. - Condition#2: A valid RO starts at least X>0 (FFS the value) symbols after the SSB. - Condition#3: A valid RO does not precede a SSB in the PRACH slot. Agreement Update the following agreement made in RAN1#116-bis meeting: For RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) to support random access operation for SBFD-aware UEs in RRC CONNECTED state, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, - For FR2, adopt the following: - Alt 1: use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211) - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols. - For FR1, consider from the following alternatives: - Alt 1: Use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211) - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols. - Alt 2: Use existing random access configurations table for paired spectrum/supplementary uplink (i.e., Table 6.3.3.2-2 in TS38.211).

Specifically, based on RAN1 agreements, RA resource in SBFD symbol can be configured by followings:

- Option 1 with Alt 1-1: reuse the existing RA resource configuration for legacy UE, and define additional rule to interpret valid ROs within the UL subband within the DL slot, i.e.,

  - For legacy UE, only the ROs configured in UL slot (and/or UL usable PRB configured within the flexible slot, e.g., RO2 in the Figure 1). The ROs configured within the DL slot is not valid.

  - For SBFD-aware UE, both ROs configured within the UL slots (e.g., RO2) and ROs configured within the UL subband in SBFD symbol (e.g., RO1) are valid.

Figure 1. An example of RACH configuration Option 1 with Alt 1-1

- Option 2: define additional RACH configurations within the UL subband in SBFD symbol (e.g., RO1), which is other than legacy RACH configuration (hereinafter, referred to as ‘SBFD RACH configuration’)

Figure 2. An example of RACH configuration Option 2

For Option 1 with Alt 1-1, given that the SBFD-aware UE and legacy UE will use the same RACH configuration for RA procedure, there would be no critical impact on current MAC procedure. In other words, the MAC procedure on selecting RACH partition, Preamble Group, and/or SSB selection will be happened using the same RA parameters for SBFD-aware UE and legacy UE. It is true that the selection of RO may be different for SBFD-aware UE and legacy UE due to different rule to identify valid ROs, but the RO validation rule is currently defined in RAN1 spec. In RA procedure, the UE selects an RO among the valid ROs defined in TS 38.213. In this sense, the MAC procedure for RO selection would be same for SBFD-aware UEs and legacy UEs in MAC perspective, i.e., select an RO among the valid ROs for selected SSB.

Proposal 1. For RACH configuration Option 1 with Alt 1-1 (use one single RACH configuration with possible enhancement), assume that the same MAC procedure is applied for both SBFD-aware UE and legacy UE.

Similarly, for SBFD-aware UE, there would be no need to define any selection rule between SBFD ROs and legacy ROs, since the UE behaviour would be same when the UE uses SBFD ROs or legacy ROs within the same RACH configuration. In other words, given that RA procedure in MAC layer does not differentiate SBFD RO and legacy RO in the same RACH configuration, so prioritization rule to select between the SBFD RO and legacy RO is not needed as well.

Proposal 2. For RACH configuration Option 1 with Alt 1-1, no prioritization rule is needed in order to select between SBFD RO and legacy RO.

For Option 2, in our understanding, it is supported in order to configure separated RA parameters (e.g., prach-ConfigurationIndex, number of SSBs per an RO, RACH partition configuration, etc.) between SBFD ROs and legacy ROs. Note that Option 1 with Alt 1-2 (i.e., using the existing RACH configuration with newly introduced parameter(s)) is not supported in RAN1 for now.

Meanwhile, according to MAC procedure, RA preamble transmission is performed as following steps:

Step 1) UL carrier selection, BWP operation

Step 2) RACH partitioning procedure

  - Based on the selected RACH partition, MAC entity identifies RRC parameters based on the selected RACH partition (e.g., RACH-ConfigGeneric including prach-ConfigurationIndex, …)

Step 3) RA type selection, between 2-step RA and 4-step RA

  - Based on the selected RA type, MAC variables are initialized based on the RRC parameters configured in RACH configuration for the selected RACH partition and RA type

Step 4) SSB selection, RA preamble selection, RO selection

In other words, according to the current RA procedure, RRC parameters for this RA procedure are identified right after the Step 2, i.e., selection of the RACH partition. Therefore, given that the RA parameters in SBFD RACH configuration and legacy RACH configuration are different, selection between SBFD RACH configuration and legacy RACH configuration should be also determined during or after the Step 2, in order to initialize the RRC parameters based on the corresponding RACH configuration. Otherwise, it would be ambiguous that which RA parameters should be applied in the following procedure, e.g., for the selection of RA type, SSB, and RA preamble group, etc., between the RA parameters from SBFD RACH configuration and legacy RACH configuration.

Proposal 3. For RACH configuration Option 2, whether the SBFD-aware UE performs RA procedure using the legacy RACH configuration or additional RACH configuration should be determined before the RA type selection.

In this sense, the prioritization rule between the SBFD RACH configuration and legacy RACH configuration is essential, since the corresponding RA parameters for the following procedure should be determined, e.g., RA type selection, SSB selection, and RO determination. In our view, for SBFD-aware UE, SBFD RACH configuration should be prioritized over legacy RACH configuration, since the legacy UE would not use the SBFD RACH configuration. In other words, for SBFD-aware UE,

- if the SBFD RACH configuration is configured, RA is performed using SBFD RACH configuration

  - Some additional conditions to use SBFD RACH configuration can be defined, e.g., RSRP threshold.

- Otherwise, RA is performed using legacy RACH configuration

Proposal 4. For RACH configuration Option 2, SBFD-aware UE performs RA procedure using the SBFD RACH configuration, if it is configured. Additional condition to use SBFD RACH configuration (e.g., RSRP threshold) can be defined, based on RAN1 discussion.

Meanwhile, according to the current RA procedure, following methods were defined in order to configure separated ROs for various purpose:

- Method 1: define a new IE for the separated RACH configuration, e.g., RACH-ConfigCommonIAB defined for IAB

- Method 2: define a feature for RACH partitioning, and use RACH configuration within AdditionalRACH-Config, e.g., RO partitioning configured by AdditionalRACH-Config for SDT, Slicing, Msg1 repetition, etc.

Similarly, for SBFD RACH configuration, one of the above methods can be reused to define SBFD RACH configuration. In our understanding, Method 2 is slightly preferred since RACH partitioning framework is introduced to define common RA procedure for separated RA partition/configuration with various purposes. In addition, the additional step is not needed to select between SBFD RACH configuration and legacy RACH partition, since it can be defined within the RACH partition selection.

Proposal 5. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, following option can be considered:

- Option A: define the new signaling of RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

- Option B: define additional feature to defined SBFD partition, and reuse the existing for AdditionalRACH-Config to configure separated ROs and other RA parameters in SBFD RACH configuration.

Support of Msg1 repetition for SBFD RA procedure

In RAN1#116 [3], it was remained as an FFS whether the Msg1 repetition is supported within SBFD symbol and across SBFD symbols.

Agreement For SBFD aware UEs in RRC CONNECTED state, at least PRACH without repetition is supported in SBFD symbols. - FFS PRACH repetition in SBFD symbols. - FFS PRACH repetition across SBFD symbols and non-SBFDs symbols.

On the other hand, in our understanding of Option 1 with Alt 1-1, the same RA parameter will be applied for SBFD ROs and legacy ROs. In other words, the preamble partitioning would be same for SBFD ROs and legacy ROs as in Figure 3. Therefore, if the Msg1 repetition is configured within the legacy RO, RACH partition for Msg1 repetition would be configured in SBFD RO as well, with the same preamble index.

Figure 3. Exemplary RACH partition configuration for Option 1

Therefore, if the Msg1 repetition is not supported for SBFD ROs in Option 1 with Alt 1-1, the corresponding preambles would not be used with any purpose for SBFD-aware UE, which causes RA resource waste.

For Option 2, based on the network configuration, the RACH partition for Msg1 repetition can be configured within the separated RACH configuration, e.g., using the new RACH configuration (e.g., RACH-ConfigCommonSBFD) or the AddditionalRACH-Config defined in Rel-17. Therefore, there would be no critical spec impacts to support the Msg1 repetition using SBFD symbol.

Given that the SBFD operation for RA procedure is beneficial for UL coverage enhancement, the additional method to further enhance the UL coverage can be applied as well using the SBFD symbol. Therefore, it would be helpful to support the Msg1 repetition using the SBFD symbols in both RACH configuration options.

Proposal 6. Msg1 repetition is supported within the SBFD symbols.

However, when the Msg1 repetition is performed, it is not desirable to mix the SBFD ROs and legacy ROs to define the RO group for Msg1 repetition for SBFD-aware UE (i.e., Msg1 repetition across SBFD symbols and non-SBFDs symbols).

According to the current procedure for Msg1 repetition, RO group is defined in order to identify the ROs used for Msg1 repetition. When the Msg1 repetition is performed, RAR is started after the last transmission of the Msg1 repetition, i.e., last RO of the configured RO group.

Then, if Msg1 repetition is performed across SBFD symbols and non-SBFDs symbols for Option 1 with Alt 1-1, there would be an issue to identify the RAR window from the network perspective. For example, assume that RA resources are configured as follows for Option 1 with Alt 1-1.

Figure 4. An example of RA configuration for Option 1.

Then, for legacy UE, the RA resources within the DL slot are not valid, so the RO group for Msg1 repetition with Msg1 repetition number 2 would be defined as follows (as in purple-colored RO group), and the RAR window would be start after the last RO of the corresponding RO group.

Figure 5. RO group for legacy UE with Msg1 repetition number 2

On the other hand, if Msg1 repetition across SBFD symbols and non-SBFDs symbols for SBFD-aware UE, different RO grouping can be defined for SBFD-aware UE. Then, for orange-colored RO group, the RAR window would be started after the corresponding RO group, as follows:

Figure 6. RO group for legacy SBFD with Msg1 repetition number 2

Then, if the RA preamble is received in blue colored RO in Figure 7 from network perspective, it is ambiguous whether the RA preamble is transmitted by SBFD-aware UE or legacy UE as follows:

- For SBFD-aware UE, it is possible that the RO group is configured as orange color in Figure 6, and the first RA preamble transmission is failed.

- For legacy UE, it is possible that the RO group is configured as purple color in Figure 5, and the first RA preamble transmission is succeeded.

Figure 7. Ambiguous network behaviour to determine RAR window

Then, the network cannot determine which RAR window (RAR window for legacy UE or for SBFD-aware UE) is running at the UE side, so the network cannot transmit the RAR properly in order to handle the both cases, especially for the case when the RAR window for legacy UE does not overlap with the RAR window for SBFD-aware UE.

Observation 1. For RACH configuration Option 1 with Alt 1-1, RAR window would be ambiguous from the network side if the Msg1 repetition is supported across the SBFD ROs and legacy ROs.

For Option 2, as explained in Proposal 3, the corresponding RACH configuration should be determined prior to the RO selection. Therefore, it would be impossible to use ROs across the legacy RACH configuration and SBFD RACH configuration.

Observation 2. For RACH configuration Option 2, it is not reasonable to use ROs in multiple RACH configurations in one RA attempt.

Therefore, for simplicity, it is suggested to support Msg1 repetition using only ROs SBFD symbols or legacy ROs for both Options.

Proposal 7. Do not support the Msg1 repetition across SBFD symbols and non-SBFDs symbols.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following observations.

Observation 1. For RACH configuration Option 1 with Alt 1-1, RAR window would be ambiguous from the network side if the Msg1 repetition is supported across the SBFD ROs and legacy ROs.

Observation 2. For RACH configuration Option 2, it is not reasonable to use ROs in multiple RACH configurations in one RA attempt.

Based on the above observations, followings are proposed:

Proposal 1. For RACH configuration Option 1 with Alt 1-1 (use one single RACH configuration with possible enhancement), assume that the same MAC procedure is applied for both SBFD-aware UE and legacy UE.

Proposal 2. For RACH configuration Option 1 with Alt 1-1, no prioritization rule is needed in order to select between SBFD RO and legacy RO.

Proposal 3. For RACH configuration Option 2, whether the SBFD-aware UE performs RA procedure using the legacy RACH configuration or additional RACH configuration should be determined before the RA type selection.

Proposal 4. For RACH configuration Option 2, SBFD-aware UE performs RA procedure using the SBFD RACH configuration, if it is configured. Additional condition to use SBFD RACH configuration (e.g., RSRP threshold) can be defined, based on RAN1 discussion.

Proposal 5. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, following option can be considered:

- Option A: define the new signaling of RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

- Option B: define additional feature to defined SBFD partition, and reuse the existing for AdditionalRACH-Config to configure separated ROs and other RA parameters in SBFD RACH configuration.

Proposal 6. Msg1 repetition is supported within the SBFD symbols.

Proposal 7. Do not support the Msg1 repetition across SBFD symbols and non-SBFDs symbols.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Report of 3GPP TSG RAN WG1 #117

[2] Report of 3GPP TSG RAN WG1 #116