3GPP TSG-RAN WG2 Meeting #127	R2- 2407462

Maastricht, Netherlands, 19th Aug – 23th Aug, 2024

Agenda Item		:	8.8.2

Source		:	LG Electronics Inc.

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

According to the RAN plenary #104 meeting, the approved work item on NTN for NR contains the following objective on DL coverage enhancements [1]:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

During RAN2 #125bis and 126 meeting, RAN2 made some progress on this objective and reached the following statements:

Agreements:

- With regard to link level enhancement, RAN2 waits for RAN1 agreement on the DL channels to enhance before starting any RAN2 work.

- We will continue the discussion on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.) in the next meetings, trying to identify questions to RAN1 for aspects where we need their input

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

In this contribution, we discuss the impact of DL coverage enhancements (e.g. cell level/beam level DTX/DRX mechanism) on legacy UEs.

2. Discussion

According to [2], due to satellite power limitations, the maximum number of satellite beams that can be simultaneously active is approximately 10% of the total number of beams within coverage. To support all satellite beams with limited power, as outlined in WID [1], satellite beams can be configured to have different beam patterns across the satellite footprint. By doing so, a satellite can control the number of simultaneously active beams and alternatively activate all beams within the satellite footprint. From a satellite's perspective, configuring satellite beams with different beam patterns can be implemented by beam hopping technique, where the active beams are alternated at regular time intervals.

Proposal 1	RAN2 should consider beam hopping technique to provide satellite beams across the satellite footprint, given the limited power and feeder link bandwidth of satellite payloads.

As mentioned in the WID [1], SSB channel enhancement other than SSB periodicity extension is not considered. If the SSB cannot be transmitted with the specified periodicity, it could impact on the RA, RLF, and BFD procedures and may cause problems with legacy UEs, e.g. being unable to camp on the cell. Therefore, even if beam hopping is used, the beam hopping should not cause impact on periodic SSB transmission.

Proposal 2	During satellite beam hopping, SSB should be transmitted at the specified periodicity to ensure that legacy UEs can detect the cell.

Based on WID [1], we would like to implement satellite beam hopping using Rel-18 network energy-saving techniques. Since satellite beam hopping switches active satellite beams at regular time intervals, we can consider the cell discontinuous transmission (cell DTX) technique, which allows data transmission only during the active periods. Cell DTX is implemented by configuring periodic active and non-active periods for a cell.

Proposal 3	Consider the Rel-18 NES cell DTX technique to implement satellite beam hopping in NTN as a baseline. FFS whether Rel-18 cell DTX enhancement is needed or not to support beam hopping operation.

To apply the Rel-18 NES cell DTX technique to satellite beam hopping, some modifications need to be considered from NES cell DTX technique. Since the NES cell DTX technique is implemented at the NR cell level and satellite beam is not visible to UEs, it is necessary to consider how the satellite beams are mapped to NR cells/beams that are visible to UEs in order to discuss the applicability of NES cell DTX to NTN beam hopping.

Before considering the mapping relationship among the three entities, i.e. satellite beam, NR cell, and NR beam, we first consider the relationships between two of these entities.

1. 	Mapping between satellite beam and NR cell

Option 1.1: Satellite beam and NR cell have a one-to-one mapping, 1:1

Option 1.2: Satellite beam and NR cell have a one-to-many mapping, 1:N

Option 1.3: Satellite beam and NR cell have a many-to-one mapping, N:1

Option 1.4: Satellite beam and NR cell have a many-to-many mapping, N:N

2. 	Mapping between Satellite beam and NR beam

Option 2.1: Satellite beam and NR beam have a one-to-one mapping, 1:1

Option 2.2: Satellite beam and NR beam have a one-to-many mapping, 1:N

Option 2.3: Satellite beam and NR beam have a many-to-one mapping, N:1

Option 2.4: Satellite beam and NR beam have a many-to-many mapping, N:N

3. 	Mapping between NR cell and NR beam

Option 3.1: NR cell and NR beam have a one-to-one mapping, 1:1

Option 3.2: NR cell and NR beam have a one-to-many mapping, 1:N

Though there seems to be multiple mapping options between satellite beam, NR cell, and NR beam, 1:1:1 mapping is a basic mapping option and should be considered. The issue is what other mapping option should be considered than 1:1:1.

For mapping options between satellite beam and NR cell, Option 1.1 is a basic mapping option and should be considered. However, Option 1.2 is same as Option 1.1 from UE perspective, and does not need to be considered. The reason is that even if a single satellite beam is mapped to multiple NR cells, the single satellite beam determines the active/non-active state of the NR cell, same as in Option 1.1. On the other hand, Option 1.3 is different from Option 1.1 because a single satellite beam cannot determine the active/non-active state of the NR cell. Thus, Option 1.3 should be considered in addition to Option 1.1. However, Option 1.4 is a combination of Option 1.1 and Option 1.3, and does not need to be considered. To conclude, it is sufficient to consider Option 1.1 and Option 1.3 among mapping options between satellite beam and NR cell.

For mapping options between satellite beam and NR beam, Option 2.1 is a basic mapping option and should be considered. However, Option 2.2 is same as Option 2.1 from UE perspective, and does not need to be considered. It is important to note that the remaining options, i.e. Option 2.3 and Option 2.4 are not feasible because satellite beams cannot have identical NR beam. Therefore, it is sufficient to consider Option 2.1 among mapping options between satellite beam and NR beam.

For mapping options between NR cell and NR beam, both Option 3.1 and Option 3.2 should be considered because they are all feasible in NR

Based on the reasoning above, possible mapping options between satellite beam, NR cell, and NR beam that should be considered are categorized into two cases:

Case A: Satellite beam : NR cell : NR beam = 1:1:1

Case B: Satellite beam : NR cell : NR beam = N:1:N

Proposal 4	Consider two cases, where single or multiple satellite beams constitute a NR cell, for implementing Rel-18 NES cell DTX operations in NTN.

For the Case A, satellite beam is equal to NR cell and NR beam. Therefore, Rel-18 NES cell DTX operation can be directly implemented in NTN.

For Case B, satellite beam is equal to NR beam. In this case, the satellite beam is equivalent to one of the NR beams within an NR cell, and thus the beam-level DTX is required. Therefore, it is required to discuss whether Rel-18 NES Cell DTX can be adapted to beam-level DTX.

In Rel-18 NES, the NW can configure different cell DTX patterns for each UE by dedicated RRC signalling. Let’s assume that followings are configured by the NW:

- SSB1:

- Serving UEs: UE1, UE2, UE3

- DTX pattern 1

- SSB2:

- Serving UEs: UE4, UE5, UE6

- DTX pattern 2

Considering that the cell DTX configuration can be set via RRC reconfiguration, the NW can configure cell DTX pattern 1 for UE 1,2,3 and cell DTX pattern 2 for UE 4,5,6. From the UE’s perspective, since only one cell DTX pattern is visible, the UE performs cell DTX without any problem using the DTX pattern corresponding to the SSB beam serving the UE. For example, UE1 will perform cell DTX using DTX pattern1, which corresponds to the DTX pattern for SSB1 from the cell’s perspective.

In summary, from the UE’s perspective, there is only one DTX pattern per cell. However, from the cell perspective, multiple DTX patterns are configured corresponding to each SSB beam. In addition, the DTX pattern activation/deactivation would be handled using DCI format 2_9, the same as NES cell DTX activation/deactivation. From the discussion, it can be concluded that beam level DTX can be implemented using the Rel-18 NES cell DTX and no additional enhancements are needed for the beam level DTX.

Observation 1	The NW can configure different cell DTX patterns for each UE with dedicated RRC signals, so that the UEs on the same SSB beam can be configured with the same DTX pattern by the NW implementation.

Proposal 5	Beam level DTX can be implemented without changing Rel-18 NES spec, even if satellite beam hopping is supported.

3. Conclusion

In this contribution, following statements are proposed:

Proposal 1	RAN2 should consider beam hopping technique to provide satellite beams across the satellite footprint, given the limited power and feeder link bandwidth of satellite payloads.

Proposal 2	During satellite beam hopping, SSB should be transmitted at the specified periodicity to ensure that legacy UEs can detect the cell.

Proposal 3	Consider the Rel-18 NES cell DTX technique to implement satellite beam hopping in NTN as a baseline. FFS whether Rel-18 cell DTX enhancement is needed or not to support beam hopping operation.

Proposal 4	Consider two cases, where single or multiple satellite beams constitute a NR cell, for implementing Rel-18 NES cell DTX operations in NTN.

Observation 1	The NW can configure different cell DTX patterns for each UE with dedicated RRC signals, so that the UEs on the same SSB beam can be configured with the same DTX pattern by the NW implementation.

Proposal 5	Beam level DTX can be implemented without changing Rel-18 NES spec, even if satellite beam hopping is supported.

4. References

[1] RP-241667 Rel-19 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] R1-2400303 Discussion on NR NTN Downlink coverage enhancements, Thales.