3GPP TSG RAN WG2 Meeting #127	 R2-2407465

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.6.2

Source: 	ITL

Title: 	Discussion on inter-CU LTM

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to inter-CU LTM are as follows:

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. - Xn-based inter-CU LTM is prioritized in Rel-19. - The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. - For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. - The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. - In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). - The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. - For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. - The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. - RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. - R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. - Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. - The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In this contribution, we discussed the remaining issues on inter-CU LTM.

Discussion

Reference configuration

In the previous meetings, there was discussion on whether to support single or multiple reference configurations for LTM candidate cells in an Inter-CU LTM scenario, but it was not decided yet. In this document, we would like to show our view on supporting single/multiple reference configuration.

Reference configuration refers to the RRC configuration commonly applied to multiple LTM candidates to reduce signaling overhead. Once LTM is determined to be performed, the UE applies the LTM candidate configuration of the target cell to the reference configuration to obtain a complete configuration. If complete configuration (ltm-ConfigComplete) is included in an LTM-Candidate IE, the LTM candidate configuration is a complete configuration.

RRC Signaling overhead

Aspects of single reference configuration in inter-CU LTM, considering the case where each CU has different configuration and capabilities, it is expected that more limited information may be included in the reference configuration than in the Intra-CU LTM scenario. Also, reference configuration will become more restricted as the number of CUs increases. Therefore, the reference configuration has less information and each candidate configuration has more information.

Aspects of multiple reference configuration in inter-CU LTM, since each CU can have a reference configuration, the reference configuration can contain the same information as the Intra-CU LTM scenario. In this case, as the number of reference configurations increases, they contain more information, while each candidate configuration can have less information.

Observation 1: In Inter-CU LTM, a single reference configuration may have a limited configuration

Interaction between gNB-CUs

In Rel-18 Intra-CU LTM preparation phase, gNB-CU may request the candidate gNB-DU to provide the lower layer configuration for purpose of generating the reference configuration or provide the lower layer reference configuration to the candidate gNB-DU.

We are not sure whether the source gNB-CU can configure the RRC configuration of the candidate gNB-CU, but we assume that it cannot. In other words, a gNB-CU should configure RRC configuration of its associated gNB-DU.

Aspects of single reference configuration in inter-CU LTM, source gNB-CU requests candidate gNB-CU(s) to provide the configuration for purpose of generating the reference configuration. Then, the source gNB-CU informs the reference configuration it has determined to the candidate gNB-CU(s). The candidate gNB-CU(s) needs to perform RRC configuration of the candidate cell based on the reference configuration from the source gNB-CU and then respond to the source gNB-CU.

Aspects of multiple reference configuration in inter-CU LTM, to generate LTM configuration, source gNB-CU requests candidate gNB-CU(s) to provide the reference configuration of candidate gNB-CU and RRC configuration of the candidate cell.

Observation 2: More interaction between CUs is needed for single reference configuration

In our understanding, it seems difficult to distinguish which option between single reference configuration and multi-reference configuration has more RRC signaling overhead and it seems to depend on the network environment. However, since the introduction of the reference configuration is intended to reduce the redundant information of each candidate cell by setting it in common (i.e., reducing signaling overhead), so we think that a single reference configuration may deviate from this purpose. And, multiple reference configuration has more simple interaction between gNB-CUs. Therefore, we prefer to introduce multiple reference configurations.

Proposal 1: Introduce multiple reference configuration for inter-CU LTM

If multiple reference configuration is adopted for inter-CU LTM, to distinguish which reference configuration the UE should apply, candidate cell(s) should be associated with reference configuration. In other words, each CU has a reference configuration, and the candidate cell(s) belonging to each CU are associated with that reference configuration.

Proposal 2: If P1 is agreed, candidate cell is associated with a reference configuration

Support of mixture of inter-CU and intra-CU LTMs

In the previous meetings, to distinguish UE behaviour on Intra-CU LTM/Inter-CU LTM, a new ID based option has been proposed. Since PDCP re-establishment and security key change is needed for inter-CU LTM which is different from intra-CU LTM, it is necessary for the UE to distinguish between Inter-CU LTM and Intra-CU LTM.

Until now, methods for UE to distinguish between Inter-CU LTM and Intra-CU LTM have not been decided yet.

- Security information in MAC CE: if security information is included in MAC CE, the UE performs the inter-CU LTM (PDCP re-establishment and security key change)

- New ID in RRC: A parameter that distinguishes CUs. If the ID of the source cell and the ID of the target cell are different, the UE performs the Inter-CU LTM (PDCP re-establishment and security key change)

  - Multiple configuration: If multiple configuration is adopted for the inter-CU LTM, a new ID should be introduced to associate reference configuration with candidate cell(s)

- Others?

At this now, the new ID based method is quite intuitive unless the UE can distinguish between inter-CU LTM and intra-CU LTM operation via MAC CE. As mentioned above, if multiple reference configuration is adopted, a new ID is configured per candidate cell and should be associated with a reference configuration.

Proposal 3: Introduce a new ID to distinguish between intra-CU LTM and inter-CU LTM

Dual connectivity

As discussed in the R18 intra-CU LTM scenario, the NR standalone scenario should be discussed initially in inter-CU LTM. Thus, in previous RAN2 meeting, RAN2 agreed to discuss NR standalone scenario first for inter-CU LTM and it will be used baseline for supporting NR-DC scenarios.

Some NR-DC scenarios are supported in intra-CU LTM. For example, MCG LTM with SCG release and MCG LTM without SCG change and intra-SN SCG LTM without MN involvement. For the inter-CU LTM in NR-DC, some scenarios have been considered with second priority in WID as follows:

- CU is acting as SN and MN is unchanged

- CU is acting as MN and SN is unchanged or released

In the previous meetings, RAN2 agreed with the LTM configuration for Inter-CU LTM in DC scenario as follows:

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

In our understanding, the above agreement means that only one LTM configuration with inter-CU candidate cell can be configured to the UE either by MN or SN. In other words, UE does not need to report the L1 measurement with inter-CU cell(s) to both the MN and SN simultaneously.

We think that some additional clarifications are needed. Since there is only one LTM configuration with Inter-CU LTM Candidate cells in the DC scenario, the following configuration should be confirmed regardless of which node (MN or SN) configures LTM configuration with Inter-CU LTM candidate cells.

- Inter-CU LTM Candidate cell can be configured either for inter-CU MN LTM or inter-CU SN LTM

- Inter-CU MN LTM and Inter-CU SN LTM can be configured in an LTM configuration simultaneously

Proposal 4: For DC, RAN2 to discuss and confirm the following LTM configuration with inter-CU LTM candidate cells

- Inter-CU LTM candidate cell can be configured either for inter-CU MN LTM or inter-CU SN LTM

- Inter-CU MN LTM and Inter-CU SN LTM can be configured in an LTM configuration simultaneously

Conclusion

In this contribution, we discussed remaining issues on inter-CU LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: Introduce multiple reference configuration for inter-CU LTM

Proposal 2: If P1 is agreed, candidate cell is associated with a reference configuration

Proposal 3: Introduce a new ID to distinguish between intra-CU LTM and inter-CU LTM

Proposal 4: For DC, RAN2 to discuss and confirm the following LTM configuration with inter-CU LTM candidate cells

- Inter-CU LTM candidate cell can be configured either for inter-CU MN LTM or inter-CU SN LTM

- Inter-CU MN LTM and Inter-CU SN LTM can be configured in an LTM configuration simultaneously

Reference

RAN2#126 meeting report