3GPP TSG-RAN WG2 Meeting #127	R2-2407466

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	7.24.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on intra-band EN-DC channel spacing

Document for:	Discussion and Decision

1. Introduction

RAN2 received a LS [1] from RAN4 on intra-band EN-DC. In this contribution, we discuss about the intra-band EN-DC channel spacing issue according to RAN4 LS.

2. Discussion

According to current TS 38.306, the contiguity capability for intra-band EN-DC can be indicated through intraBandENDC-Support. The UE can indicate either contiguous (i.e. through absence of the field), non-contiguous, or both contiguous and non-contiguous capability for one (NG)EN-DC combination. In 38.331, there are separate bits defined for “non-contiguous” and “both”. In other words, the UE is able to declare support of either one of these two capabilities separately.

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |

According to RAN4 LS, from RAN4 perspective, the UE supporting ‘non-contiguous’ should not preclude the support of ‘contiguous’ in the same band, since support of ‘non-contiguous’ is kind of more stringent implementation. However, such restriction is not reflected in current specs. The UE already in the market is not expected to follow such requirement in any conformance test. It is impossible to do any NBC change now in RAN2.

The issue came out when the usable spectrum is only contiguous spectrum in some regions. Operators are motivated to deploy the contiguous EN-DC combinations for better system performance. In such combinations, the UEs reporting “non-contiguous” in intrabandENDC-Support may not be able to access to corresponding cells.

One solution is the upgraded Rel-18 UE indicates “both” for intrabandENDC-Support. It means the UE is required to meet the requirement for both contiguous and non-contiguous spectrum. Another solution provided by RAN4 is to introduce a new capability to indicate support of intra-band non-contiguous spectrum with nominal channel spacing. Nominal channel spacing is defined in RAN4 to illustrate the contiguous spectrum between adjacent carriers. With the new capability indicated, the requirement for non-contiguous spectrum (e.g. MPR/A-MPR) can be followed even though there is nominal channel spacing between EUTRA and NR carriers. In this way, it is simpler for the upgraded UEs to support the new capability.

Proposal 1: Introduce new capability to indicate support of intra-band non-contiguous (NG)EN-DC with nominal channel spacing.

Besides, in current spec, there is another capability intrabandENDC-SupportUL defined when there is different contiguity capability between DL and UL. This capability is only applicable for special EN-DC combinations in RAN4 spec, e.g. DL DC_(n)48CA, UL DC_48A_n48A. If the UE supports same contiguity between DL and UL, the legacy field intrabandENDC-Support will be applied. In our view, the new capability can be applied to DL and/or UL, only if the corresponding direction is indicated with “non-contiguous” by intrabandENDC-Support/intrabandENDC-SupportUL. There is no need to indicate the new capability for DL and UL separately in the same band combination. For example, when “non-contiguous” is supported in both DL and UL (i.e. intrabandENDC-Support is set to “non-contiguous”) and the new capability is signalled, the UE supports intra-band non-contiguous EN-DC with nominal channel spacing for both DL and UL. When “contiguous” is supported in DL and “non-contiguous” is supported in UL (i.e. intrabandENDC-Support is set to “contiguous” and intrabandENDC-SupportUL is set to “non-contiguous”), and the new capability is signalled, the UE supports intra-band non-contiguous EN-DC with nominal channel spacing only in UL.

Lastly, in last meeting, for the inter-band EN-DC combination with multiple intra-band components, it was agreed to introduce a list to indicate the contiguity capability for each component. We understand the new capability is a common capability for all the DL/UL intra-band EN-DC components with “non-contiguous” indicated.

In conclusion, the new capability should be introduced in per-BC level, which is applicable for all the DL and/or UL intra-band EN-DC component(s) with non-contiguous capability in the BC. The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/ intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

Proposal 2: The new capability is in per-BC level, which is applicable for all the DL and/or UL intra-band EN-DC component(s) with non-contiguous capability in the BC.

Proposal 3: The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

3. Conclusion

In this contribution, we have following proposals:

Proposal 1: Introduce new capability to indicate support of intra-band non-contiguous (NG)EN-DC with nominal channel spacing.

Proposal 2: The new capability is in per-BC level, which is applicable for all the DL and/or UL intra-band EN-DC component(s) with non-contiguous capability in the BC.

Proposal 3: The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

4. Reference

[1] R4-2410693, LS on intra-band EN-DC channel spacing.