3GPP TSG RAN WG2 Meeting #127	 R2-2407470

Maastricht, Netherlands, Aug. 19th – 23th, 2024

Agenda Item:	8.6.3

Source: 	ITL

Title: 	Discussion on event triggered L1 measurement

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to measurement enhancements are as follows:

Agreements on measurement enhancements for LTM: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we discussed the event triggered L1 measurement reporting for LTM.

Discussion

In R18 LTM (L1/L2 layer triggered mobility), to support dynamic switching among the configured candidate cell and reduce handover latency, L1 measurement based mobility was introduced. Compared to the traditional L3 measurement, L1 measurement has less latency since the L1 measurement reporting through L1/L2.

L1 measurement reporting is configured and triggered by NW. NW can configure periodic/semi-persistent report on PUCCH or configure aperiodic/semi-persistent on PUSCH. It will cause signalling overhead and the measurement report will be reported at a delayed time.

In L3 measurement reporting, NW can configure some events to trigger measurement reporting as follows [2]:

Event A1: Serving becomes better than absolute threshold; Event A2: Serving becomes worse than absolute threshold; Event A3: Neighbour becomes amount of offset better than PCell/PSCell; Event A4: Neighbour becomes better than absolute threshold; Event A5: PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2; Event A6: Neighbour becomes amount of offset better than SCell;

Event definition

In the previous meeting, some L1 measurement events (Event LTM2-5) are agreed and Event LTM1 is still remaining on FFS.

- (FFS) Event LTM1: Beam of serving cell becomes better than absolute threshold;

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

Since the network decides the cell switch based on the L1 measurement report, the network does not need to receive L1 measurement report for purposes other than mobility. If the network receives Event LTM1 from the UE indicating that the beam of serving cell becomes better than threshold, it is unclear what kind of thing for mobility can be done in the network. Therefore, we think the agreed Event LTMs are sufficient.

Proposal 1: RAN2 not to introduce Event LTM 1: Beam of serving cell becomes better than absolute threshold

What beam(s) of the serving cell and neighboring cell is used for event evaluation

In the previous meeting, there was discussion about which beam should be evaluated for LTM events, but it is still remaining on FFS.

For the beam(s) of serving cell,

- Current beam which is indicated TCI state

- Best beam

For the beam(s) of neighboring cell,

- Beam(s) indicated in L1 measurement resource configuration

First of all, It is necessary to confirm whether multiple beams need to be evaluated for event evaluation. For the serving cell, we think that there is no need to check multiple beams for event evaluation since L1 measurement report for LTM is used for the mobility. However, for the neighboring cell, evaluating multiple beams can help the network to determine or set the cell switch (e.g., TCI state activation, CFRA resource). Here, multiple beams can be SSB indices or CSI-RSs indicated within the L1 measurement resource configuration.

Proposal 2: For the serving cell, single beam is used to evaluate Event LTMx

Proposal 3: For the neighboring cell, multiple beams are used to evaluate Event LTMx

Association measurement event with L1 measurement resource configuration

In RAN2#125bis, RAN2 agreed that measurement event is associated with L1 measurement resource configuration as following:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

In Rel-18 LTM, CSI resource configuration (LTM-CSI-ResourceConfig) is distinguished by an ID, and an SSB index(s) can be indicated for each candidate cell ID. And the CSI report configuration (LTM-CSI-ReportConfig) is configured including the CSI resource configuration ID and associated with UL resource.

In Rel-19 LTM, RAN2 has not decided which UL resource to use for the L1 measurement report, but like Rel-18 CSI resource configuration, the L1 measurement resource configuration needs to be distinguished at least for each candidate cell, so it is reasonable to distinguish it with an ID.

At this now, two options can be considered as follows:

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate L1 LTM measurement event configuration to L1 measurement resource configuration.

- Option 2: Introduce list of L1 measurement resource configuration ID in the L1 LTM measurement event configuration

Proposal 4: RAN2 discusses the following options to associate L1 LTM measurement events with L1 measurement resource configuration:

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate L1 LTM measurement event configuration to L1 measurement resource configuration.

- Option 2: Introduce list of L1 measurement resource configuration ID in the L1 LTM measurement event configuration

Conclusion

In this contribution, we discussed measurement enhancement for LTM. According to discussion in section 2, we have the following proposals:

Proposal 1: RAN2 not to introduce Event LTM 1: Beam of serving cell becomes better than absolute threshold

Proposal 2: For the serving cell, single beam is used to evaluate Event LTMx

Proposal 3: For the neighboring cell, multiple beams are used to evaluate Event LTMx

Proposal 4: RAN2 discusses the following options to associate L1 LTM measurement events with L1 measurement resource configuration:

- Option 1: As in L3 measurement, introduce LTM measurement ID (LTM-MeasId) to associate L1 LTM measurement event configuration to L1 measurement resource configuration.

- Option 2: Introduce list of L1 measurement resource configuration ID in the L1 LTM measurement event configuration

Reference

RAN2#126 meeting report