3GPP TSG-RAN WG2#127	      					      R2-2407471

Maastricht, Netherlands, August 19 – 23, 2024

Agenda item:	8.1.2.2

Source: 	Kyocera

Title: 	Considerations for the Five Functionalities Conditions

Document for:	Discussion

Introduction

RAN2 decided to discuss different functionalities in offline discussions at the RAN2#126 meeting. In the 1st round of email discussion ([POST126][032][AIML PHY] LCM (Intel, Samsung)) [1], the rapporteur summarized the following conclusion based on the opinions from each company.

Conclusion - 2.1 Supported functionalities - Proposal 1: supported functionalities refer to functionalities that UE can indicate by using UE capability signalling. - 2.2 Configured functionalities - Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed. - 2.3 Applicable functionalities - Proposal 3: Applicable functionalities refers to functionalities that the UE is ready to apply for model inference. - Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities. - 2.4 Activated Functionalities - Proposal 5: Activated functionalities refers to functionalities already activated and performing inference. - 2.5 Available Functionalities - Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

In this contribution, we discuss Proposals 2, 4, and 6, which are the discussion points stated hereinabove. Furthermore, we will refer to Supported Functionalities, Available Functionalities, Applicable Functionalities, Configured Functionalities, and Activated Functionalities collectively as 'Functionalities Conditions' to distinguish them from the Functionalities indicated by use cases and sub-use cases.

Discussion

RAN2 Agreements

The current RAN2 agreements [2][3] are shown in the following.

RAN2#125bis - Agreements - 1. Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to. - 2. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. - Agreements for positioning and beam management - 1. Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. - 2. Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality. - 3. FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

RAN2#126 - 1. RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc. - 2. We will work offline on the definitions for functionality types and define what is availability. - 3. The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability. - 4. For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning - 5. For BM use case, as a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive

Configured functionalities

The rapporteur reached the following proposal regarding Configured functionalities in the conclusion of the email discussion.

Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

In CA, the SCell is configured with the UE. When the SCell is applicable (i.e., the radio condition is good), the gNB activates the SCell of the UE via MAC CE. When the SCell is not applicable (i.e., the radio condition becomes bad), the gNB deactivates the SCell, but the gNB does not need to deconfigure the SCell from the UE.

In CA, the SCell is configured to the UE. When the SCell is applicable (i.e., the radio condition is good), the gNB activates the SCell of the UE via MAC CE. When the SCell is not applicable (i.e., the radio condition becomes bad), the gNB deactivates the SCell, but the gNB does not need to de-configure the SCell from the UE. Currently, we believe all functions are configured to the UE. In other words, no function can be activated without configuration. For example, even if the UE is capable of CA, it does not mean the CA function is configured. The CA (specifically, SCell) can be activated only after the CA is configured.

Based on current operations, it is best to consider that the activation of the model on the UE side can only occur after configuration from the NW side. This is because, without such a process, the NW side would not be able to manage the UE's operations. Since AI/ML operations in the use cases cannot be completed by the UE alone, management from the NW side is necessary. Therefore, we believe that this principle should also be applied to AI/ML operations. Furthermore, our preference is for NW configuration to be applicable to both proactive reporting and reactive reporting.

RAN2 should agree that Configured functionalities are configured from the NW side to the UE.

RAN2 should agree that no AI/ML functionality can be activated without configuration as a Configured functionalities from NW side.

RAN2 should agree NW configuration is required for both proactive reporting and reactive reporting.

Direction of Signalling for the five Functionalities Conditions

In view of Proposal 1, it is also important to clarify the direction of signalling for the five different functionalities conditions in the UE-sided model in Table 1, which could be considered as baseline.

Table 1: Direction of Signalling for UE-Sided Model

| Functionalities Conditions to be achieved | Direction of Signalling |
| --- | --- |
| Supported Functionalities | UE → NW |
| Available Functionalities | UE → NW |
| Applicable Functionalities | UE → NW |
| Configured Functionalities | NW → UE |
| Activated Functionalities | NW → UE |

RAN2 should agree to use Table 1 as the baseline for the signalling directions of the five different functionalities conditions.

Applicable functionalities

The rapporteur reached the following conclusion in the email discussion.

Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

Furthermore, the rapporteur has provided the following summary and conclusions regarding this matter.

Summary from Q5 discussion: - Companies have diverged view on relationship between configured functionalities and applicable functionalities. - View 1: configured functionalities may or may not be applicable. - View 2: configured functionalities are always applicable because NW should configure functionalities within applicable functionalities. - View 3: no definition of configured functionalities. Therefore, no need to discuss.

In Proposal 2, it is stated that the activation of AI/ML functionalities can only be performed if they are configured functionalities. Therefore, we support View 2 above. In this case, we think that the NW can proceed to activate AI/ML when it is deemed configured.

RAN2 should agree that configured functionalities are always applicable.

Available Functionalities

The rapporteur reached the following conclusion in the email discussion.

Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

Regarding available functionalities, the rapporteur has also provided the following summary.

- Companies have quite diverged view on defining available functionalities separately from applicable functionalities. - Separate definition: - available functionalities are the super set of applicable functionalities e.g. due to NW sided additional conditions. - As a starting point, we could have two different definitions to discuss further details between UE and NW. - Not to define available functionalities - Applicability already imply that it has at least one available model. - Applicability is determined by UE i.e. applicability should not be defined from NW point of view only. - Need further discussion about specific cases and relationship between applicability and availability especially the case where there is no applicable functionality despite having available models.

First, we consider that Applicable functionalities are to be set as ‘applicable or not applicable’, and Available functionalities are to be set as ‘available or unavailable’. Furthermore, 'applicable/not applicable' and 'available/unavailable' should be considered as being used at different stages, since applicability relates to the situations or conditions under which a function can be applied, while availability depends on the current state or configuration.

We believe that it is possible to converge on each of these combinations. The reasons for this will be explained based on Table 2 below. This table shows the interactions between the applicability and the availability of functionalities, while also explaining why Available functionalities might be considered unnecessary.

Table 2: Applicable and Available Functionalities Results

| # | Applicability | Availability | Result |
| --- | --- | --- | --- |
| 1 | Applicable | Available | Included in Applicable functionalities |
| 2 | Applicable | Unavailable | Included in Applicable functionalities |
| 3 | Not applicable | Available ⇒ Unavailable | Converges to a state of unavailable |
| 4 | Not applicable | Unavailable | Whether to notify as not applicable functionalities is another discussion |

- 3. The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability.

- #1, #2:
According to the following agreements from RAN2#126, the applicable functionalities have already included available functionality. 

For #2, the following example illustrates a scenario whereby a functionality is applicable but unavailable.

  - Ex #1:
Applicable: The AI/ML model is trained and ready for inference.

Unavailable: The UE is short on computing resources, so it cannot execute this AI/ML model.

Outcome: The AI/ML model is 'applicable' because it is trained and ready to be used, but it is 'unavailable' due to the lack of UE computing resources.

- #3: We believe this case converges to the unavailable case. This can be better understood by considering the following example scenarios.

  - Ex #2:
Scenario: The computational resources (GPU or CPU) for the system are sufficient, and the AI/ML model can be executed.

Reason for Non-Applicability: The AI/ML model has not been trained on the current data or environment, making accurate predictions impossible. This means that while the functionality is not applicable in the current context, it could potentially be supported with the appropriate training on relevant data.

Outcome: Although the model can be executed, the results are unreliable, so it is considered practically unavailable.

- #4: Whether to consider this as not applicable functionalities is for a future discussion.

Based on #3, we believe following observation.

The not-applicable but available case converges to being unavailable.

Based on the above, we think that Applicable functionalities are sufficient for the situations classified as applicable/not applicable and available/unavailable, and therefore, Available functionalities are unnecessary.

RAN2 should agree that Available functionalities are unnecessary, since it is included within Applicable functionalities.

Conclusion

In this contribution, we discuss the issues of configured Functionalities, Applicable functionalities, and Available functionalities for AI/ML for the NR air interface. RAN2 is kindly asked to take into account the proposals below:

Proposal 1	RAN2 should agree that Configured functionalities are configured from the NW side to the UE.

Proposal 2	RAN2 should agree that no AI/ML functionality can be activated without configuration as a Configured functionalities from NW side.

Proposal 3	RAN2 should agree NW configuration is required for both proactive reporting and reactive reporting.

Proposal 4	RAN2 should agree to use Table 1 as the baseline for the signalling directions of the five different functionalities conditions.

Proposal 5	RAN2 should agree that configured functionalities are always applicable.

Observation 1	The not-applicable but available case converges to being unavailable.

Proposal 6	RAN2 should agree that Available functionalities are unnecessary, since it is included within Applicable functionalities.

References

- [POST126][032][AIML PHY] LCM (Intel, Samsung)

- “RAN2 Chair’s Notes”, RAN2#125bis

- “RAN2 Chair’s Notes”, RAN2#126