3GPP TSG RAN WG2 Meeting #127	 R2-2407473

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda Item:	8.8.4

Source: 	ITL

Title: 	Discussion on support of broadcast service

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to support of broadcast service is included as follows:

Agreements: - For MBS broadcast service, both EFC and EMC are supported. - RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. - MBS broadcast intended service area is provided via system information - For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS - When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this contribution, we discussed the intended service area and UE behavior to support of broadcast service.

Discussion

The MBS feature is introduced for the purpose of transmitting the same data/service to UEs in a specific service area. The specific service are can be defined by a cell list or a tracking area list in legacy MBS. Since non-terrestrial networks have large coverage than terrestrial networks, it is difficult to apply the MBS feature to the NTN system. In other words, NTN coverage has multiple tracking area codes and the radius of one cell is tens or hundreds of km.

Figure 1. Example of TN coverage information

Basically, location-based methods for mobility were introduced in the NTN system. In R18 NTN, TN coverage information is introduced to explain TN area by using reference location and distance radius. Figure 1 shows an example of TN coverage information. The network informs TN coverage information through system information, and the UE does not need to measure the frequency associated with TN coverage if the distance between UE and the reference location is smaller than the threshold (Distance radius).

Figure 2. Example of MBS service area

We think the easiest way to represent the MBS service area is to reuse the TN coverage information introduced in R18. However, since the MBS service area may not be in a circular shape, the methods discussed in R18 for expressing the geographic area of TN coverage can be considered for the MBS service area. For example, the boundary line of MBS service area or a parameter using the polygon shape captured in TS 23.032 for MBS service area.

In Option 1, MBS service area can be expressed with one reference location and radius. If the intended service area is not circular, a plurality of circular areas can represent one intended service area. In other words, each circular area has an identifier, and the intended service area for a specific service may have more than one circular service area identifier. The UE can determine whether to perform MBS service it can perform by checking the distance to all configured circular areas.

In Option 2, the network transmits the boundary information of MBS service area to the UE, and the UE can check whether the corresponding MBS service will be performed through the distance between UE and points. In Option 3, the UE can decide whether to perform MBS service based on the distance to the vertex of the polygon.

Options 2 and 3 have multiple reference locations, which increases signaling overhead and conditions for the UE to determine. Here, if the MBS service area is in more than one NTN cell, it is difficult to determine which conditions are appropriate to check depending on the UE location. Therefore, we prefer to reuse Option 1 (Circles format).

Proposal 1: Reuse circles (reference location and distance radius) format to model MBS service area

In RAN2#125bis, RAN2 agreed on the following scenarios for the intended service area for MBS in NTN:

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area)

Accordingly, in RAN2#126, several issues were raised and have not been decided as follows:

- Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

- Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

In our understanding, these are about UE behaviour in a situation where MBS can be received outside the intended service area (i.e., intended service area covers a portion of a NTN cell). In this case, whether the UE should not establish MRB(s) and should release established MRB(s).

The intended service area will be determined by the network based on the service, we think which means that MBS is only valid in the intended service area. Therefore, even if a UE within an NTN cell receives an MBS service, it can be invalid. So it is unnecessary for the UE to establish or maintain MRB(s) outside the intended service area.

Proposal 2: A UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area

Proposal 3: A UE should release its established MRB(s) associated with a service limited to an intended service area when exiting the area

Conclusion

In this contribution, we discussed intended service area for MBS. According to discussion in section 2, we have the following proposals:

Proposal 1: Reuse circles (reference location and distance radius) format to model MBS service area

Proposal 2: A UE should not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area

Proposal 3: A UE should release its established MRB(s) associated with a service limited to an intended service area when exiting the area

Reference

RAN2#126 Meeting report