3GPP TSG-RAN WG2 Meeting #127	R2-2407476

# Maastricht, Netherlands, 19th - 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0895 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE determining the clock quality information update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TRS_URLLC-NR-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For TRS_URLLC, it was agreed that the UE should confirm the update of the clock quality information when there is a change of the event ID or the gNB ID in RAN2#122 meeting. The agreement is excerpted below: 2. Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED. However, in current TS 38.300, the procedure for the UE in RRC_INACTIVE or RRC_IDLE determining if there is clock quality information update available at the gNB only includes the description of SIB9 information, while the description of gNB ID is missing. This is not consistent with the agreement, also is not aligned with the related stage-3 specification as below (TS 38.331 clause 5.2.2.4.10), which specifies that the UE shall notify upper layers the change of clock quality when the gNB ID changes. Upon receiving SIB9 with eventID-TSS, the UE shall perform the actions below if requested by upper layers: … 1> if the storedGlobalGnbID within the VarTSS-Info is different from the global gNB identity derived from SIB1: 2> notify upper layers that clockQualityDetailsLevel may have changed; 2> store the eventID-TSS received within SIB9 as the storedEventID within VarTSS-Info; 2> store the global gNB identity derived from SIB1 as the storedGlobalGnbID within VarTSS-Info.. |
| Summary of change: |  | Adding the missing description on gNB ID and clarify that the UE in RRC_INACTIVE and RRC_IDLE determines if there is clock quality information update based on SIB9 information and the gNB ID derived from SIB1. Impact Analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: TRS_URLLC Inter-operability: If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If the network is implemented according to this CR while the UE is not, there is no inter-operability issue, assuming UE implementation is following stage-3 specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage-2 specification is incomplete and misleading on the procedure of the clock quality information updating determination. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

#### 16.8.2.3	Network timing synchronization status monitoring towards UE

The gNB may receive clock quality reporting control information for a UE from the AMF, see TS 23.501 [3]. The clock quality reporting control information contains the clock quality detail level (i.e., "metrics" or "acceptable/not acceptable indication") and clock quality acceptance criteria for the UE (if the clock quality detail level equals "acceptable/not acceptable indication"). Based on the clock quality reporting control information, the gNB determines what clock quality information provide to the UE:

-	If the clock quality detail level equals "clock quality metrics", the gNB provides clock quality attribute values supported by the gNB to the UE.

-	If the clock quality detail level equals "acceptable/not acceptable indication", the gNB indicates "acceptable" to the UE if the gNB's timing synchronization status matches the acceptance criteria received from the AMF; otherwise, the gNB indicates "not acceptable" to the UE.

To provide clock quality information to the UEs:

-	For UEs in the RRC CONNECTED state, the gNB uses unicast RRC signalling. The RRC signalling includes Event ID and clock quality information.

-	For UEs that are not in the RRC_CONNECTED state, the UE first needs to establish or resume the RRC connection to receive the clock quality information from the gNB via unicast RRC signalling. The gNB broadcasts Event ID in SIB9 to notify its timing synchronization status. Event ID or gNB ID change serves as a notification for the UEs reading the SIB information that the RAN timing synchronization status information has changed.

The following figure describes the signalling procedure of gNB reporting clock quality information to a UE:

Figure 16.8.2-1: Signalling procedure of gNB reporting clock quality information to a UE

0.	The gNB is pre-configured with a threshold for each RAN TSS attribute it supports as described in in TS 23.501 [3]. When there is a primary source event as defined in TS 38.401 [4], the gNB may detect a change on its timing synchronization state (e.g., degradation, failure, recovery).

1.	The gNB notifies a change on its timing synchronization operation using Event ID in SIB9. The Event ID scope is local to gNB.

2.	The UE in RRC_INACTIVE or RRC_IDLE determines if there is clock quality information update available at the gNB based on SIB9 information and the global gNB identity derived from SIB1. For a UE in RRC_CONNECTED state, steps 2-3 can be skipped.

3.	If there is a RAN timing synchronization status update available, the UE's RRC layer indicates this to the NAS layer which may request the RRC layer to initiate RRC Setup or RRC Resume procedure.

4.	The gNB determines clock quality information reporting to the UE (e.g., metrics or "acceptable/not acceptable").

5.	The gNB sends the clock quality information to the UE via unicast RRC signalling.

End of Changes