3GPP TSG-RAN WG2 Meeting #127	R2-2407479

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Measurement Reduction based on RRM Measurement Prediction

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#125bis the following sub-use cases were agreed for RRM measurement prediction.

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results.

Based on the e-mail discussion [1], the two main identified objectives are:

- Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency and reduces the required signalling.

- Mobility optimization. Predict future UE DL radio measurements and use them to optimize mobility decisions.

Moreover, in RAN2#126, we agreed to prioritize the following sub-use cases.

| Case number | Prioritization | Evaluation scenario combination | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this document we present our results related to spatial, temporal and inter-frequency measurement prediction by considering different measurement reduction rates.

# 2	Discussion

## 2.1	Intra-Cell Spatial Prediction Aspects

One of the sub-use-cases identified in 3GPP TR 38.774 study item on AI/ML for inter-cell mobility in NR is intra-cell measurement prediction, which is an extension of the TR 38.843 beam prediction use case to intra-cell mobility. Figure 2.1-1 shows the 5G UE DL radio measurement model specified in 3GPP TS 38.300. The cell-level Layer-3 (L3) measurement (C), which is used in measurement event conditions and measurement reporting (D) that are the basis of handover decisions in the 3GPP Rel-15 baseline handover method, are derived from the Layer-1 beam-level measurements (A1). These measurements may be either RSRP, RSRQ or SINR, as configured in the measurement configuration in the RRCReconfiguration message (TS 38.300 and 38.331). To get the cell-level L3 measurement C, the beam level measurements are first consolidated into one measurement B. This is done by averaging the measurement of K best beams, as configured by the network in the RRCReconfiguration. The consolidated cell-level measurement B is then filtered with the Layer-3 filter to get the measurement C.

Figure 2.1-1: 3GPP TS 38.300 Measurement Model

## 2.1.1 Simulation setup

In RAN2#126, the following was agreed.

10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS.

In RAN2 meeting #126, a number of companies were presenting AI/ML solutions and evaluation results for Cases 1 and 3. These included LSTM and CNN ML models trained to predict either the missing beam-level measurements (A or A1) then deriving the cell-level filtered measurement with the TS 38.300 baseline method, i.e., Case 1, or to predict either B or C from the subset of beam-level measurements (A or A1). In case the unfiltered cell level measurement B is predicted with ML, the baseline L3 filter is applied in a separate step to get the L3 cell measurement C. Evaluation results submitted to RAN#126 were indicating that Case 3 was proving up to 3-times higher prediction accuracy than Case 1. To evaluate Case 3, we set up the 7-site simulation scenario based on TR 38.843 and as agreed in RAN#126. Table 2.1.1-1 shows the simulation assumptions.

Table 2.1.1-1: Simulation parameters for the 7-site scenario based on TR38.843

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 60 kmh |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 370K |
|  | Testing | 120K |
| AI/ML model input/output | Model input | Reduced set of unfiltered L1 measurements |
|  | Model output | Filtered L3 measurements |
| AI/ML model description | Model type | Random Forests |
|  | Model complexity in number of parameters | 0.3M |

## 2.1.2 Simulation results

The results are shown in Table 2.1.2-1. Measurements are reduced by measuring only half of the beams in each beam sweep. The half of the beams that are measured is selected in the following different ways:

- Measuring only the even-indexed beams in each complete beam sweep.

- Measuring only the odd-indexed beams in each complete beam sweep.

- Measuring the even-indexed beams in the one sweep and measuring the odd-indexed beams in the subsequent sweep (interleaved approach).

To predict the L3 measurements from the reduced set of L1 measurements we use the following methods:

- A random forest that takes as an input the reduced set of measurements from the current and previous beam sweep and predicts the unfiltered L3 measurements.

- A non-ML approach that estimates the unfiltered L3 measurements (labelled B in Figure 2.1-1) by taking the RSRP of the best beam among those that have been measured in the current and previous beam sweep (Take Max).

The output of those two methods is then filtered to give the predicted filtered L3 measurements (labelled C in Figure 2.1-1). The mean absolute error (MAE) between the predicted and ground truth filtered L3 measurements (RSRPs, in dB) is shown in Table 2.1.2-1.

Table 2.1.2-1: Evaluation results. The mean absolute error (in dB) between the predicted and ground truth L3 measurements (RSRP).

| Reduction Method | Random Forests | Take Max |
| --- | --- | --- |
| Even Indexed | 0.9782 | 1.7372 |
| Odd Indexed | 0.9351 | 1.5784 |
| Interleaved | 0.7581 | 0.5549 |

Our results show that interleaving the indices of the measured beams and then taking the maximum RSRP of those beams as the estimate of the L3 measurements (B) gives a lower mean absolute error than ML-based random forests. This is because interleaving the measurements increases the chance that the best beam will be included in the measured subset and hence gives an accurate estimate of the L3 measurements.

The above results show that the way in which the measured subset of beams is selected significantly affects the prediction KPIs (mean absolute error in this case). Therefore, it is important that the UE or the network identifies the optimal subset of beams to measure to achieve the desired trade-off between the number of saved measurements and the prediction error.

Observation 1: The way the measured subset of beams is selected can significantly affect the prediction error of the L3 measurements.

Proposal 1: The UE or the network identifies the optimal subset of beams to measure. RAN2 to discuss the criteria that can be used for the selection of the subset of the beams.

Additionally, since beam measurements are typically taken in time steps of tens of milliseconds by a UE with limited computational power, it is important to investigate the effect of using complex ML models on the inference delay and energy efficiency. A model that is too complex can easily negate the gains obtained by saving the measurement, either in terms of inference delay or energy saving. Also, the life-cycle management of more complex models is more challenging.

Proposal 2: When analysing the different beam prediction methods, aspects related to the computational complexity requirements and inference delays of the different methods should be considered.

Since the grid of beams has a specific spatial arrangement in network deployments, it is expected that the optimal subset of beams to measure will change over time as the UE moves in space and as the location of the best beam thus changes. Moreover, we observe that beams which are closer together spatially have more correlated measurements than those that are further apart spatially.

Observation 2: For spatial domain RRM measurement predictions, a beam’s RSRP is usually correlated with the RSRPs of a certain subset of beams.

Figure 2.1.2-1 Dynamic selection and optimization of the Set B of measured beams

Therefore, a dynamic selection of the optimal subset that adapts to the changing radio conditions can perform better than a fixed subset that is always constant over time. To evaluate this observation, we run the same simulation scenario as before, but where the subset of beams to measure dynamically changes based on the previous best beam and its most correlated beams, as shown in Figure 2.1.2-1. The predicted L3 measurements are derived from the RSRP of the best beam among those that are measured. Table 2.1.2-2 shows the results of this evaluation.

Table 2.1.2-2: Evaluation results for the dynamic selection of the subset of beams to measure

| Measurements Saved | 15% | 20% | 25% | 30% | 40% | 50% |
| --- | --- | --- | --- | --- | --- | --- |
| Mean Absolute Error (dB) | 0.033 | 0.059 | 0.071 | 0.091 | 0.220 | 0.290 |

Figure 2.1.2-2: CDF of the absolute error at 50% measurement savings for the three methods: random forests, take max, and optimal subset selection

Figure 2.1.2-2 compares the cumulative distribution functions (CDF) of the absolute error from the three measurement reduction methods described above (at 50% saving rate): random forests (interleaved), take max (interleaved), and optimal subset selection. Even though the mean absolute error ranges between 0.76 and 0.29 in these methods, we see from the CDFs that approximately 5% of the errors are above 2 dB. An error above 2dB may be unacceptable in some cases, and thus it suggests the importance of considering the distribution of the resulting errors and not just their mean.

Observation 3: The distribution of the prediction errors gives a better picture of the performance of the model than the average error alone.

Observation 4: Iterative selection/optimization of the set of measured beams provides higher accuracy without introducing complex models.

Proposal 3: Study methods for deriving the optimal subset of beams to be measured by the UE.

## 2.2	Intra-Cell Time Domain Prediction Aspects

Figure 2.2-1: Measurement reduction by predicting/extrapolating skipped measurement time steps from past measurements

In time-domain measurement reduction, measurement time steps are skipped completely to reduce the number of measurements a UE needs to take. The measurements of the skipped time steps are extrapolated from the past measurements, as show in Figure 2.2-1. This was evaluated by first predicting on beam measurements (A or A1) and then computing a cell level L3 measurement from the beam level measurements (Case 1) or predicting cell-level measurements based on cell-level measurements computed from the measured beams in the measured time steps (Case 2). The two evaluated cases are shown in Figure 2.2-2 with 50% measurement reduction, i.e., measuring every other timestep. The measured values are shown in arrows with solid lines, the predicted ones with dotted lines and the current measurements predicted by the model are highlighted in red.

Figure 2.2-2: Cases 1 and 2 in time domain RRM measurement prediction for measurement reduction

Proposal 4: For time domain measurement reduction, RAN2 to prioritize Case 1 and Case 2.

To establish a baseline for the intra-cell time domain measurement reduction, we evaluated time domain measurement reduction using sample and hold and linear regression. The evaluation scenario assumptions are shown in Table 2.2-1.

Table 2.2-1: Simulation scenario for the time domain measurement reduction baseline evaluation

| Reported simulation assumptions | UE trajectory option | Option 3 |
| --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |
|  | UE speed | 60 kmh |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 32 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | 50% |
| Data Size (Number of Samples) | Training | 370K |
|  | Testing | 120K |
| AI/ML model input/output | Model input | Case 1: A time series of L1 beam measurements Case 2: A time series of unfiltered cell-level measurements |
|  | Model output | Filtered cell-level L3 measurements |
| AI/ML model description | Model type | Sample and hold, linear regression |
|  | Model complexity in number of parameters | - |

Table 2.2-2: Evaluation results with sample and hold, mean absolute error in dB

| Measurements Saved | 25% | 50% | 75% |
| --- | --- | --- | --- |
| MAE (dB) | 0.50 | 0.79 | 1.36 |

Table 2.2-3: Evaluation results with optimized linear regression, mean absolute error in dB

| Measurements Saved | 25% | 50% | 75% |
| --- | --- | --- | --- |
| Case 1 MAE (dB) | 0.48 | 0.87 | 1.54 |
| Case 2 MAE (dB) | 0.47 | 0.86 | 1.52 |

Table 2.2-2 shows the evaluation results with sample and hold and Table 2.2-3 with linear regression. The cell-level filtered Layer 3 measurement based on reduced measurement time steps is compared against a cell-level filtered Layer 3 measurement based on all measurements. The mean absolute error (MAE) is computed over all measurement steps, including measured and predicted steps, after the Layer 3 filtering. In case of sample and hold, both cases 1 and 2 have identical results, since considering the beam consolidation it doesn’t matter at which point the imputation is done. In our evaluation, linear regression was performing slightly better with lower measurement reduction rates, but with higher rates sample and hold was more accurate. In our evaluation, there was very little difference between Case 1 and 2 in linear regression.  The CDF of the prediction error is shown in Figure 2.2-3.

Figure 2.2-3: The CDF of sample and hold and linear regression Case 1

Compared to the spatial measurement reduction, although the predicted measurements have a higher error than in spatial prediction, averaged over all measurements the time domain prediction-based reduction has a comparable accuracy. System-level simulations may be required to evaluate the impact of different prediction error distributions on mobility performance.

Observation 5: Compared to the measurement reduction based on spatial measurement prediction, time domain measurement prediction has a different prediction error distribution.

Observation 6:  System-level evaluation may be required to understand the impact of the different prediction error distributions on the mobility performance.

Proposal 5: System level simulation studies to be done to compare the mobility performance impact of different measurement reduction methods.

## 2.3	Inter-Cell, Inter-Frequency Measurement Prediction

Figure 2.3-1: A HetNet scenario

A UE needs measurement gaps for frequencies (e.g. inter-frequency, inter-RAT) which cannot be measured while the UE is transmitting or receiving to/from the serving cell. The measurements gaps are configured by the serving cell as part of the measurement configuration that is provided to the UE by means of RRC Reconfiguration message. During the measurement gaps, the UE cannot receive/or send on downlink/uplink shared channels of the serving cell and as such they can have a negative impact on the data rate of the UE.

Observation 7: Inter-frequency measurements are costly due to the required measurement gaps.

AI/ML can be used to reduce the number of costly inter-frequency measurements that a UE needs to perform. As an example, Figure 2.3-1 shows a HetNet scenario, where a macro cell layer B provides coverage and micro cell layer A provides additional capacity but not full coverage. Connecting to layer A is preferred whenever possible, for improved network performance and for better load balancing. This may include either dual connectivity or handover between the layers. In the baseline scenario, when connected to the macro layer B, in addition to making measurements in the layer B, the UE must also measure the micro layer A to determine if it would be in the coverage of a micro cell.

However, with measurement data from both layers, we can train an ML model that learns the correlation between the radio measurements in the two layers and can based on layer B measurements predict layer A measurements or if a UE connected to the macro layer B would be able to connect to a micro layer A, as shown in Figure 2.3-2. This may require a cluster approach, where measurement from more than one cell in layer B are used as input to the prediction. A time series of layer B measurements may also be used as input. The model may predict the measurement for a selected cell or selected cell(s) in Layer A, or the measurement of the strongest cell in Layer A. In this inter-frequency use case, we start with Case 2, i.e., using cell-level measurements in ML input and output.

Figure 2.3-2: Inter-frequency measurement prediction model

To evaluate the above concept, we set up a simulation scenario as in Table 2.1.1-1 with two frequency layers (2 GHz and 4 GHz, FR1-FR1) and collected measurements from both frequencies to train a machine learning model. The input is a time series of the RSRP measurements of the serving cell in the first frequency (2 GHz) and the output is the RSRP of the co-located cell of the second frequency (4 GHz). This was compared to the baseline of finding the average RSRP difference between the two frequencies and adding that difference to the RSRP measurements of the higher frequency.

The mean absolute error between the predicted RSRPs and the ground truth is shown in Table 2.3-1, where we summarize the results of our preliminary evaluation. We can achieve reasonable prediction error using linear regression, although it’s not significantly better than the mean shift baseline described above. Further evaluations are still needed, including using the cluster approach where we take the measurements of the neighbouring cells as an additional input.

Table 2.3-1: Preliminary evaluation results for inter-frequency measurement prediction

| Method | Mean Absolute Error |
| --- | --- |
| Linear Regression | 1.47 |
| Mean Shift | 1.56 |

Observation 8: A machine learning model may be trained to predict measurements on frequency layer A based on measurements from frequency layer B. This may be used to reduce the number of inter-frequency measurements a UE needs to take.

Proposal 6: In inter-frequency case, start with Case 2, i.e., using cell-level measurements.

Figure 2.3-3: Frequency domain measurement event prediction

The ML model can also be used in frequency-domain measurement event prediction, as shown in Figure 2.3-3. Based on measurements in frequency B, a machine learning model is predicting if a measurement event condition A4, A5 or B1 in frequency A is fulfilled. For example, a machine learning approach is trained to predict, if the signal strength of the strongest cell in set A of cells satisfies a specified measurement event condition based on measurements of cells or beams in Set B. The measurement event condition may be as in A4 or B1 measurement events. The required inter-frequency measurements can be reduced by configuring them only when an inter-frequency measurement event condition is predicted to be fulfilled.

Observation 9: Inter-frequency measurement event prediction can be used to reduce the number of inter-frequency measurements a UE needs to take.

Based on the predicted inter-frequency measurement event, the UE may be, for example, configured to measure the other frequency layer including configuring the required measurement gaps. Costly inter-frequency measurements are saved by taking them only when the model is already predicting an inter-frequency measurement event based on intra-frequency measurements, indicating that a connection is likely possible. Since any connection or handover is determined based on actual measurements, very high prediction accuracy is less critical

Observation 10: Predicted inter-frequency measurement events can be used to decide, when to configure the UE to measure the other frequency layer including configuring the required measurement gaps.

Proposal 7: RAN2 to study inter-frequency measurement reduction based on inter-frequency measurement event prediction.

# 3	Conclusion

This document has made the following observations:

Observation 1: The way the measured subset of beams is selected can significantly affect the prediction accuracy of the L3 measurements.

Observation 2: For spatial domain RRM measurement predictions, a beam’s RSRP is usually correlated with the RSRPs of a certain subset of beams.

Observation 3: The distribution of the prediction errors gives a better picture of the performance of the model than the average error alone.

Observation 4: Iterative selection/optimization of the set of measured beams provides higher accuracy without introducing complex models.

Observation 5: Compared to the measurement reduction based on spatial measurement prediction, time domain measurement prediction has a different prediction error distribution.

Observation 6:  System-level evaluation may be required to understand the impact of the different prediction error distributions on the mobility performance.

Observation 7: Inter-frequency measurements are costly due to the required measurement gaps.

Observation 8: A machine learning model may be trained to predict measurements on frequency layer A based on measurements from frequency layer B. This may be used to reduce the number of inter-frequency measurements a UE needs to take.

Observation 9: Inter-frequency measurement event prediction can be used to reduce the number of inter-frequency measurements a UE needs to take.

Observation 10: Predicted inter-frequency measurement events can be used to decide, when to configure the UE to measure the other frequency layer including configuring the required measurement gaps.

And proposed the following:

Proposal 1: The UE or the network identifies the optimal subset of beams to measure. RAN2 to discuss the criteria that can be used for the selection of the subset of the beams.

Proposal 2: When analysing the different beam prediction methods, aspects related to the computational complexity requirements and inference delays of the different methods should be considered.

Proposal 3: Study methods for deriving the optimal subset of beams to be measured by the UE.

Proposal 4: For time domain measurement reduction, RAN2 to prioritize Case 1 and Case 2.

Proposal 5: System level simulation studies to be done to compare the mobility performance impact of different measurement reduction methods.

Proposal 6: In inter-frequency case, start with Case 2, i.e., using cell-level measurements.

Proposal 7: RAN2 to study inter-frequency measurement reduction based on inter-frequency measurement event prediction.

# 4	References

[1] R2-2405941, Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO), RAN2#126, Fukuoka Japan, May, 2024.