3GPP TSG-RAN WG2 Meeting #127	R2-2407480

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.3.2.2

Source:	Nokia

Title:	Mobility Optimization based on RRM Measurement Prediction

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#125bis the following sub-use cases were agreed for RRM measurement prediction.

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results.

Based on the e-mail discussion [1], the two main identified objectives are:

- Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency and reduces the required signalling.

- Mobility optimization. Predict future UE DL radio measurements and use them to optimize mobility decisions.

Moreover, in RAN2#126, we agreed to prioritize the following sub-use cases.

| Case number | Prioritization | Evaluation scenario combination | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we discuss our views on the mobility optimization goal and highlight the need for performing system level simulations in order to obtain a holistic view on the impacts of the RRM measurement predictions into the optimization of the mobility procedures.

# 2	Discussion

## 2.1	Mobility optimization goal

Among the identified goals for the SI is also the improvement of the handover performance. As was already discussed in RAN2#126, the FR1-to-FR1 handover procedures are already highly optimized, and it would be difficult to obtain significant gains. On the other hand, for FR2-to-FR2 handover procedures, additional benefits associated with using AIML to enhance the performance of such procedures are expected. As an example, note that in FR2 the ISD is relatively smaller than FR1. For the NW to be able to make efficient handover decisions, it needs to receive measurements from the UE, which would need to measure a larger number of beams when compared to FR1.

RAN2 agreed to study with high priority the FR2-to-FR2 intra-frequency temporal domain case A measurement prediction for the mobility optimization goal. Nevertheless, we think that at least the following issues need to be further clarified:

- The expected use of the predicted future measurements for further mobility procedure optimization.

- The coupling of the predicted future measurements with certain mobility procedures.

- The KPIs that are relevant with respect to the mobility optimization goal.

Regarding the first point we understand the predicted measurements may be used by UEs to at least predict HOFs/RLFs  or measurement events, which can be then signalled to the NW. It should be clarified and discussed in RAN2, if additional use cases for using predicted future measurements for mobility optimization are foreseen.

Observation 1: At least for the cases of the HOF/RLF prediction and the measurement event prediction the exact optimization decisions and actions taken based on the predictions should be studied.

Regarding the second point, we understand that the predicted future measurements may be used for different purposes in the context of mobility. For example, in both the measurement event prediction and failure prediction use-cases RAN2 has discussed the indirect prediction procedures, wherein the same predicted measurements may be utilized in predicting failures and/or measurement events. The required configuration from the network to the UE should be discussed in such scenarios and what part of the configuration could be shared with the two use cases.

Proposal 1: RAN2 to discuss the required configuration from the network to the UE, when predicted future measurements are used in failure and measurement event prediction.

Regarding the KPIs that would be relevant in considering the mobility optimization goal, we think that considering both AIML-related metrics and system-level metrics would provide a holistic view of the improvements and trade-offs in the context of this goal. The system-level gains would depend on the ML metrics and on the decision logic and the actions made based on the ML predictions. The required ML accuracy and robustness depend on how the ML predictions are used.

Observation 2: The ML performance metrics provide a starting point. To evaluate the specific objectives and trade-offs in the context of the mobility optimization goal, the impact in mobility metrics should be evaluated.

Observation 3: The required ML accuracy and robustness depend on how the ML predictions are used.

While the selection of KPIs should be tailored to each use case and the applied ML approach, we still recommend considering a set of common KPIs for the mobility performance (for inference). Therefore, we propose the following:

Proposal 2: RAN2 should consider at least a selection of the following common mobility related KPIs as discussed in TR 36.839 as a starting point for the metrics evaluated in each use case:

- HO performance: total number of HOs, unnecessary HOs, Ping-Pong HOs, etc.

- Failure performance: total number of HO failure, RLF, percentage of failure (RLF+HOF)/(HO+ RLF+HOF) in %, etc.

- HO timing performance: HO interruption time, Time of outage (ToO), time of staying (in a cell, in a beam, in a UE panel), etc.

- Measurement performance: measurement reduction, measurement accuracy (absolute and relative), CDF of measurement difference, etc.

- QoS related KPIs such as downlink/uplink throughput, latency

## 2.2	System level simulations

It is difficult to conclude about the benefits of measurement reduction on mobility optimization without considering their impact and understanding their implications on the mobility metrics/KPIs and the entire system. For example, in the case of measurement reduction, less measurements will essentially result in the increased errors in the reported values.  It is crucial to understand how much error and what distribution in the predicted measurements can be tolerated without leading to a significant reduction in mobility performance. Similarly for RLF prediction, what actions can be taken based on predicted RLFs and what accuracy and compromise between true and false positive rates is required for each of these actions?

To properly evaluate this, it would be necessary to model the impacts of applying the predictions or actions taken based on the predictions to the handover procedure as well as the RLF and re-establishment procedures to evaluate the impact observed on the agreed mobility metrics. For example, in case of RLF prediction, the cost of actions taken to prevent or mitigate the predicted RLFs (including false actions taken based on false positive predictions) must be evaluated against the gains from prevented or mitigated RLFs. Further metrics may need to be agreed to evaluate the system-level scenarios. For example, since the length of the respective interruption times influences the degree to which a UE experiences a degradation in throughput, it would also be helpful to incorporate some type of traffic model at a later stage of the study.

Another example could relate to the fact that well-predicted measurements or HO can match well instantaneous channel conditions. However, due to such predictions, the frequency of HOs per UE could increase. Since HOs cause interruptions in data transmission, it could be that on the system level such frequent HOs will cause longer interruptions, i.e., decreased overall user experience. Similarly in the case of RLF prediction, it depends how this information is utilized and the potential reduction in RLFs needs to be also evaluated against any trade-offs, for example if number handovers is significantly increased. Furthermore, depending on which random access procedures are considered (2-step, 4-step; CBRA, CFRA) for different scenarios, i.e. HO vs RRC re-establishment vs beam failure recovery (if modelled), could influence the results.

Observation 4: System-level simulations will justify the tolerance margins related to the amount and distribution of the errors and will be used to conclude about real system gains associated with AIML predictions.

Proposal 3: RAN2 should discuss incorporating system-level simulations into the study to better understand what actions should be taken by the network in response to predictions, as well as the effects that prediction errors and the distribution of predictions have on the system KPIs.

These questions impact not only the results of the evaluations performed by RAN2 but would also aid the RAN4 work in defining various test cases, which may consider the interplay between existing RRM requirements and  new mechanisms considered in the RAN2 study. One new challenge when it comes to defining the requirements and test cases relates to the uncertainty associated with prediction data, where UEs may report measurement or RLF predictions in a probabilistic manner (i.e. certain confidence) with an associated prediction window. Another potential challenge is that mobility metrics/KPIs and corresponding gains should account for the latencies associated with decisions and procedures. These latencies are defined by RAN4 Core requirements. Therefore, it is critical to support the RAN4 discussions early enough with well-informed, holistic evaluations based on system-level simulations rather than only relying on offline evaluation of ML performance metrics.

Observation 5: System-level simulations would also support RAN4 when defining the RRM requirements and test cases associated with any new mechanisms considered by RAN2.

# 3	Conclusion

This document has made the following observations:

Observation 1: At least for the cases of the HOF/RLF prediction and the measurement event prediction the exact optimization decisions and actions taken based on the predictions should be studied.

Observation 2: The ML performance metrics provide a starting point. To evaluate the specific objectives and trade-offs in the context of the mobility optimization goal, the impact in mobility metrics should be evaluated.

Observation 3: The required ML accuracy and robustness depend on how the ML predictions are used.

Observation 4: System-level simulations will justify the tolerance margins related to the amount and distribution of the errors and will be used to conclude about real system gains associated with AIML predictions.

Observation 5: System-level simulations would also support RAN4 when defining the RRM requirements and test cases associated with any new mechanisms considered by RAN2.

And proposed the following:

Proposal 1: RAN2 to discuss the required configuration from the network to the UE, when predicted future measurements are used in failure and measurement event prediction.

Proposal 2: RAN2 should consider at least a selection of the following common mobility related KPIs as discussed in TR 36.839 as a starting point for the metrics evaluated in each use case:

- HO performance: total number of HOs, unnecessary HOs, Ping-Pong HOs, etc.

- Failure performance: total number of HO failure, RLF, percentage of failure (RLF+HOF)/(HO+ RLF+HOF) in %, etc.

- HO timing performance: HO interruption time, Time of outage (ToO), time of staying (in a cell, in a beam, in a UE panel), etc.

- Measurement performance: measurement reduction, measurement accuracy (absolute and relative), CDF of measurement difference, etc.

- QoS related KPIs such as downlink/uplink throughput, latency

Proposal 3: RAN2 should discuss incorporating system-level simulations into the study to better understand what actions should be taken by the network in response to predictions, as well as the effects that prediction errors and the distribution of predictions have on the system KPIs.