3GPP TSG-RAN WG2 Meeting #127	R2-xxxxxx

Maastricht , Netherlands , 19 August – 23 August 2024

Agenda item:	8.6.2

Source:	CEWiT

Title:	LTM enhancements for Inter-CU mobility

WID/SID:	NR_Mob_Ph4-Core;- Release 19

Document for:	Discussion and Decision

# Introduction

Given below are the agreements in RAN2 #125 and RAN2 #126:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. - Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. - UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells. - Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM. - Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. - PDCCH ordered early RACH is supported for inter-CU LTM. - For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option. - Upon inter-CU LTM execution, UE performs - MAC reset - RLC re-establishment - PDCP re-establishment - Security key update - FFS if there is an inter-CU LTM w/o security key change. - L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

# 2	Inter-CU LTM

In order to support intra-CU and Inter-CU handovers seamlessly in Release 19, we must come up with configurations which keep in mind the existing provisions in intra-CU handover from Release 18. We are discussing every phase and associated changes that are required in inter-CU LTM.

## 2.1	Handover Preparation

Candidate Configuration Preparation

While the source gNB-CU only required information from its own DUs in intra-CU LTM, inter-CU LTM the CUs will have to be in sync. This requires Xn coordination between the CUs. Source CU will ask candidate CUs to perform admission control using the existing HO Request message with an LTM indication. The remaining IEs that exist in the message would remain intact. The response would contain all the lower layer configurations for the requested CGIs. This is further consolidated at the source CU to form a common CSI resource configuration. This along with the candidate mapping will be sent to the candidate-CUs. This is keeping in mind the subsequent LTM. Keeping this as the reference, the each candidate CU would send back a delta configuration which the UE applies when that candidate becomes the new target.

Proposal 1 : Handover Request to be sent from Source CU to Candidate CUs with an indication that it is for an LTM handover

Proposal 2 : Candidate-CU has to collect and consolidate the lower layer configuration from associated DUs using the existing Rel-18 intra-CU procedures

Proposal 3 :  Source-CU has to assign unique candidate IDs to each candidate cell

Proposal 4 : The consolidated union of candidate and resource configurations are sent back to each candidate CUs in order to be used for subsequent LTM.

Proposal 5 : The candidates send back the delta candidate configurations which is to be applied upon attaching to that candidate as a part of subsequent LTM.

Proposal 6 : Source DU includes both reference configuration and delta configuration per candidate  cell in the RRC Reconfiguration. This forms the complete configuration to the UE based on its instantaneous serving cell.

## 2.2	Early synchronization

From a UE perspective, Early synchronization is quite similar to the existing procedures in Rel-18 intra-CU LTM. The TA transfer was done from DU to CU via F1AP interface. For inter-CU LTM, it is expected to come from candidate CU to source CU. Xn interface must support this. However, we further need to provide other information such as C-RNTI and NCC. These can be provided in a message RACH information transfer message to the source CU. This needs to be sent as an LS for RAN3 to discuss and confirm. In case of subsequent LTM, the L1 measurement and reporting configurations would be maintained at the UE (ie. the configurations which were set by the last received RRC Reconfiguration)

TA acquisition

Proposal  7 :  Candidate-CU to calculate and send TA information, C-RNTI and NCC value that the UE must use in RACH information transfer message (RAN3 to decide on the actual message to be used)  over Xn interface after receiving the preamble from the UE (based on the PDCCH order received by the UE)

Proposal  8 : Similar to TA information being transferred in the F1AP interface, new C-RNTI and NCC must also be facilitated. RAN3 to look into this.

L1 measurement reporting

Proposal 9: UE to report L1 measurements to the subsequent targets according to the CSI report configuration received in the previous RRC Reconfiguration, for subsequent LTM.

## 2.3	Handover Execution

In this phase, based on LTM L1 measurement reports the source-CU decides on the target. The target and the TA (if needed) will be intimated in the same way as it is done in Release 18 intra-CU LTM (is. Via Cell Switch MAC CE). But in inter-CU LTM source needs to inform C-RNTI and NCC must be sent in LTM cell switch command MAC CE when he target is decided  at the source-CU. Once the LTM cell switch command MAC CE is received, if the UE has a valid TA, it will proceed to sending the first UL packet to the target. This marks the successful completion of RACH-less LTM, similar to Rel-18 procedure. When the UE does not have a valid TA, UE will have to perform RACH-based LTM, similar to Rel-18 procedure.

Proposal 10 :  New C-RNTI, if any, must come in cell switch command MAC CE. FFS - when C-RNTI is assumed to be maintained.

Security Key Derivation

Proposal 11: For inter-CU, Security key (K-gnB*) derivation method has to be indicated to UE in MAC CE; based on the NCC value being changed or unchanged from the previous handover, H/V derivation can be applied. FFS - when there is no security key change, eg. Intra-CU LTM.

Handover Cancel to other candidates

Proposal 12:  Source-CU sends Handover Cancel to all the Candidate-CUs after the completion of subsequent LTM duration

## 2.4 Stage-2 Flow Diagram

# 3	Conclusion

This document has made the following proposals:

Proposal 1 : Handover Request to be sent from Source CU to Candidate CUs with an indication that it is for an LTM handover

Proposal 2 : Candidate-CU has to collect and consolidate the lower layer configuration from associated DUs using the existing Rel-18 intra-CU procedures

Proposal 3 :  Source-CU has to assign unique candidate IDs to each candidate cell

Proposal 4 : The consolidated union of candidate and resource configurations are sent back to each candidate CUs in order to be used for subsequent LTM.

Proposal 5 : The candidates send back the delta candidate configurations which is to be applied upon attaching to that candidate as a part of subsequent LTM.

Proposal 6 : Source DU includes both reference configuration and delta configuration per candidate  cell in the RRC Reconfiguration. This forms the complete configuration to the UE based on its instantaneous serving cell.

Proposal  7 :  Candidate-CU to calculate and send TA information, C-RNTI and NCC value that the UE must use in RACH information transfer message (RAN3 to decide on the actual message to be used)  over Xn interface after receiving the preamble from the UE (based on the PDCCH order received by the UE)

Proposal  8 : Similar to TA information being transferred in the F1AP interface, new C-RNTI and NCC must also be facilitated. RAN3 to look into this.

Proposal 9: UE to report L1 measurements to the subsequent targets according to the CSI report configuration received in the previous RRC Reconfiguration, for subsequent LTM.

Proposal 10 :  New C-RNTI, if any, must come in cell switch command MAC CE. FFS - when C-RNTI is assumed to be maintained.

Proposal 11: For inter-CU, Security key (K-gnB*) derivation method has to be indicated to UE in MAC CE; based on the NCC value being changed or unchanged from the previous handover, H/V derivation can be applied. FFS - when there is no security key change, eg. Intra-CU LTM.

Proposal 12:  Source-CU sends Handover Cancel to all the Candidate-CUs after the completion of subsequent LTM duration.

# References

[1] Revised Work Item: NR mobility enhancements Phase 4. RP-240299, Apple Inc, China Telecom

[2] R2-2405062, Discussion on inter-CU LTM, ZTE Corporation

[2] LTM enhancements for Inter-CU mobility, R2-2403238, CEWiT

[3] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[4] A Comprehensive Formal Analysis of 5G Handover; Aleksi Peltonen et.al.