3GPP TSG RAN WG2 #127														           R2-2407484

Maastricht, Netherlands, August 19th -23rd , 2024

Agenda item:	         8.3.2.1

Source:	CEWiT

Title:	Initial Simulation results for RRM Measurement Prediction

Document for:	     Discussion and Decision

# Introduction

In the SID on AI/ML Mobility , it was agreed to study the simulation parameters and AI/ML methods for different sub use cases. Additionally, in the RAN1#126[1] meeting, more agreements were reached on defining the AI/ML terminologies, frameworks, use cases, evaluation methodologies, KPI’s etc.

Agreements Agreements on simulation assumptions 1 For 2nd study goal i.e. to enhance handover performance, evaluation exercise will focus on FR2 to FR2 intra-frequency scenario. 2 For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases 4 for RRM sub case 1, it is up to company to report L1 RSRP difference 5 Definition of measurement reduction for intra-frequency scenario is defined as: Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured 6 For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation. 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window. 8 Intra-frequency intra-cell temporal domain prediction can be applied for all RRM sub cases. And it is up to company to report applied RRM sub case together with simulation result. 9 For both Intra-frequency and inter-frequency inter-cell prediction, the measurement on cell for measurement should not be reduced in both temporal and spatial domain 10 No traffic model is simulated 11 During simulation UE is dropped 100% outdoor 12 It is up to company’s implementation to select how to distribute the UE 13 To agree not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8) and blockage (7.6.4) for channel modelling (38.901) 14 LOSsoft is optionally modeled in the channel modeling 15 To agree on following parameters for FR2: 1, 30GHz as central frequency 2, 200m as ISD 3, UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 as baseline channel modelling 16 Simulation assumptions discussed in section 2.3.1~2.3.3 is taken as baseline also for use cases other than RRM measurement prediction. Any update is subject to further discussion on other use cases. - Simulation assumptions Agreements 1 average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged 2: Measurement reduction rates e.g. 1/2, for both intra-frequency intra-cell temporal domain prediction case B and spatial domain prediction. Revision in RAN2#127 is open. 3: One prediction window for FR1 and FR2 respectively as starting point. The detail value to be decided in the post email discussion. 4: UE trajectory option is up to company’s implementation and report 5: UE trajectory boundary processing is up to company’s implementation and report 6: For study goal 2, the candidate speeds are 60,90,120 km/h and company can report UE speed along with simulation result 7: For study goal 1, the candidate speeds are 30,60,90km/h and company can report UE speed along with simulation result 8: To decide on the values in table 2.4-2 in post email discussion. Table 2.4-2 Measurement period; Recommended value. FR1 to FR1 intra-frequency w.o. gap; 200ms. FR1 to FR1 inter-frequency with gap; 120ms. FR2 to FR2 intra-frequency w.o. gap; 480ms. 9: To agree on 80MHz as bandwidth for FR2 10: To agree on 10m for BS antenna height for FR2 11: Fast fading is necessary for RRM sub case 2 too. 12: The shadowing fading of two different frequency layers can be reported by company 13: To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain. Scenario priority Agreements: Case number; Prioritization; Evaluation scenario combination; target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell.

In this contribution we present our initial simulation results for RRM Measurement prediction in temporal domain and our views on the evaluation methodologies and KPI’s.

# 2. Discussion

In this document, we will present our results for the temporal domain, covering Case A  for Sub Use Case 1.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

## 2.1 Evaluation Methodology

In general, machine learning process can be considered to have four steps, i.e., dataset generation, data 				pre-processing, AI/ML model training and AI/ML model testing evaluation. In the first step, a dataset 					containing beam specific information like beam RSRP measurements needs to be generated using an 					appropriate method.

The dataset should include the inputs for the AI/ML model and the output labels corresponding to the output that the AI/ML should predict. Before the dataset is used for any purpose, it needs to be split into 3 parts which is then used for the AI.ML model training, validation, and testing. The second step is data per-processing which ensures that the input to the AI/ML is in the proper range. One of the most common techniques used in the data per-processing step is data normalization which converts the data which is in a varied range to fit in some know range such as [-1, 1] or [0, 1]. In the third step, the AI/ML model is trained using the training dataset and at each training epoch, the performance of the models is evaluated using the validation dataset. Once the model is trained, the test dataset is used to determine the performance of the model. The AI/ML model is properly trained if the testing performance is within the acceptable limits.

Case 4 - FR2 to FR2 intra-frequency temporal domain case A

Sub Case 1- To predict beam level results, then generate cell level results based on the predicted beam

results

In Case A, measurement result in future of one cell e.g., cell A is predicted based on historical ones of the

same cell A.

Fig : Intra-cell temporal domain prediction – case A[2]

2.2 Evaluation assumptions

In our simulations we followed the simulation agreements that were agreed in the RAN2#126 meeting. The detailed simulation assumptions are given in the Appendix. According to the simulation assumptions, the gNB is equipped with a four panel which has 64 antennas The gNB antenna array supports 16 transmitting beams using 8 beams in horizontal and 2 beams in vertical. So, in total there are 16 possible transmit-beams, 10 users in each sector. Measurement period as 480ms and sample period 20ms.

The dataset was generated using system level simulations based on the simulation parameters given in the appendix. Out of the around  80640 datasets generated using SLS, 80% of data points are used for training, 10% for validation and 10% for testing for each sector. Since the RSRP values can be quite varied, we normalize the RSRP values before giving it as input to the AI/ML model.

We modeled the ML model as neural network with three fully connected LSTM layer and one dense layer in output. The activation function used is linear. The DNN was trained for a maximum of 150 epochs with a batch size of 8. The lost function used was MSE(mean square error) and an Adam optimizer was used to minimize the loss.

In the subsequent discussion simulated results are given.

2.3 Simulation results

The data is preprocessed with an observation window of 20 instances and a prediction window of 4 instances, the CDF plot reflects how well the model predicts the RSRP differences based on those 20 observations.

The input to the model is a multi-dimensional tensor of shape (#Users, #Measured_Timesinstances, #TX_beams) and the output of the model is (#Predicted_Timeinstances, #TX_beams)

Fig : CDF of average L1 RSRP difference

The x-axis represents the average RSRP difference in dBm. Negative values indicate that the predicted slots had lower RSRP than the actual measurements, while positive values suggest higher predicted RSRP.

Observations :

Observation 1. Some beams show tighter CDF curves, indicating more consistent prediction accuracy, while others have wider spreads, suggesting variability in prediction performance.

Observation 2. The CDF curves for different beams show varying characteristics, suggesting that the RSRP difference behavior is not uniform across beams.

Observation 3. The curves that cross the zero RSRP difference point represent beams where the predicted slots resulted in no average difference in RSRP, meaning the predictions were accurate in those cases.

# 3. Conclusion

Observation 1. Some beams show tighter CDF curves, indicating more consistent prediction accuracy, while others have wider spreads, suggesting variability in prediction performance.

Observation 2. The CDF curves for different beams show varying characteristics, suggesting that the RSRP difference behavior is not uniform across beams.

Observation 3. The curves that cross the zero RSRP difference point represent beams where the predicted slots resulted in no average difference in RSRP, meaning the predictions were accurate in those cases.

# References

- [1] R2-24xxxxx Summary of [POST126][031][AIMob] Simulations (OPPO)_v21_Interdigital, RAN #126

- [2]3GPP TR 38.843 V18.0.0

# Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | 60km/h |
| UE distribution | 100% outdoors |
| BS Antenna Configuration | [One panel: ( Mg, Ng, M, N, P,) = (1,1,4, 8, 2), (dV, dH) = (0.5, 0.5)λ as baseline] Azimuth angles (degrees) = [-78.75, -56.25, -33.75, -11.25, 11.25, 33.75, 56.25, 78.75] Zenith angles (degrees) = [112.5, 157.75] |
| BS Tx Power | 55 dBm |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5 m |