3GPP TSG-RAN WG2 Meeting #127                                           	R2-2407487

Maastricht, 19.-23.08.2024

Source:	Telit Communications S.p.A., Novamint , Sateliot , Thales

Title:	Further considerations on S&F operation from device perspective

Agenda Item:	8.9.2

Document for:	Discussion and Decision

# Introduction

The present document is a revision of R2-2404979. It was updated to reflect the actual discussion progress within RAN2 and also parts from SA2 discussions where relevant are additionally reflected and providing some more justification examples for the various proposals. However, the proposals itself have been kept according to their core part.

In the last RAN2 meeting following agreements were made related to S&F Satellite operation.

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Even though in many areas RAN2 needs to wait until discussion in other groups has further progressed, some important decisions were already made especially by SA2, which are reflected hereafter:

Extract of SA2 [2] conclusions on KI#2 (Store and Forward):

1)	When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available then eNB switches off and does not broadcast any signal.

……

7) The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

# Discussion

S&F will be an important feature for delay tolerant traffic especially at the beginning for the deployments with lower number of satellites and a limited number of ground stations. As a satellite constellation and associated ground station infrastructure deployment evolves, or also depending on satellite position within an orbit (whether feeder-link is in reach or not) or on operator’s policy on how to operate the satellite, a given satellite will likely support both transfer methods, namely, S&F satellite operation mode and real time (RT) mode, and be able to switch between the two modes as appropriate throughout the orbit time (i.e. a satellite can be able to operate in RT mode in a geographical area where the satellite can have a feeder link while the same satellite will switch to S&F mode in other parts of the orbit for which no feeder link is available).

It is important for the UE to know, and especially the application on the UE, which of the two modes, i.e. S&F mode or RT mode, is available at a given time.

The knowledge on S&F is important also for applications on the device, e.g. whether to use TCP/IP or UDP protocol for the transfer.

Proposal 1: UE should receive an indication whether S&F satellite operation is activated in an NTN cell or not.

Proposal 2: If the S&F satellite operation mode is indicated to be as not activated, or no S&F satellite operation mode indication is provided (e.g. in a pre-Rel-19 NTN cell), the UE should assume that the NTN cell is operating in RT mode (i.e. default/normal mode).

Proposal 3: Given that a pre-Rel-19 UE will not be able to decode the above proposed S&F indicator, it needs to be discussed whether and, in case, how pre-Rel-19 UEs could be barred from accessing to an NTN cell operating in S&F mode upon operator policy.

It needs to be discussed whether this could also be done with the existing cellBarred and cellBarredNTN. (please note that the Satellite community do not believe we should use cellBarred or cellBarredNTN for this purpose)

S&F is especially suited for delay-tolerant traffic, however additional indication on expected delivery delay would be important for the device.

Delivery delay is at least the time it requires to reach the next feeder link connection opportunity but may even be larger if not all data can be offloaded during visibility of the feeder link from the satellite. Hence an upper boundary indication of S&F expected delivery time is important in general for the device.

The UE could also use the expected delivery time to evaluate whether the data transfer service using S&F mode is suitable or not for the service needs.

Proposal 4: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

In case both methods are made simultaneously available in an NTN cell (e.g. in the case of a satellite with an established a feeder link but still using the S&F mode to serve specific connections/users based on operator’s policies), the NTN cell should be able to indicate the UEs that the two modes are available and the UE should be able to decide / indicate to the network the preferred transfer method for the provided data.

Finally, it is still up to the satellite network e.g. in case that S&F was indicated as sufficient to perform an earlier delivery than indicated by e.g. using RT traffic or forwarding via ISL. Network decision may depend on remaining storage capabilities but no need to specify as it is up to network when indicated maximum delivery time is still met.

As depending on satellite position RT traffic or depending on exceeding capabilities also unavailability of the respective methods should be possible to indicate.

Proposal 5: Satellite should be able to indicate (broadcast) the duration during which  S&F or RT operation are respectively available or unavailable.

# Conclusion

Proposal 1: UE should receive an indication whether S&F satellite operation is activated in an NTN cell or not.

Proposal 2: If the S&F satellite operation mode is indicated to be as not activated, or no S&F satellite operation mode indication is provided (e.g. in a pre-Rel-19 NTN cell), the UE should assume that the NTN cell is operating in RT mode (i.e. default/normal mode).

Proposal 3: Given that a pre-Rel-19 UE will not be able to decode the above proposed S&F indicator, it needs to be discussed whether and, in case, how pre-Rel-19 UEs could be barred from accessing to an NTN cell operating in S&F mode upon operator policy.

Proposal 4: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

Proposal 5: Satellite should be able to indicate (broadcast) the duration during which  S&F or RT operation are respectively available or unavailable.

# Reference

[1] R2-2403733 Report from Break-Out Session on NR NTN and IoT NTN

[2] TR 23.700-29 v1.0.0 [FS_5GSAT_Ph3_Arch]