3GPP TSG RAN WG2 Meeting #127	 R2-2407488

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda Item:	8.1.2.2

Source:	Futurewei

Title:	Discussion on LCM for UE-Side Models (Beam Management)

Document for:	Discussion

# Introduction

During RAN2 meeting #126, functionality-based LCM for UE-side models was discussed and some agreements have been achieved [1]. The discussion was focused on the following topics.

- Terminology alignment

- Availability/applicability of Functionality

- Network-side additional conditions

- UE side additional/applicability conditions

The discussions resulted in the following agreements.

Agreements

- RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.

  - FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

- We will work offline on the definitions for functionality types and define what is availability.

- The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

- For NW-side additional conditions, RAN2 assumes that RRC signalling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning.

- For BM use case, as a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive.

- The Chair also arranged a post-meeting discussion on the definitions of various terminologies and the reactive/proactive framework for applicable functionality.

[POST126][032][AI/ML PHY] LCM (Intel/Samsung)

Intended outcome:

Phase 1: Agreeable definitions (Samsung)

Phase 2: Reach common understanding of reactive/proactive framework for applicable functionality.

Deadline:  long

The Moderator proposed the following as the discussion results of Phase 1, which focused on the definitions of terminologies.

- Proposal 1: supported functionalities refer to functionalities that UE can indicate by using UE capability signalling.

- Proposal 2: RAN2 discuss further whether definition on configured functionalities is needed.

- Proposal 3: Applicable functionalities refer to functionalities that the UE is ready to apply for model inference.

- Proposal 4: RAN2 discuss further relationship between configured functionalities and applicable functionalities as a part of discussion on the need of defining configured functionalities.

- Proposal 5: Activated functionalities refer to functionalities already activated and performing inference.

- Proposal 6: RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

Proposals came out from the discussion of Phase 2 are listed below, which focused on the signalling framework for applicable functionality reporting.

- Proposal 1:

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

- Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE.

- Proposal 4: “Step 4”: UE reports applicable functionality in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

- Proposal 5: Step 5:

1) Network configures full inference configuration to UE after applicable functionality reporting, if configuration of supported functionality is not provided in Step 3 (i.e. full inference configuration is provided in Step 5).

2) If configuration of supported functionality is agreed to be provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- Proposal 6: The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5.

  - FFS on initial activation of applicable functionality if configuration of supported functionality is agreed to be provided in Step 3.

  - FFS on additional L1/L2 signaling for activation/deactivation.

- Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer).

  - FFS on the order of inference configuration and applicable functionality reporting,

  - FFS on functionality activation.

Earlier in meeting #125bis (Changsha), RAN2 reached the following agreements for LCM for UE-side model.

Agreements (LCM for UE-side model)

- Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress.

  - “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available.

  - FFS what functionality refers to.

- Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Agreements (LCM for UE-side model)

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.

  - The NW configures AI/ML functionalities via RRC/LPP message.

  - FFS what the configuration contains.

  - FFS how to report applicable functionality and what is applicable functionality.

- FFS how the two approaches will be specified and whether we can combine them into one procedure.

  - FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

Agreements (LCM for UE-side model)

- For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

- “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

In this contribution, we will further discuss the details of LCM for UE-side models.

# Definitions of Terminologies

## Deactivation

In RAN2 meeting #126, the discussion of functionality activation/deactivation resulted in the following agreement.

RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved.

  - FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

Some companies at the meeting think deactivation means switching to other models, while others think deactivation means falling back to the legacy solution at the functionality level (for which the discussion is about). We are the proposer of the latter; we think that at functionality level we do not need to control/select individual models to be used, for each functionality. Therefore, there are only two solutions: the non-AI/ML based, i.e., the existing solution, and the AI/ML-based solution. Once we deactivate the AI/ML-based solution (i.e., the functionality), we will have to fall back to the legacy solution. Note that switching means the LCM is at the model level, which is outside the scope of functionality LCM. Switching from a functionality (say BM) to another functionality (say Positioning) does not make sense.

Proposal 1: For functionality-based LCM, deactivation refers to falling back to the legacy solution.

## Configurable/Configured/Available Functionality

In the post-meeting email discussions, most companies have agreed on the following terminologies, as proposed by the moderator.

- Proposal 1: supported functionalities refer to functionalities that UE can indicate by using UE capability signalling.

- Proposal 3: Applicable functionalities refer to functionalities that the UE is ready to apply for model inference.

- Proposal 5: Activated functionalities refer to functionalities already activated and performing inference.

However, there are other terminologies/issues that need further studies. These include

- whether the definition on configured functionalities is needed,

- the relationship between configured functionalities and applicable functionalities, if configured functionalities is defined,

- whether the definition of available functionalities is needed, given applicable functionalities has been defined.

Configurable Functionality

In RAN2 we have agreed that when a UE reports its supported functionality to the NW, the corresponding model may not be available. Therefore, the NW does not know whether the supported functionality is ready to be configured, unless additional indication is exchanged, e.g., through configurable functionality.

We proposed to add the following definition of Configurable Functionality.

Proposal 2: Configurable functionalities refer to supported functionalities that are ready to be configured by gNB/LMF.

Configured Functionality and Applicable Functionality

In our opinion, once a configurable functionality is configured by gNB or LMF, it becomes a configured functionality. An applicable functionality (functionality that is ready for inference) is a superset of configured functionality, because not every functionality needs to be configured by the gNB/LMF.

Some companies think configuration of functionalities may be different for different purposes of LCM (i.e. inference, training, performance monitoring). However, we think it should be used only for model inference. This is because we normally will not say configured functionality at training stage as we will not train a functionality, but a model. Similarly, we will not say configured functionality at performance monitoring stage; instead, we may say configuration for performance monitoring.

Proposal 3: Configured functionalities refer to configurable functionalities that have been configured by gNB/LMF.

Observation 1: some applicable functionalities do not need to be configured so they can be used for inference without being configured by the gNB/LMF.

Proposal 4: Configured functionality is a subset of applicable functionality.

Applicability

In RAN2 meeting #126 [1], the group has reached the following agreement on the applicability of a function. However, the detail of applicability is FFS.

The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it.

- FFS other details on what is applicability/non-applicability.

It is our understanding that the applicability of a functionality, besides ready for model inference, refers to the fact that the functionality is suitable to be applied for inference in the current environment, including the additional conditions of the NW side (e.g., antenna setting etc.) and the capability/capacity of the UE side (computational power, memory/storage capacity etc.). This is because the underlying model supporting the functionality may not be suitable for all inference environments.

Proposal 5: Applicability of a functionality refers to the fact that the functionality is suitable to be applied for inference in the current environment.

Available Functionality

One topic discussed during the post-meeting email discussion was whether we should separate available functionalities from applicable functionalities which has been agreed by most companies.

It is our understanding that the meaning of “available” could be expressed by either “supported” or “applicable” which have been agreed upon; it brings neither clarity nor additional meanings that cannot be covered by the other two terms. In addition, it could create more confusion between “available” and “applicable”, as well as “available” and “configured”.

Proposal 6: RAN2 does not define and use the term “available functionalities”.

# Applicable functionality reporting

At the second phase of the post-meeting discussion, we discussed the signalling framework for applicable functionality reporting. Among the proposals from the moderator, the following is about the configuration from NW to UE.

- Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

First, we would like to discuss sub-bullet 2) on NW-side additional conditions.

In Step 1 and Step 2 (see moderator’s Proposal 1), NW has configured the UE to report supported functionalities and UE has reported them. We believe in this step, Step 3, the NW should configure the UE with its additional conditions and this step should be mandatory. As we mentioned in the Applicability discussion of Section 2.2, this step will make sure the supported model is applicable for the current environment. This step also turns a supported functionality into a configured functionality and makes it applicable (i.e., ready) for inference.

Second, for sub-bullet 3), the current proposal says the configuration of supported functionalities is FFS. It is our understanding that the configuration of each functionality depends on its use case. Therefore, we can leave this to the discussion of each use case.

Proposal 7: Revised moderator’s Proposal 2 as below.

- Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

According to the moderator proposals, the NW configures the UE with additional conditions in Step 3 and the UE reports applicable functionalities in Step 4. There was also a discussion on whether Step 5 is needed in reactive reporting.

- Proposal 5: Step 5:

1) Network configures full inference configuration to UE after applicable functionality reporting, if configuration of supported functionality is not provided in Step 3 (i.e. full inference configuration is provided in Step 5).

2) If configuration of supported functionality is agreed to be provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

In Proposal 6, the moderator concluded that “The applicable functionality is initially activated by receiving its configuration when it is provided in Step 5.”

It is our understanding that Step 5 is actually the step of functionality activation. Additional configurations (or “full inference configuration” as in the moderator’s proposal) on top of Step 3 configuration can be associated with this step of activation. We think it will be clearer if we revise Step 5 and Step 6 by combining them, making it the step of activation. We try to provide a proposal based on this thought.

- Proposal 5: Step 5: Functionality Activation

1) Network activates AI/ML-based functionality with full inference configuration (if necessary) to UE, after receiving applicable functionality reporting from UE.

2) FFS on additional L1/L2 signaling for activation/deactivation.

We can then remove the original Proposal 6 of Phase 2 discussion.

Proposal 8: Combine moderator’s Proposal 5 and Proposal 6 as the new Proposal 5. Remove the original Proposal 6.

- Proposal 5: Step 5: Functionality Activation

1) Network activates AI/ML-based functionality with full inference configuration (if necessary) to UE, after receiving applicable functionality reporting from UE.

2) FFS on additional L1/L2 signaling for activation/deactivation.

Proposal 7 of the Moderator’s conclusion is about positioning Case 1, as listed below.

- Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signalling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer).

  - FFS on the order of inference configuration and applicable functionality reporting,

  - FFS on functionality activation.

Note that positioning Case 1 refers to UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning.

We think this proposal might have messed up functionality with the underlying model supporting the functionality. Although there are no detailed statements in the TR or agreements in R19 discussions on the granularity of a functionality, from the following quotes from the TR, we can see that a functionality is roughly at the level of an AI/ML-enabled Feature or FG.

- Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature, whereby AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

- For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG.

Therefore, we need to be cautious when we talk about “a change in applicable functionality”; do we mean a change from one functionality to another functionality, or any changes within the same functionality? In our understanding, a change to the functionality of positioning Case 1 means the new functionality is not applicable for positioning Case 1; it would be some other functionalities applicable to other use cases. For this reason, we believe the original intent of this proposal would be to update the same functionality for the same use case, such as positioning Case 1. We suggest changing the original proposal as below.

Proposal 9: Revise the moderator’s Proposal 7 as below.

- Proposal 7: For positioning Case 1, UE can report a change an update in applicable functionality by sending applicable functionalities functionality updates upon change via LPP signalling (unsolicited information transfer). UE can also provide applicable functionality reporting update as a response to LMF request (solicited information transfer).

  - FFS on the order of inference configuration and applicable functionality reporting,

  - FFS on functionality activation.

So far, we have talked about applicable functionality reporting. However, there is another important aspect the UE will need to report to the NW to ensure the LCM operation at the UE side, and that aspect is the UE’s capability or capacity for model inference, for example, its computational power and memory.

This is also toughed in the Moderator’s proposal #3 as listed below.

- Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE.

The “UE-side additional conditions (internally known by UE)” in this proposal can include the UE’s capability or capacity for model inference. However, we believe this information should also be made known to the NW (not only to the UE itself) for various reasons. This information may include UE’s computational power, memory, storage, model inference time, battery level etc.

With this information, the NW can have a better understanding of the resources available at the UE side and activate/deactivate AI/ML-based functionality at the right time/situation. For example, if the UE’s current computational power or memory is not able to support the inference of a certain functionality, the NW will not activate the functionality.

To capture this aspect, we proposed a revised Proposal 3 and Proposal 4.

Proposal 10: Revise the moderator’s Proposal 3 and Proposal 4 as below.

- Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities, such as computational power and memory) and model availability in device. FFS whether other configurations can be considered by UE.

- Proposal 4: “Step 4”: UE reports applicable functionality and its capabilities/capacities in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

# Conclusions

In this contribution, we discussed our views on LCM for UE-side models. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: For functionality-based LCM, deactivation refers to falling back to the legacy solution.

Proposal 2: Configurable functionalities refer to supported functionalities that are ready to be configured by gNB/LMF.

Proposal 3: Configured functionalities refer to configurable functionalities that have been configured by gNB/LMF.

Observation 1: some applicable functionalities do not need to be configured so they can be used for inference without being configured by the gNB/LMF.

Proposal 4: Configured functionality is a subset of applicable functionality.

Proposal 5: Applicability of a functionality refers to the fact that the functionality is suitable to be applied for inference in the current environment.

Proposal 6: RAN2 does not define and use the term “available functionalities”.

Proposal 7: Revised moderator’s Proposal 2 as below.

- Proposal 2: “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

Proposal 8: Combine moderator’s Proposal 5 and Proposal 6 as the new Proposal 5. Remove the original Proposal 6.

- Proposal 5: Step 5: Functionality Activation

1) Network activates AI/ML-based functionality with full inference configuration (if necessary) to UE, after receiving applicable functionality reporting from UE.

2) FFS on additional L1/L2 signaling for activation/deactivation.

Proposal 9: Revise the moderator’s Proposal 7 as below.

- Proposal 7: For positioning Case 1, UE can report a change an update in applicable functionality by sending applicable functionalities functionality updates upon change via LPP signalling (unsolicited information transfer). UE can also provide applicable functionality reporting update as a response to LMF request (solicited information transfer).

  - FFS on the order of inference configuration and applicable functionality reporting,

  - FFS on functionality activation.

Proposal 10: Revise the moderator’s Proposal 3 and Proposal 4 as below.

- Proposal 3: UE decides the applicable functionalities based on NW-side additional conditions (if available), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities, such as computational power and memory) and model availability in device. FFS whether other configurations can be considered by UE.

- Proposal 4: “Step 4”: UE reports applicable functionality and its capabilities/capacities in the following scenarios:

1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

2) As response to NW-side additional condition (if available), FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

# References

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004