3GPP TSG RAN WG2 Meeting #127	 R2-2407489

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda Item:	8.1.3

Source:	Futurewei

Title:	Discussion on Data Collection for NW-side Model Training

Document for:	Discussion

# Introduction

In the RAN19 WID [1], the following items are listed as RAN2 or RAN2-led WI research areas for the NR_AIML_Air project.

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

In RAN2 meeting #125bis (Changsha), we reached the following agreements for Data Collection for NW-side Model Training.

Agreements

- For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches.

- We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 (Fukuoka), we advanced the discussions further and reached the following agreements for Data Collection for NW-side Model Training.

Agreements

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

- Enhance the immediate MDT framework to support periodical reporting.

  - FFS whether and what event-based reporting is supported, and

  - FFS on network request reporting.

- For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.

  - FFS how to handle case when single RRC message is not sufficient.

  - FFS if there will be any further enhancement needed pending RAN1 agreement.

In this contribution, our discussion will focus on the mechanisms and principles identified for data collection for network side model training during R18, as indicated by the Chair.

# Background

In the TR [2], Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As indicated by the TR, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting.

Table 7.3.1.2-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signalling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signalling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signalling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signalling (~20ms) - Air interface signalling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) - Air interface signalling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
**** Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.

The TR [2] also captured the following considerations for network-side data collection (Section 7.2.1.3.1).

In addition, A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms.

- The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

- On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

# Discussion

## Event-triggered and reactive reporting

In RAN2 meeting #126 [4], one of the agreements we achieved was the following.

Enhance the immediate MDT framework to support periodical reporting.

- FFS whether and what event-based reporting is supported, and

- FFS on network request reporting.

What it says is that we will support and enhance periodical reporting, but event-based and on-demand (network request) reporting mechanisms are FFS. However, in the TR [2], it clearly says both event-based and on-demand based reporting approaches are part of the “general data collections principles”.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In addition, if we looked at the data collection methods identified in R18 (Table 7.3.1.2-1 in Section 2 of this TDoc), we can see both event-based and on-demand-based (network request) reporting mechanisms are identified as reporting types. For example,

- Event-triggered reporting is identified for

  - Immediate MDT: collected by TEC/OAM

  - L3 measurement: collected by gNB

  - LPP: collected by LMF

- On-demand/Network requested reporting is identified for

  - Logged MDT: collected by TEC/OAM

  - Early measurements: collected by gNB

In fact, for some use cases, such as positioning, event-triggered reporting can be more efficient and up-to-date, because measurements only needs to be done when the UE starts to move, which saves the traffic generated by periodical reporting. In addition, allowing event-based and on-demand reporting provides flexibility of control and reporting.

Based on this consideration, we propose to support both event-based and on-demand (network request) reporting mechanisms.

Proposal 1: Support both event/trigger-based and on-demand/network request-based reporting mechanisms in addition to periodical reporting for NW-side data collection.

## RRC Segmentation

In RAN2 meeting #126 [4], one of the agreements we achieved on RRC signaling was the following.

For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.

- FFS how to handle case when single RRC message is not sufficient.

In this agreement, one thing for FFS is the handling of large amount of logged L1 measurements data when single RRC message is not sufficient. We know that the size of an RRC message is constrained by the underlying protocol layers, particularly the PDCP. The maximum size of an RRC message is typically limited by the PDCP SDU size, which should be fewer than 9K bytes. This limit ensures that the RRC message can be efficiently transmitted over the air interface without segmentation issues. However, when we have data that exceeds the size one RRC message can handle, the common approach we use to address the issue is segmentation. Based on our understanding, 3GPP does not have explicit limit on the number of segments; the only limit in the practice may be that all segments of an RRC message must be transmitted before sending another RRC message, which may not belong to NW-side data collection.

Proposal 2: When single RRC message is not enough to handle large data size for logged L1 measurement results, segmentation at RRC level can be applied while RRC message competition should be resolved.

## Data logging, UE capability/capacity for data processing and storage

In the TR [2], the “general data collections principles” also include principles regarding UE’s capability and capacity for data collection for NW-side models.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In RAN2 meeting #125bis (Changsha), we reached the following agreements for Data Collection for NW-side Model Training.

- For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches.

- We aim that the same measurement framework is applied to both gNB-centric data collection and OAM-centric data collection for NW-side data collection.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

There are two issues we would like to discuss here, the support of data logging and UE’s capability/ capacity for logged data processing/storage.

Data logging

In RAN2 meeting #126 [4], the group has agreed to take immediate MDT as the baseline for OAM-centric data collection (see the agreement below).

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

However, whether logged MDT will be used and enhanced has not been determined. Based on the TR we quoted above, it is expected that data logging will be supported by the UE.

- UE to support data logging

We understand there is concern regarding the use of UE memory by logged data (which we will address a bit later), we think overall if we can log collected data and send multiple of them in one RRC transmission, it will save the number of signaling exchanges and it should be encouraged (as long as delay requirement is met). Therefore, data logging at the UE should be supported.

Proposal 3: Support data logging and enhancement to logged MDT for data collection for NW-side models.

UE’s capability/ capacity for logged data processing/storage

We mentioned that there is concern regarding the use of UE memory for logging collected data for NW-side models.

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

This is a practical concern because the resources at the UE side are limited, for example, the computational power and memory (the collected data will be in the memory for the BM and Positioning use cases, unlike some of the application-level use cases). We believe it is necessary to provide a mechanism to report a UE’s capability/capacity for storing measured data temporarily for NW-side model training. Logged MDT may only be used when the UE has enough available resources to support it. Otherwise, immediate MDT should be used for data collection.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power or available memory. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# Conclusions

In this contribution, we continued to present our observations and views on topics related to NW-side data collection. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Support both event/trigger-based and on-demand/network request-based reporting mechanisms in addition to periodical reporting for NW-side data collection.

Proposal 2: When single RRC message is not enough to handle large data size for logged L1 measurement results, segmentation at RRC level can be applied while RRC message competition should be resolved.

Proposal 3: Support data logging and enhancement to logged MDT for data collection for NW-side models.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power or available memory. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024