3GPP TSG RAN WG2 Meeting #127	 R2-2407490

Maastricht, The Netherlands, August 19 – 23, 2024

Agenda Item:	8.1.4

Source:	Futurewei

Title:	Discussion on Data Collection for UE-side Model Training

Document for:	Discussion

# Introduction

On the topic of data collection for UE-side model training, we achieved the following agreements at RAN2 meeting #126 [4].

Agreements

- Solution 1a) has no specification impact.

Agreements

- For the options identified to realize the different levels of data content visibility

- Full visibility for standardized data content.

- Partial visibility for partially standardized data content.

- No standardized visibility

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

- Capture table below (Table 7.2.1.3.2-1) in TR (Rapporteur will clean up and refine and review over email)

- Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

- The Chair also assigned the following post-meeting email discussion on the TP based on the result of the discussions of this meeting.

- [POST126][034][AIML PHY] TP for data collection  (Ericsson)

Intended outcome: Prepare and review TP capturing table in minutes and agreements on data collection

Deadline:  long

The discussion results were captured at the following.

##### 7.2.1.3.2	 Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). FFS whether according to RAN2 there might be 3GPP specification impact.

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server. FFS whether according to RAN2 there might be 3GPP specification impact.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc. The result of this analysis can be found below in Table 7.2.1.3.2-1.

It is worth noting that for the different options, the data content visibility was discussed. The different levels of data content visibility are captured in the Note 3 in the Table 7.2.1.3.2-1.

Note:	RAN2 discussed that, except for the case of standardized data content, the data content visibility could be achieved via SLA (Service Level Agreement). However, SLA is out of RAN2 scope.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

Related to privacy, it has been stressed in RAN2 the importance that any potential mechanism to collect UE side data for model training purposes (including the options 1a, 1b, 2, 3 listed above) must comply with privacy protection regulations, requirements, laws and/or policies. An informative Annex is included at the end of this document capturing examples of privacy concerns for different stakeholders participating in the discussion.

In this contribution, we will further discuss the issues based on the result of the post-meeting email discussions.

# Discussion

## General Discussion

### Termination Entities

First termination entity vs final termination entity

So far, we have been using the term “first termination entity” but we don’t have a definition of it. The rapporteur suggested initial definition as below.

The "(First)termination entity" refers to the entity that receives and stores data transmitted from the UE, which possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. The purpose of introducing the term ‘(first) termination entity’ is to emphasize the entity’s role in management and possession of the collected data.

As the result of the discussion, most companies agreed on the following, as proposed by the email rapporteur in post-meeting 126 discussion.

- For solutions 1a the first termination entity of UE-side data collection is the OTT server.

- For solutions 1b the first termination entity is the server for UE-side data collection. FFS the server is inside or outside of MNO.

- For solutions 2 the first termination entity of UE-side data collection is inside the CN.

- For solutions 3 the first termination entity of UE-side data collection is the OAM.

During the discussion, ambiguity arose between the terms “termination entity” and “first termination entity”. In the rapporteur’s original questions, the term used was “termination entity”. Many companies pointed out that we should only discuss the first termination entity, which handles data collection. In other words, there may exist the second or final termination entity for further data transfer and processing. It is our opinion that these two terms need to be clearly defined to avoid ambiguity in future discussions.

Here are our proposed definitions.

Proposal 1: The first termination entity refers to the first network element/function that receives and stores data transmitted from the UE.

- A first termination entity possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. I

- A first termination entity may not be the place where UE-side model is trained, in which case the entity is a final termination entity.

Proposal 2: The (final) termination entity refers to the place where the UE-side model training is done.

- Depending on the data collection procedure, a final termination can also be the first termination entity and vice versa.

### MNO’s Level of Controllability

A topic arose during the discussion because people had different understanding of it; that was level of controllability. The rapporteur proposed definitions for different levels (as below), but the group didn’t have enough time for further discussion.

- Full Control: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing the volume of data. For example, the UE should start the data transfer only if that is allowed by the MNO/NW.

- Partial Control: The MNO has some degree of control over the data transfer but may be limited by certain factors such as agreements with third parties. For example, the UE can start the data transfer without involvement of MNO/NW as long as the tunnel is available.

- No Control: The MNO has no capability to influence or manage the data transfer.

We think the common understanding of the level of controllability is essential for the discussions. We can start with these high-level definitions first and figure out all the details later.

Proposal 3: Define the following terminologies related to the level of controllability.

- Full Control: The MNO has the capability to manage data transfer to the server for UE-side data collection. This may include initiating, terminating, and fully managing the volume of data. For example, the UE should start the data transfer only if that is allowed by the MNO/NW.

- Partial Control: The MNO has some degree of control over the data transfer but may be limited by certain factors such as agreements with third parties. For example, the UE can start the data transfer without involvement of MNO/NW as long as the tunnel is available.

- No Control: The MNO has no capability to influence or manage the data transfer.

Note the functions can be controlled are not exclusive and may be extended or revised.

### MNO’s Visibility of Data Content

MNO’s visibility of data content refers to the extent to which the MNO understands what the data is or is about. Many companies agreed that 'visibility' of data content entails the MNO's ability to at least be aware of, access, and comprehend the data being transferred to some degree. However, it is our understanding that for some data, the MNO may not have the right to access and comprehend (for example, those related to UE’s privacy and security).  In other words, there are concerns related to regulatory compliance for these types of activities. In this case, some of the UE data needs to be pre-processed, for example, anonymized, before being sent to the data collection server.

Proposal 4: When collecting data for UE-side model training, data needs to be pre-processed to remove sensitive and privacy information associated with the UE.

## Discussion of Option 1b

### AI/ML-specific Data Transfer Path

In Table 7.2.1.3.2-1, data transfer path for Option 1b) is described as “UE  Server for data collection for UE-side model training/OTT server”. However, this does not match the text description above, which says

1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

We note that the sign forward slash “/” usually means “or” in English. So this expression may confuse people and make them think from the UE the data can be sent to either the “server for data collection for UE-side model training” or the “OTT server”. In fact, based on the text description, the data will be first sent to the “server for data collection for UE-side model training”.  After that, it could be sent to the “OTT server”. This needs to be corrected in the table.

Proposal 5: In Table 7.2.1.3.2-1, correct data transfer path for Option 1b) as “UE  Server for data collection for UE-side model training/  OTT server”.

### Feasibility of Option 1b

After discussing this option with SA experts and other companies, we found Option 1b has the basic support from existing 3GPP framework, but the framework needs to be enhanced in many aspects in order to make it feasibility.

The current standards supporting Option 1b are 3GPP TS 23.288 defined by SA2 and TS 26.531 defined by SA4 (Data collection and reporting: General Description and Architecture). The reference architecture is shown in the figure below (from TS 26.531).

Figure. Reference architecture for data collection and reporting

As a brief introduction, TS 23.288 (Clause 6.2.8) envisages a set of high-level procedures by which data is collected by a Network Data Analytics Function (NWDAF) from UE Application(s) via an intermediary Application Function. TS 26.531 defines a generic reference architecture for data collection, reporting and subsequent exposure that satisfies those procedures, including the logical functions involved and the logical reference points between them.

However, we note that this framework was not originally designed for collecting PHY layer data for AI/ML use cases. If we want to use this framework, we need to enhance it in many aspects. Below is just a partial list of them.

- TS 26.531 is a SA4 spec and PHY layer AI/ML data (e.g., radio measurement) is not supported by this SA4 spec. The scope of TS 26.531 covers “specifications of codecs for speech, audio, video, graphics and other media types related to emerging services such as extended realities (XR) and gaming, as well as the system and delivery aspects of such contents.”

- TS 26.531 requires a UE Application (which provides data to an AF) to be configured by the Application Service Provider (ASP) and to be assigned an Application ID from the ASP. This requirement would give the control of data collection to the ASP which is at the application level. Obviously, it is not feasible for the PHY layer use cases.

Observation 1: Existing 3GPP standards do not support Option 1b and need enhancements. The feasibility of Option 1b for data collection for UE-side model training needs cross-RAN/SA studies and discussions.

# Conclusions

In this contribution, we continued to present our observations and views on data collection for UE-side model training. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: The first termination entity refers to the first network element/function that receives and stores data transmitted from the UE.

- A first termination entity possesses the authority to oversee the subsequent handling of this data, such as data cleaning, forwarding, sharing, and analysis, among others, in compliance with privacy policies, security protocols, and any regulatory compliance requirements. I

- A first termination entity may not be the place where UE-side model is trained, in which case the entity is a final termination entity.

Proposal 2: The (final) termination entity refers to the place where the UE-side model training is done.

- Depending on the data collection procedure, a final termination can also be the first termination entity and vice versa.

Proposal 3: Define the following terminologies related to the level of controllability.

- Full Control: The MNO has the capability to manage data transfer to the server for UE-side data collection. This may include initiating, terminating, and fully managing the volume of data. For example, the UE should start the data transfer only if that is allowed by the MNO/NW.

- Partial Control: The MNO has some degree of control over the data transfer but may be limited by certain factors such as agreements with third parties. For example, the UE can start the data transfer without involvement of MNO/NW as long as the tunnel is available.

- No Control: The MNO has no capability to influence or manage the data transfer.

Note the functions can be controlled are not exclusive and may be extended or revised.

Proposal 4: When collecting data for UE-side model training, data needs to be pre-processed to remove sensitive and privacy information associated with the UE.

Proposal 5: In Table 7.2.1.3.2-1, correct data transfer path for Option 1b) as “UE  Server for data collection for UE-side model training/  OTT server”.

Observation 1: Existing 3GPP standards do not support Option 1b and need enhancements. The feasibility of Option 1b for data collection for UE-side model training needs cross-RAN/SA studies and discussions.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2024

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024