3GPP TSG-RAN WG2 Meeting #127                               R2-2407493

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Agenda item:		7.24.2.2

Source:				Samsung, Ericsson

Title:					Discussion on remaining issues on cell individual offset

Document for:		Discussion & Decision

1	Introduction

In this contribution, we discuss on remaining issues for cell individual offset (CIO) in reportConfig, which RAN2 has not discussed.

2	Discussion

In RAN2#123bis, RAN2 agreed to introduce the cell individual offset in reportConfig. This enables NW to configure a different CIO per event for a neighboring cell. Then, UE can apply different CIO for each event for the same cell.

<RAN2#123bis> R2-2311221 Configuration of cell individual offset in ReportConfig NTT Docomo, Ericsson, KDDI corporation, BT Plc., AT&T, Orange, Turkcell, Deutsche Telekom discussion Rel-18 Observation 1 When cells with different radio capabilities are be deployed on the same frequency, having a cell individual offset may causes issue for UE accessing the cell. Proposal 1 Cell specific offset value is introduced within the ReportConfig IE. - ZTE thinks that to support this we would need a new capability Agreements on cell individual offset - Cell specific offset value is introduced within the ReportConfig IE. A new capability will be added

Next, in RAN2#126, RAN2 agreed for NW to configure the new CIO per a frequency for a PCI.

What RAN2 have agreed for the new CIO and related fields (in ReportConfigInterRAT and ReportConfigNR) are captured in RRC specification as follows:

<TS 38.331 v.18.2.0> ReportConfigInterRAT information element <SKIP> EventTriggerConfigInterRAT ::= SEQUENCE { <SKIP> cellIndividualOffsetList-r18 SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF CellIndividualOffsetList-EUTRA-r18 OPTIONAL -- Need R <SKIP> } <SKIP> CellIndividualOffsetList-EUTRA-r18 ::= SEQUENCE { physCellId-r18 EUTRA-PhysCellId, cellIndividualOffset-r18 EUTRA-Q-OffsetRange, carrierFreq-r18 ARFCN-ValueEUTRA OPTIONAL -- Need R } CellIndividualOffsetList-EUTRA field descriptions. carrierFreq Indicates the EUTRA frequency for which cellIndividualOffset is applicable. If the field is not configured, the EUTRA frequency indicated by carrierFreq within the MeasObjectEUTRA of the measID associated with this ReportConfigInterRAT applies.. cellIndividualOffset Cell individual offsets applicable to a specific measurement event. If this field is present, the UE, for the same cell, shall ignore the cell individual offset configured within the MeasObjectEUTRA of the measID associated with this ReportConfigInterRAT.. physCellId Physical cell identity of a E-UTRAN cell in the cell list.. ReportConfigNR information element <SKIP> EventTriggerConfig ::= SEQUENCE { <SKIP> cellIndividualOffsetList-r18 SEQUENCE (SIZE (1..maxNrofCellMeas)) OF CellIndividualOffsetList-r18 OPTIONAL, -- Need R <SKIP> } <SKIP> CellIndividualOffsetList-r18 ::= SEQUENCE { physCellId-r18 PhysCellId, cellIndividualOffset-r18 Q-OffsetRangeList, ssbFrequency-r18 ARFCN-ValueNR OPTIONAL -- Need R } CellIndividualOffsetList field descriptions. cellIndividualOffset Cell individual offsets applicable to a specific measurement event. If this field is present, the UE, for the same cell, shall ignore the cell individual offset configured within the MeasObjectNR of the measID associated with this ReportConfigNR.. physCellId Physical cell identity of a cell in the cell list.. ssbFrequency Indicates the NR frequency of SS for which cellIndividualOffset is applicable. If the field is not configured, the NR frequency of SS indicated by ssbFrequency indicated within the MeasObjectNR of the measID associated with this ReportConfigNR applies..

RAN2 has discussed and introduced the new CIO for legacy handover. i.e., cellIndividualOffsetList is defined in EventTriggerconfig in ReportConfigNR. This enables NW to configure different CIO per event for the same cell, improving flexibility for network configuration/management. However, we assume CHO can also have the same benefit from this new CIO (i.e., NW can configure different CIO per conditional event, and then UE applies the CIOs to evaluate CondEvent). Given it is early stage of CHO implementation, we think this enhancement could be useful. As far as we know, this issue has not been discussed in RAN2. So, we would like to discuss on whether to introduce CIO for conditional handover.

Proposal 1. RAN2 to discuss whether to introduce CIO in reportConfig for conditional handover.

According to the current spec (as captured above), the new CIO was also introduced in ReportConfigInterRAT. It means the new CIO can be configured/used for neighboring LTE cells (i.e., for NR to LTE handover). What we wonder and clarify here is, there is a need for the new CIO for 1) handover from LTE to NR, or 2) handover from LTE to LTE. Technically, we can have the same motivation with legacy handover and conditional handover, to support the new CIO for handover from LTE. Besides, considering EN-DC is one of most common network deployments, it would be beneficial to introduce the new CIO for handover from LTE. However, we also have concern on whether this can be implemented at this late stage. Hence, we also would like to discuss on whether to introduce CIO in reportConfig for handover from LTE (i.e., LTE to NR handover, or LTE to LTE handover).

Proposal 2. RAN2 to discuss whether to introduce CIO in reportConfig for handover from LTE (i.e., LTE to NR handover, or LTE to LTE handover).

3	Conclusion

Based on the discussion above, we propose:

Proposal 1. RAN2 to discuss whether to introduce CIO in reportConfig for conditional handover.

Proposal 2. RAN2 to discuss whether to introduce CIO in reportConfig for handover from LTE (i.e., LTE to NR handover, or LTE to LTE handover).

4	Reference