3GPP TSG-RAN WG2 Meeting #127                               R2-2407494

Maastricht, Netherlands, 19th – 23rd Aug, 2024

Agenda item:		7.0.2.8

Source:				Samsung

Title:					Discussion on extensions of ToAddModList

Document for:		Discussion & Decision

1	Introduction

During the Rel-16 ASN.1 review, there were several extensions of ToAddModLists, but it was found they were extended in a different way. To avoid ASN.1 inconsistency and misuse for extension of ToAddModLists, RAN2 agreed to have a CR [1] which introduced clause A.4.3.6 in current RRC specification. This annex provides the guideline for how to extend ToAddModList, as captured below:

### A.4.3.6 Non-critical extensions of lists with ToAddMod/ToRelease <SKIP> – When fields are added to the list element structure, an extension marker should normally be used if available. If no extension marker is available or if overhead or other considerations prevent using the extension marker, an extension structure should be created for the new fields, with the suffix "Ext" added to the end of the field name and the element structure type name (before the -vNxy suffix), and a parallel ToAddMod list introduced to hold the new structures, also with the "Ext" suffix. The field description table should indicate that the parallel list contains the same number of entries, and in the same order, as the original list. No new ToRelease list is typically needed (unless the list element ID type changes). It should typically be ensured that the contained fields in the "Ext" elements are releasable without release and add of the entire list element; this can, for instance, be ensured by having the new fields be OPTIONAL Need R. If multiple extensions of the same list are needed, the version suffix should distinguish the lists (e.g. listElementToAddModListExt-vNwz added after listElementToAddModListExt-vNxy). The result is as shown in the following example: -- /example 2/ ASN1START ContainingStructure ::= SEQUENCE { listElementToAddModList SEQUENCE (SIZE (1..maxNrofListElements)) OF ListElement OPTIONAL, -- Need N listElementToReleaseList SEQUENCE (SIZE (1..maxNrofListElements)) OF ListElementId OPTIONAL, -- Need N ..., [[ -- Parallel list listElementToAddModListExt-vNxy SEQUENCE (SIZE (1..maxNrofListElements)) OF ListElementExt-vNxy OPTIONAL -- Need N ]], [[ -- Second parallel list from a later spec version listElementToAddModListExt-vNwz SEQUENCE (SIZE (1..maxNrofListElements)) OF ListElementExt-vNwz OPTIONAL -- Need N ]] } ListElement ::= SEQUENCE { elementId ListElementId, field1 INTEGER (0..3), field2 ENUMERATED { value1, value2, value3 } } ListElementExt-vNxy ::= SEQUENCE { field3-rN BIT STRING (SIZE (8)) OPTIONAL -- Need R } ListElementExt-vNwz ::= SEQUENCE { field4-rN INTEGER (0..255) OPTIONAL -- Need R } -- ASN1STOP <SKIP>

However, we still found several ASN.1 which does not follow the guideline when new fields are added to the list element structure. In this paper, we would like to propose update the ASN.1 which does not follow the guideline, and remind RAN2 of keeping this guideline in mind.

2	Discussion

cg-SDT-ConfigLCH-RestrictionToAddModList-r17 and cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 can be included in RRCRelease message:

SDT-MAC-PHY-CG-Config-r17 ::= SEQUENCE { -- CG-SDT specific configuration cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-ConfigLCH-Restriction-r17 OPTIONAL, -- Need N <SKIP> ..., [[ cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-ConfigLCH-Restriction-v1800 OPTIONAL, -- Need N <SKIP> } <SKIP> CG-SDT-ConfigLCH-Restriction-r17 ::= SEQUENCE { logicalChannelIdentity-r17 LogicalChannelIdentity, configuredGrantType1Allowed-r17 ENUMERATED {true} OPTIONAL, -- Need R allowedCG-List-r17 SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16 OPTIONAL -- Need R } CG-SDT-ConfigLCH-Restriction-v1800 ::= SEQUENCE { cg-SDT-MaxDurationToNextCG-Occasion-r18 ENUMERATED { ms10, ms100, sec1, sec10, sec60, sec100, sec300, sec600, sec1200, sec1800, sec3600, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Need R }

cg-SDT-ConfigLCH-RestrictionToAddModListExt-v1800 is the extension of cg-SDT-ConfigLCH-RestrictionToAddModList-r17, but it does not follow the highlighted text in the guideline. i.e., the suffix “Ext” is not shown in the newly added field name (i.e., CG-SDT-ConfigLCH-Restriction-v1800). Thus, we propose to add the suffix “Ext” for the field name (i.e., CG-SDT-ConfigLCH-RestrictionExt-v1800).

Proposal 1. Update CG-SDT-ConfigLCH-Restriction-v1800 to CG-SDT-ConfigLCH-RestrictionExt-v1800.

For MUSIM gap configuration, the following ASN.1 is used. musim-GapToAddModListExt-v1820 is the extension of musim-GapToAddModList-r17.

MUSIM-GapConfig-r17 ::= SEQUENCE { musim-GapToReleaseList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-GapId-r17 OPTIONAL, -- Need N musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 OPTIONAL, -- Need N musim-AperiodicGap-r17 MUSIM-GapInfo-r17 OPTIONAL, -- Need N ..., [[ musim-GapToAddModListExt-v1820 SEQUENCE (SIZE (1..3)) OF MUSIM-GapExt-v1820 OPTIONAL, -- Need N musim-GapKeep-r18 ENUMERATED {true} OPTIONAL -- Need N ]] } MUSIM-Gap-r17 ::= SEQUENCE { musim-GapId-r17 MUSIM-GapId-r17, musim-GapInfo-r17 MUSIM-GapInfo-r17 } MUSIM-GapExt-v1820 ::= SEQUENCE { gapPriority-r18 GapPriority-r17 }

However, gapPriority-r18 within MUSIM-GapExt-v1820 is mandatory. In order to release this field (i.e., gapPriority-r18), NW needs to release the entire list element via musim-GapToReleaseList and add the rest of the fields (i.e., within MUSIM-Gap-r17) again. That is why the highlighted guideline recommend to use optional field with Need R. However, we understand it is NBC to update mandatory field to optional. Therefore, we would like RAN2 to discuss whether to update gapPriority-r18 in MUSIM-GapExt-v1820.

Proposal 2. Discuss on whether to update gapPriority-r18 in MUSIM-GapExt-v1820 which is mandatory field, to optional field with Need R, which is NBC.

sl-FreqInfoToAddModListExt-v1800 is the extension of sl-FreqInfoToAddModList-r16. Each entry of sl-FreqInfoToAddModListExt-v1800 (i.e., SL-FreqConfigExt-v1800) includes several optional fields, but they are Need M. Again, they cannot be released without releasing and adding the entire list element. Therefore, based on the highlighted guideline, we also propose RAN2 to discuss whether update Need code (from Need M to Need R) for the fields in SL-FreqConfigExt-v1800.

<SKIP> SL-PHY-MAC-RLC-Config-r16::= SEQUENCE { <SKIP> sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 OPTIONAL, -- Need N <SKIP> } SL-PHY-MAC-RLC-Config-v1700 ::= SEQUENCE { <SKIP> sl-FreqInfoToAddModListExt-v1800 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigExt-v1800 OPTIONAL, -- Need N <SKIP> } SL-FreqConfig-r16 ::= SEQUENCE { sl-Freq-Id-r16 SL-Freq-Id-r16, sl-SCS-SpecificCarrierList-r16 SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier, sl-AbsoluteFrequencyPointA-r16 ARFCN-ValueNR OPTIONAL, -- Need M sl-AbsoluteFrequencySSB-r16 ARFCN-ValueNR OPTIONAL, -- Need R frequencyShift7p5khzSL-r16 ENUMERATED {true} OPTIONAL, -- Cond V2X-SL-Shared valueN-r16 INTEGER (-1..1), sl-BWP-ToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF BWP-Id OPTIONAL, -- Need N sl-BWP-ToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-Config-r16 OPTIONAL, -- Need N sl-SyncConfigList-r16 SL-SyncConfigList-r16 OPTIONAL, -- Need M sl-SyncPriority-r16 ENUMERATED {gnss, gnbEnb} OPTIONAL -- Need M } SL-Freq-Id-r16 ::= INTEGER (1.. maxNrofFreqSL-r16) SL-FreqConfigExt-v1800 ::= SEQUENCE { absenceOfAnyOtherTechnology-r18 ENUMERATED {true} OPTIONAL, -- Need M sl-FreqSelectionConfigList-r18 SEQUENCE (SIZE (1..8)) OF SL-FreqSelectionConfig-r18 OPTIONAL, -- Need M sl-SyncTxDisabled-r18 ENUMERATED {true} OPTIONAL, -- Need M sl-EnergyDetectionConfig-r18 CHOICE { sl-MaxEnergyDetectionThreshold-r18 INTEGER (-85..-52), sl-EnergyDetectionThresholdOffset-r18 INTEGER (-13..20) } OPTIONAL, -- Need M ue-ToUE-COT-SharingED-Threshold-r18 INTEGER (-85..-52) OPTIONAL, -- Need M harq-ACK-FeedbackRatioforCW-AdjustmentGC-Option2-r18 INTEGER (10..100) OPTIONAL, -- Need M ... }

Proposal 3. Discuss on whether to update Need code (from Need M to Need R) the fields in SL-FreqConfigExt-v1800.

3	Conclusion

Based on the discussion above, we propose:

Proposal 1. Update CG-SDT-ConfigLCH-Restriction-v1800 to CG-SDT-ConfigLCH-RestrictionExt-v1800.

Proposal 2. Discuss on whether to update gapPriority-r18 in MUSIM-GapExt-v1820 which is mandatory field, to optional field with Need R, which is NBC.

Proposal 3. Discuss on whether to update Need code (from Need M to Need R) the fields in SL-FreqConfigExt-v1800.

4	Reference

[1] R2-2102292, ASN.1 guidelines for extension of lists using ToAddMod structure, MediaTek Inc.