3GPP TSG RAN WG3 Meeting #125	R2-2407498

Maastricht, Netherlands, 19 - 23 August, 2024
Agenda Item:	8.8.5

Source:    TCL.

Title:         Discussion on support of NTN regenerative payload architecture

Document for:	Discussion and Decision

# Introduction

As part of Release 19, a new work item is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support non-terrestrial network architecture with 5G system functions on board of satellite (i.e., regenerative payloads) [1]. The major objectives the new work item are as detailed below:

- Support of regenerative payload [RAN3, RAN2, RAN4]

- Specify the support of gNB on board in TS 38.300

- Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3]

Note: if any additional necessary stage-3 specifications impact for e.g., NG is identified, RAN3 will handle it.

The discussion regarding of possible enchantments to address these objectives has been started during RAN 2#125/#126 and RAN3#123 and 124 3GPP meetings, and during RAN2#126 the following agreement have been made:

Agreements: - Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any 2. Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any - RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed - RAN2 assumes that network verified UE positioning is supported for regenerative architecture - RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero

This report discusses addressing UE RRC connected and/or inactive procedures impacts due to Xn and/or NG failure in NTN regenerative access.

# Discussion

3GPP is currently discussing the regenerative payload architecture of NTN satellite access in both SA and RAN WGs As part of Release 19, it is proposed to define further enhancements for NG-RAN based Non-Terrestrial Networks to support non-terrestrial network architecture with 5G system functions on board of satellite (i.e., regenerative payloads). In such an architecture the gNB can be deployed on-board of a satellite terminating the Uu radio interface toward the UE (via the service link) and terminating the NG network interface toward the 5GC (via the feeder link). Additionally, different gNBs hosted by two regenerative satellite payloads may often be interconnected to each other’s by the mean of an inter-satellite link (ISL) interface for exchanging of Xn protocol messages. Under such architecture, unlike terrestrial networks (TN) and transparent NTN access where NG and Xn interfaces are presumed to be always available, in regenerative NTN access, NG and/or Xn logical interfaces may often be disconnected, switched, suspended, or updated due to satellite movement or any other physical or logical factors that may impact the performance of the of feeder link between gNB and AMF or the inter-satellite links (ISLs) between the satellite gNBs [2, 3, 4]. For example, if a UE is accessing from an NTN access with gNB on-board, there will always be a single NG logical link for such a UE towards its selected AMF/UPF and one or more Xn interface toward its last serving gNB at a given time (Figure 1). If a XN and/or link failure or switching occurred (Figure 2), the following UE related RRC connection related procedures maybe impacted:

- UE NG link related procedures such as initial UE message, initial UE context setup message and/or the paging message between UE serving gNB and selected AMF.

- UE XN link related procedures such as UE context retrieval request and response messages between the satellite gNBs serving UE and last UE serving satellite gNB.

Figure 1: UE accessibly form NTN Regenerative Access

An NG or XN failure or disconnection impacting one of the above mention procedures, may lead to interruption of UE RRC resumption, RRC reestablishment, or setup procedures but this issue can be easily addressed by enhancing of Xn or NG protocols without the need of rejecting of UE RRC connection  as discussed in [5]. However, if the failure occur on both Xn /inter-satellite and NG /feeder links at the time when UE is trying to initiate and RRC connection, this may lead to a temporal loss of both the link between the UE’s current and last serving satellite gNBs as well as the link between UE current serving satellite gNBs and the UE’s selected AMF (Figure 2). During this time of such connection loss, the gNB may unable to complete UE RRC connection process. This issue will become more severe when the coverage toward another satellite gNB is no available for the UE. Additionally, if there is the failure is due to a physical or hardware issue this may lead to a total isolation of such a gNB. In this case, any UE RRC connection request toward such a gNB from the UE will be just waste of UE power.

- Observation 1: A failure of only NG or XN link can be easily addressed without the need of rejecting of UE RRC connection by enhancing of Xn or NG protocols. Only a failure of both NG and XN links may require of rejecting UE connection.

Figure 2: An Example of a Scenario of Inter-satellite and Feeder Link Failure when a coverage is available or unavailable toward a last gNB

In case of XN and NG failure, UEs serving satellite gNB may try to recover the failed ng or xn links and/or [5]. If the failed link is not recoverable, the serving satellite gNB may reject the UE RRC connection procedures indicating a failure of Xn or NG. However, before rejecting the UE RRC connection, it may  request exchanging of neighboring XN/NG links information from surrounding satellite gNb or AMF to identify the gNBs that was that serving or currently is serving or has available coverage toward the UE, and the status of its Xn, NG or NG and Xn links toward UE’s selected AMF. Accordingly, when rejecting the UE RRC connection, the satellite gNBs may redirect UE RRC connection toward such a gNB or may redirect the UE to TN gNB node if the link status information exchange indicated of all NG/Xn links are unavailable and only TN is available (Figure 3).

- Proposal 1: When rejecting the UE RRC connection, the satellite gNBs may redirect the UE RRC connection toward

- Option 1: all neighboring satellite gNB or;,

- Option 2: A satellite gNB selected based on neighboring gNB XN/NG links information or ;

- Option 3: A TN gNB node in case of TN and NTN coexistence.

Figure 3: An Example of a Scenario of Inter-satellite and Feeder Link Failure when a coverage is available and unavailable toward a last gNB

In case that failure is involving a physical and logical leading to isolation of gNB, a periodical exchange of satellite gNB links information may be performed by all surrounding satellite gNB and AMF nodes to detect the failed satellite gNB node. If the failed satellite gNB node is detected,   as gNB or AMF node who detected the failed node shall send an RRC signalling or broadcast a system information block (SIB) singling to restrict UE/UEs from initiating from initiating any RRC ,NAS connection ,physical synchronization or cell selection/re-selection request toward such a satellite gNB node (Figure 4).

- Proposal 2: If the failure is involving a physical and logical leading to isolation of a satellite gNB, an RRC signalling or a system information block (SIB) to restrict UE/UEs being served by such a satellite gNB node from initiating from initiating any RRC ,NAS connection ,physical synchronization or cell selection/re-selection request toward it.

Figure 4: A Scenario of Inter-satellite and Feeder Link Failure Leading to a gNB isolation

# Conclusion

In this contribution, we have discussed UE RRC connected and/or inactive procedures impacts due to Xn and/or NG failure in NTN regenerative access. Based on the discussion we proposed to discuss the following observation and proposals:

- Observation 1: A failure of only NG or XN link can be easily addressed without the need of rejecting of UE RRC connection by enhancing of Xn or NG protocols. Only a failure of both NG and XN links may require of rejecting UE connection.

- Proposal 1: When rejecting the UE RRC connection, the satellite gNBs may redirect the UE RRC connection toward

- Option 1: all neighboring satellite gNB or;,

- Option 2: A satellite gNB selected based on neighboring gNB XN/NG links information or ;

- Option 3: A TN gNB node in case of TN and NTN coexistence.

- Proposal 2: If the failure is involving a physical and logical leading to isolation of a satellite gNB, an RRC signalling or a system information block (SIB) to restrict UE/UEs being served by such a satellite gNB node from initiating from initiating any RRC ,NAS connection ,physical synchronization or cell selection/re-selection request toward it.

# References

- RP-240775, “Revised WID: Rel-19 WI Non-Terrestrial Networks (NTN) for NR Phase 3”, THALES, CATT, RAN#103, Maastricht, The Netherlands, March 18-22, 2024.

- R3-243017,” LS on Support of Regenerative-based Satellite Access” SA2(VIVO), Fukuoka, Japan, 20 - 24 May, 2024.

- R2-2405156” Regenerative payload for NR NTN” Samsung. Fukuoka, Japan, May 20th-24th, 2024

- R2-2404799 ” Considerations on regenerative payload in NTN” Lenovo, Fukuoka, Japan, May 20th-24th, 2024.

- R3-244634 ”Discussion on NTN regenerative architecture” TCL, Maastricht, Netherlands, 19 - 23 August, 2024